3GPP TSG-SA2 Meeting #171	S2-2508208

Wuhan, China, October 13 - 17, 2025	(revison of xxxx)

Source: 	vivo

Title: 	KI#2 Solution #24 updates and interim conclusion updates

Document for: 	Approval

Agenda Item: 	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes an update on KI#2 solution#24 to address ENs and interim conclusion update.

1	Discussion

This paper proposes to update the solution#24 of KI#2 to address some Editor’s notes.

To address the following EN, some new clause clarifying the input data should be collected, the possible output data, and the possible actions of NFs are added.

The User Plane Traffic Exception can be classified into three types: burst, malicious, and suspicious traffic. Corresponding mitigation actions include traffic splitting or switching for burst traffic, traffic dropping for malicious traffic, and traffic redirection for suspicious traffic.

Editor's note:	Input data and output data of the new analytics ID are FFS. What actions can be done by which consumers based on corresponding user plane traffic exception is FFS.

Editor's note:	What specified user plane traffic pattern data should be collected to train ML model is FFS.

For the last EN, the collocated UPF and AnLF can interact with each other based on existing SBA service, e.g. Nnwdaf_AnalyticsSubscription server and Nupf_EventExposure service.

Editor's note:	Whether and how to do interaction between the collocated UPF and AnLF is FFS.

| Category | Parameters | Resource solution | details |
| --- | --- | --- | --- |
| Time info | Time slot start | #23, #24, #27, #29, #36 | Time slot start within the Analytics target period. |
|  | Duration | #23, #24, #27, #29, #36 | Duration of the time slot. |
|  | time period(s) | #22, | expected/observed time period(s) of abnormal user plane observations |
|  | Start time & end time | #31 | Start time / end time of the specified traffic flow. |
| Identifier info | application ID | #22, #27, #29, #31, #33, #36 |  |
|  | UPF ID | #23, #27, #29, #31, #36 |  |
|  | IP packet filter | #23, #27, #31, #33, #36 | Target Packets or packet patterns are identified by packet pattern ID (#22) |
|  | UE ID | #22, #23, #27, #29 | PDU session ID (#24) QFI (#33) PDU session List (#36) PDU sessions (SUPI(s), DNN, S-NSSAI) |
| Exception level | predicted severity UP performance level Traffic burst index traffic anomaly risk levels exception level | (#23) (#27) (#29) (#35) (#36) | For example: risk level, e.g. Informative, Warning, High, Critical. |
| Exception type | type of abnormal user plane observations | (#22) |  |
|  | UP pattern type | #23 | Predicted UP pattern type, e.g. DDoS, misbehaving server/UE, burst, malformed packets, unknown packets, duplicate packets, fragmented packets, etc. |
|  | Exception type | #24 | Indicates the type of exception, i.e. one of burst, malicious, suspicious. |
|  | cause | #27 | The cause of potential/ detection UP performance degradation, e.g. unexpected service flow, unexpected traffic volume or burst size, etc. |
|  | Abnormal packet indicator | #29 | Indicates whether unclassified, malformed, or suspicious packet flows were detected during the slot. Value: Yes / No. |
|  | abnormal detection information | #30 |  |
|  | Reason of abnormality | #31 | The reasons of abnormal packets, e.g. failure transmission, repeat transmission. |
|  | Malicious type suspicious identification | #34 | Malicious type of the identified Application ID or data flow: e.g. Phishing websites, Telecommunication fraud websites or apps, Malicious emails. Whether the data flow or Application ID input by the NF consumer is suspicious |
|  | exception ID | #36 | The identifier of the exception predicted by NWDAF Exception ID can be, e.g. DDoS, too much data, too high or too low rate…… |
|  | Flow stability score exception trend | #29 #36 | Indicates how consistent the traffic is over time: Stable / Medium / Unstable. Predicted trend (up/down/unknown/stable) |
| Traffic pattern | Traffic characteristics | #27 | The traffic characteristics may include the UL/DL/ round trip of: Data Burst Size, traffic patterns, traffic volume or the number of data packets, traffic rate (e.g. packet rate, bit flow rate, etc.), packet delay, loss rate, throughput, etc. within the measurement period. |
|  | Traffic prediction information | #30 #34 |  |
|  | Traffic Pattern Report | #31 | List of traffic pattern report towards the traffic over the specified UPF. |
|  | traffic characteristics | #35 |  |

Observation 1: Time info is mandatory in analytics output;

Observation 2: Identifier is mandatory in analytics output, with different granularities (UE, PDU, UPF, Application).

Observation 3: Most solutions introduced exception level to indicate severity.

Observation 4: Most solutions introduced exception type to classify anomalies.

Observation 5: Some solutions also introduced traffic pattern as part of the output.

Proposal 1: Document observation 1 to 5 in the Agreed Principles for KI#1 UC#1.

| Consumer NF | Actions | Resource solution | details |
| --- | --- | --- | --- |
| SMF | configure UPFs | #22 | block abnormal traffic |
|  | Instruct the UPF | #23 | Overload controls, e.g. reduce UPF load. Re-selections, e.g. to less loaded UPF. …… |
|  | initiate UPF reselection | #27 | distribute the load across UPF instances upon detecting abnormal traffic patterns |
|  | notifying UPF to execute traffic gating …… | #27 | e.g. notifying UPF to execute traffic gating or shaping, enforcing maximum data rate or adjusting QoS parameters. |
|  | modify FARs, reassess forwarding behaviour, or update UP path configurations | #29 | To modify FARs, reassess forwarding behaviour, or update UP path configurations in cases of burstiness, idle prediction, or QoS profile mismatch (e.g. 5QI misalignment). |
|  | select/reselect suitable UPF | #31 | SMF could select/reselect suitable UPF for new PDU sessions, adjust N4 rules, etc. |
|  | update FAR | #33 | If the NF consumer is SMF, the SMF may update FAR based on the Analytics result without instruction from PCF. |
|  | Selective traffic handling Re-selections | #36 | Selective traffic handling, via node rules in a N4 Node level procedure where SMF instructs UPF to apply actions (drop, throttle, inhibit DLDR) on data flows that match the provided packet filters. |
| PCF | apply PCC rules | #22 | block abnormal traffic, or to optimize the user plane performance optimization by adjusting the QoS |
|  | Generates or updates the PCC rules | #23 | indicate to drop packet from the abnormal traffic filter or degrade the QoS level of the abnormal traffic filter |
|  | provision policies to SMF | #27 | e.g. executing traffic gating or shaping, enforcing bandwidth parameters or adjusting QoS parameters. |
|  | Adjust QER, 5QI assignment…… | #29 | adjust QER, 5QI assignment, shaping profiles, or DLDR configuration via PCC rules |
|  | update PCC rules at the SMF | #34 | it can instruct the SMF to gate the traffic or ask the SMF to release existing PDU session or/and reject future establishment request(s) |
|  | formulates UPF function set upgrade/downgrade strategies | #35 | PCF formulates UPF function set upgrade/downgrade strategies based on NWDAF’s analytics, considering UPF load, traffic anomaly risks, and service priorities. |
|  | Generates the PCC rule(s) | #36 | indicate to drop packet from the abnormal traffic filter or degrade the QoS level of the abnormal traffic filter. |
| OAM | configure SMF and/or UPF | #22 | block abnormal traffic or to increase capacity |
| AF | take action to correct abnormal traffic | #22 | control abnormal traffic to/from (IoT) devices or to reconfigure some mis-configured devices. |
| UPF | adjusting message frequency or apply downlink traffic suppression | #27 | control the number of DLDR (Downlink Data Report) messages sent to SMF by e.g. adjusting message frequency |
|  | traffic handling optimization | #30 | e.g. data rate limitation, traffic filtering, etc. |
|  | sends an N4 Session Report to SMF | #30 | SMF may decide to update N4 rules or updated SM policy. |
|  | allocate more resource adjust interface thresholds | #31 | detect malicious IP packets, allocate more resource before the bursts, adjust interface thresholds, etc. based on the Analytics ID. |

Observation 6: The analytics consumer is usually PCF, NWDAF, AF, or OAM.

Observation 7: SMF is the most frequent consumer, typically taking actions to configure or instruct UPF (e.g., reselection, FAR updates).

Observation 8: PCF consumes analytics mainly for policy control (e.g., apply/update PCC rules, QoS adjustments).

Observation 9: UPF as consumer may perform local optimizations (e.g., traffic filtering, allocate more resource).

Proposal 2: Document Observations 6–9 in the Agreed Principles for KI#1 UC#1.

2. Proposal

It is proposed to add the following contents to TR 23.700-04 1.0.2.

Start of Change

## 6.24	Solution #24: Collocation of UPF and NWDAF containing AnLF to support efficient performance of User Plane

### 6.24.1	High level principles

The NWDAF containing MTLF is used to train ML model of User Plane Traffic Exception.

The UPF is collocated with NWDAF containing AnLF and it is used to generate analytics result of User Plane Traffic Exception based on the ML model from the NWDAF containing MTLF.

Based on the analytics result of User Plane Traffic Exception, the consumer performs corresponding mitigation actions to handle burst, malicious or suspicious traffic.

### 6.24.2	Description

#### 6.24.2.0	General

This solution is to address Key Issue#2: AI/ML-assisted user plane traffic pattern and behaviour analysis to support efficient performance of User Plane.

Since UPFs in the network have plenty observed information and measurements data about UP traffic, the NWDAF containing MTLF can use these input data from UPFs to train ML model of User Plane Traffic Exception. This ML model will be provided to NWDAF containing AnLF to generate analytics of User Plane Traffic Exception by performing ML model inference process.

However, considering latency and big data volume of input data reporting from UPF to the NWDAF containing AnLF during ML model inference process, this solution proposes to collocate the UPF and the NWDAF containing AnLF together. In this case, the ML model from the NWDAF containing MTLF will be offloaded to UPF collocated with AnLF, then the UPF can generate analytics result itself and this result can assist the consumer to perform some corresponding mitigation actions, e.g. handle burst, malicious or suspicious traffic.

#### 6.24.2.1	Input Data

Table 6.24.2.1-1: UP Pattern data collected by NWDAF

| Information | Source | Description |
| --- | --- | --- |
| UE ID | SMF, UPF | Identifier of the UE (e.g. SUPI) associated with the collected UP-related data. |
| UE IP Address | SMF, UPF | IP address assigned to the UE for the observed UP-related data. |
| S-NSSAI | SMF, UPF | Single Network Slice Selection Assistance Information associated with the observed UP-related data. |
| DNN | SMF, UPF | Data Network Name corresponding to the PDU Session. |
| PDU Session ID | SMF | Identifier of the PDU Session. |
| UPF ID | UPF | Identifier of the UPF (e.g. IP address or FQDN). |
| UP related data | SMF, UPF | User Plane–related data collected for the session. May include one or more of the following: |
| > Traffic description | UPF | Detailed description of the observed traffic, e.g. IP tuple (SrcIP, DstIP, SrcPort, DstPort, Protocol). (IP 5-tuple or 3-tuple) |
| > Traffic direction | UPF | Indicates the direction of traffic, i.e. uplink or downlink |
| > Duration | SMF, UPF | Observation time range for the collected data (start and end timestamps). |
| > UPF interface info | SMF, UPF | Identifier and address of the UPF interface where traffic was observed (e.g. N3 for RAN side, N6 for DN side). |
| > Delay information | SMF, UPF | Latency observed at relevant interfaces, e.g. N6 (N6 Delay), N3/N9 (GTP-U path delay), N4 (PFCP Heartbeat delay), as defined in clauses 5.33.8, 5.24.5, and 6.2.2 of TS 29.244 [14]. |
| > PFCP session info | SMF, UPF | Summary of PFCP session context (e.g. PDR/FAR/QER/URR/BAR) status and any PFCP errors per clause 7.6 of TS 29.244 [14] for the UP pattern. |
| > Sampled info | UPF | Information related to the traffic sample rate, if sampling is used. |
| > Traffic volume | UPF | Total traffic size (including average, maximum, minimum). |
| > Number of packets | UPF | Total number of packets within the observed traffic. |

Note: Additional input data types and their descriptions can be considered as described in Solution #23.

#### 6.24.2.2	Output data

Table 6.24.2.2-1: Output Analytics of UP Traffic Exception statistics

| Information | Description |
| --- | --- |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Exception type | Indicates the exception, i.e. one of burst, malicious, suspicious. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > traffic description | Information describing the observed traffic for identification. May include one or more of the following: |
| >> UPF ID | Identifier of the UPF where the traffic was observed. |
| >> PDU session ID | Identifier of the PDU Session related to the traffic. |
| >> Qos flow ID | Identifier of the QoS Flow related to the traffic. |
| >> IP tuple | IP flow information, e.g. SrcIP, DstIP, SrcPort, DstPort, Protocol. (IP 5-tuple or 3-tuple) |

Table 6.24.2.2-2: Output Analytics of UP Traffic Exception predictions

| Information | Description |
| --- | --- |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Exception type | Indicates the type of exception, i.e. one of burst, malicious, suspicious. |
| > Exception level | Indicate the severity level of exception, e.g., low, medium, high. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > traffic description | Information describing the observed traffic for identification. May include one or more of the following: |
| >> UPF ID | Identifier of the UPF where the traffic was observed. |
| >> PDU session ID | Identifier of the PDU Session related to the traffic. |
| >> Qos flow ID | Identifier of the QoS Flow related to the traffic. |
| >> IP tuple | IP flow information, e.g. SrcIP, DstIP, SrcPort, DstPort, Protocol. (IP 5-tuple or 3-tuple) |
| > Confidence | Confidence of this prediction. |

#### 6.24.2.3	Example actions

Table 6.24.2.3-1: Examples of NF actions for exception traffic mitigation

| Exception type |  | Analytics consumer | Example actions of consumers |
| --- | --- | --- | --- |
| Burst traffic |  | SMF | The SMF may perform traffic splitting or traffic switching. - For the traffic splitting, the SMF may use ULCL mechanism to route some traffic to a new UPF to mitigate the load of the original UPF, as described in clause 4.3.5.4 of TS 23.502 [3]. - For the traffic switching, the SMF may change the PSA UPF from the UPF suffering from burst traffic to a new UPF for some PDU session to mitigate the load of original UPF. The SMF may perform different session modification procedures based on the different SSC mode of PDU session as described in clause 4.3.5 of TS 23.502 [3]. |
| Malicious traffic | UPF | The UPF may drop the traffic based on malicious traffic description. |
| Suspicious traffic | UPF | The UPF may send suspicious traffic information to SMF, and the SMF may perform traffic redirection for further traffic analysis or traffic isolation. - For the traffic redirection, the SMF may redirect the suspicious traffic to another network entity or a specific UPF for further traffic analysis or traffic isolation based on the redirection information which may be obtained from the interactions between SMF and PCF as described in clause 6.1.3.12 of TS 23.503 [4]. |
|  | SMF | The SMF may perform traffic redirection for further traffic analysis or traffic isolation. |

### 6.24.3	Procedures

#### 6.24.3.1	Support of efficient performance of User Plane by collocating UPF and NWDAF containing AnLF

Figure 6.24.3.1-1: Support of efficient performance of User Plane by collocating UPF and NWDAF containing AnLF

0.	[Optional] SMF could be the analytics consumer and subscribes to the NWDAF containing AnLF collocating with UPF for analytics result of User Plane Traffic Exception, by invoking the Nnwdaf_AnalyticsSubcription_Subscribe service operation. The subscription request may include the exception type to indicate the type of exception, e.g. burst, malicious and suspicious, and filter information, e.g. target time interval and reporting information.

1.	AnLF collocated with UPF subscribes to NWDAF containing MTLF for a trained ML Model of User Plane Traffic Exception, by invoking the Nnwdaf_MLModelProvision_Subscribe service operation.

2.	To train the ML model of User Plane Traffic Exception, the NWDAF containing MTLF collects input data from UPF(s), by invoking the Nupf_EventExposure_Subscribe service operation.

3-4.	The UPF(s) collects user plane traffic pattern data (e.g. UPF local data, N3 measurements, N6 measurements, as defined in table 6.24.2.1-1) and notifies the NWDAF containing MTLF by invoking the Nupf_EventExposure_Notify service operation.

NOTE 1:	To avoid much impacts on UPF performance and reduce signalling load for data reporting, the data reporting from UPF for ML model training can be done during off-peak time.

NOTE 2:	N3 measurements, e.g. UL/DL packet drop rate GTP-U, UL/DL packet delay GTP as defined in existing specifications don't have new RAN impacts.

Editor's note:	If it is feasible to train models outside peak hours but still update models quick enough for new observed malicious traffic is FFS. Whether and what other solutions to reduce data reporting load for model training are FFS.

5.	NWDAF containing MTLF trains ML model of User Plane Traffic Exception based on training data collected from data sources e.g. UPFs.

6.	The NWDAF containing MTLF provides the trained ML model to the AnLF collocated with UPF, by invoking Nnwdaf_MLModelProvision_Notify service operation.

7.	Based on the ML model from the NWDAF containing MTLF, the UPF collocated with AnLF collects user plane traffic pattern data, e.g. UPF local data, N3 measurements, N6 measurements, etc. as input data for ML model inference. Then the AnLF collocated with UPF generates analytics result of User Plane Traffic Exception based on the ML model. The consumer (e.g. the UPF) performs specific action related to the User Plane Traffic Exception.

The interactions between a collocated UPF and AnLF are performed using existing SBA services. For example, the UPF may invoke the Nnwdaf_AnalyticsSubscription service to request analytics results for a User Plane Traffic Exception, while the NWDAF containing the AnLF may invoke the Nupf_EventExposure service to collect the corresponding input data.

8.	[Optional] After receiving the analytics results of malicious or suspicious traffic, the UPF may perform actions related to the User Plane Traffic Exception, such as dropping malicious traffic or send the suspicious traffic information to the SMF, as described in table 6.24.2.3-1.

9a.	[Conditional] If the exception type of User Plane Traffic Exception analytics results is suspicious, the UPF may inform the SMF of the suspicious traffic information via an N4 message for further traffic analysis or traffic isolation. The suspicious traffic information could include suspicious traffic exception indication, the traffic description and the corresponding time.

9b.	[Conditional] After receiving the suspicious traffic information from UPF, the SMF may perform specific actions related to the suspicious traffic information, such as traffic redirection as described in table 6.24.2.3-1.

10a.	[Optional] If the SMF has subscribed to the analytics result of User Plane Traffic Exception in step 0, the NWDAF containing AnLF shall send the analytics results to the SMF to indicate the User Plane Traffic Exception.

10b.	[Optional] After receiving the User Plane Traffic Exception notification from the NWDAF containing AnLF, the SMF may perform specific actions based on exception type, such as traffic splitting and traffic switching for burst traffic, as described in table 6.24.2.3-1.

### 6.24.4	Impacts on services, entities and interfaces

NWDAF containing AnLF (collocated with UPF):

-	Retrieves ML model for Traffic pattern from NWDAF containing MTLF.

-	Generates analytics of User Plane Traffic Exception based on the ML model.

NWDAF containing MTLF:

-	Trains and provides ML model of User Plane Traffic Exception.

UPF:

-	Executes mitigation actions corresponding to the traffic exception notification.

-	Sends suspicious traffic information to the SMF.

SMF:

-	Executes mitigation actions corresponding to the suspicious traffic information.

Next Change

#### 7.1.2.2	Agreed Principles for Use Case #1

-	The service consumer of the analytics service may be SMF, PCF, OAM, UPF and AF. The consumer may take the output Analytics into account.

-	To provide analytics to the consumers, the NWDAF may provide the output analytics including the following output parameters:

-	Time information (e.g. time slot start and duration)

-	Identifier with different granularities (e.g. UE, PDU, UPF, Application).

-	Exception level to indicate severity level.

-	Exception type to classify anomalies.

-	To derive the output analytics, the following input data may be collected from the UPF to support the NWDAF-based analytics:

-	Information to identify the abnormal traffic.

-	The SMF as consumer may perform actions, e.g. instruct UPF (re-selections, modify FARs)

-	The PCF as consumer may perform actions, e.g. update PCC rules, apply PCC rules.

-	The UPF as consumer may perform actions, e.g. traffic filtering.

End of Changes