SA WG2 Meeting #171	S2-2508223

13 – 17 Oct, 2025, Wuhan, China	(revision of S2-2508125)

Source:	CATT

Title:	[WT#1.2, Localized Service Access] Support of Localized Service Access offered by PLMN in 6G

Document for:	Approved

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes to update the WT scope of supporting localized service as baseline and provide KIs for further discussion.

- Justifications

1.1 Motivation on Distribute Subnetwork

Compared to 5G networks, 6G networks will not only support connectivity services, but also support beyond connectivity services such as AI, computing, data, and sensing. With the growth of connectivity and beyond connectivity services, higher requirements will be placed on the network's capabilities. Different service scenarios also require diverse and differentiated network capabilities, performance and security. Vertical industries also hope that the network can provide access and service capabilities more closer to achieve lower communication latency. 5G local mobile communication networks to lag behind initial market expectations. The 6G networks should support on-demand network function customization, simplify network, alternatively supporting deployment options with smaller granularity and modular solution tailored for vertical use cases to address the above issues and new requirements.

On the other hand, the discussion on Local area service related content in SA1 has gone through three meetings, and the justification of the content is how 6G systems can support localized service access, but no conclusion has been reached yet.

Therefore, 6G networks should be considered as a more flexible, dynamic, and cost-effective network deployment solution, such as the coexistence of PLMN network and localized services provide by PLMN. PLMN network mainly meet the requirements of wide area coverage and ubiquitous new services, while localized services is a network established by operators for specific users, have specific services, capabilities and resources to meet specific requirements.

1.2 Gap Analysis:

In 5GS, Network Slicing, 5G LAN, and Non-public Network  allowing to meet the requirements from different users and verticals, However, Network Slice configuration is not flexible and agile enough to meet various dynamic service requirements of different LANs, 5G LAN is not suitable for service isolation and data security, SNPN network cannot be directly connected to PLMN network, PLMN network and PNI-NPN network share centralized network functions, and lack of isolation mechanism, which may not meet the high-quality service and security requirements of key data processing.

The above solutions cannot provide dynamic, flexible deployment and on-demand localized service access base on user’s requirements or intents. It cannot quickly and agilely introduce new services or network functions, and cannot guarantee the service continuity for UE in the event of UE mobility or network failure. It also cannot complete local authorization and verification without leaving the operator's network.Therefore, this contribution introduces the Distribute Subnetwork to support localized service access in 6G.

1.3 Proposal on Technical Impact:

6GS should support localized services:

- 6G Architecture, high-level procedures and NFs to support localized Service provide by PLMN.

- 6GS support redesign or reconstruct NFs in localized Service , e.g., NF tailor, customized, simplification and localization.

- Support for local authorization of UE.

- Support for service continuity due to UE mobility.

- At the SA2#170 meeting in Gothenburg Sweden, the S2-2508125 as the baseline of supporting localized service access was postponed. This paper mainly update the WT scope base on the S2-2508125 and gives the KI(s) on localized service access based on the SA2#170 progress.

- The main content of the update are about NFs, service continuity and start or stop for localized service. The following aspects are main reasons:

- Because different UE requires different localized services, the corresponding NF for their localized services will also be different.

- Besides UE mobility can trigger the service continuity, the failure of localized services currently serving the UE can also have a significant impact on the service continuity.

- The localized services provided by 6G networks for UE will not be predefined or pre-configured, but will be started or stopped as needed based on UE requirements.

**** First Change ****

Annex A.X. WT#1.2 Scope for localized service access

The scope of localized service access is to study the overall architecture design and enablers for localized service access in 6G, including:

NOTE 1: Support for localized service as defined in 5G is the starting point for the discussion on support of localized service access in 6G.

- 1)	Authorization and authentication of UE for localized service access

2)	NFs, interface and interaction procedure between PLMN network and networks providing localized service, or between networks providing localized service:

3)	Whether and how to support UE mobility between PLMN network and network providing localized services, or between networks providing localized service

4)	Whether and how to support service continuity for localized service, including access to localized service after leaving the original network providing localized service or network providing localized service failure.

5)  Whether and how to simply the network to support PLMN network providing localized services start or stop base on the users requirements.

**** Second Change ****

5.X. Key Issue #X: Support localized service access in 6GS

In order to support localized service access in 6GS, the following aspects are to be studied:

1)	Whether and how the Architecture, NFs and procedures enhancements to support localized service access in 6G.

2)	Whether and how to simply the network to support localized service access start or stop base on the users demand.

3)	Whether and  how to support the localized service discovery and selection.

4)	Whether and how to support the UE select and access the localized service.

5)	Whether and how to support the localized service switch to guarantee the services continuity for UE.

- 6)	Whether and how to support the local authorization and authentication of UE for localized service access .

**** End of Changes ****