SA WG2 Meeting #171	S2-2508225

13 - 17 October, 2025, Wuhan, China	(revision of S2-2508132)

Source:	vivo, MediaTek, CAICT, ZTE, China Mobile, TNO?

Title:	[WT#5 Data framework] WT scope and KI on 6G Data framework

Document for:	Approval

Agenda Item:	20.6.5

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution discusses and proposes work task scope and corresponding KI description for WT#5 on 6G Data framework.

- Discussion

The key open issue from last SA2 meeting is the division of responsibilities among SA2, SA5, and RAN WGs for the following data categories:

  - Data from Core network and 3rd AF

  - Data from UE (e.g. for Sensing, for AI in CN)

  - Data from RAN (e.g for Sensing, for AI in CN)

While some companies suggested SA2 should initially focus only on CN and AF data, with other data types (e.g., RAN or broader domains) requiring further discussion, the majority view is that SA2 can address CN, AF, UE, and RAN data, in line with SA2’s end-to-end architecture responsibility.

Issue: division of responsibilities among SA2, SA5, and RAN WGs needs to be identified.

- 5G general data collection for AIML in SA2 and SA5

Observation 1: Good Practice in 5G era for AIML data collection between SA2 and SA5.

SA2 and SA5 had a very good discussion on data collection topic in 5G, and the boundary between SA2 and SA5 is clear, SA2 is responsible for per UE per service data and SA5 is responsible for management data.

OAM can invoke data/analytics service provided by SA2 NWDAF, and vice versa, 5GC NWDAF can also invoke management data service provided by OAM.

- 5G data collection from RAN (for positioning, sensing and AI) in SA2

Fig 3: data collection for AI based positioning

Observation 2 for 5G AI based positioning:

In R19, SA2 studied how the 5GC NWDAF/LMF collects data from RAN for model training, inference and performance monitoring for AI based positioning, which reuses NRPPa procedures rather than via OAM.

Fig 4: data collection for RAN based sensing

Observation 3 for 5G RAN based sensing:

It is essential RAN should directly report sensing measurement data to SF rather than via OAM to allow Sensing Function to generate real-time Sensing result.

In addition, SA2 is responsible to define E2E procedure among UE, RAN and CN, sensing measurement data reporting via OAM may bring unnecessary coordination among WGs and may have side impact on 3GPP progress.

Fig 5: dataset/model parameter sharing (two side model)

Observation 4 for 5G dataset/model parameter sharing (for two side model training)

Assuming RAN directly reports positioning measurement data and sensing measurement data to CN NFs, it looks also feasible to report dataset/model parameter to Data Collection Function in CN.

- 5G data collection from UE in SA2

Fig 6: UE data collection via UP in SA2 (one side mode)

Observation 5 for UE data collection in SA2 via UP (for RAN AI UE-sided model training):

21 solutions were captured in TR 23.700-04 and partial interim conclusion has been reached.

Proposal 1: Assuming 6G has the same service requirements (e.g. for positioning, sensing, and AI) as in 5G and other potential ones (e.g. energy, etc.), SA2 shall start the study whether and how to collect data from UE and/or RAN under the 6G data framework considering the ongoing work in 5GA.

4）Further analysis on roles and responsibilities division among SA2, SA5 and RAN WGs:

If Data Producer is RAN node, CN NF or UE or AF, and Data consumer is CN NF, UE or AF, with Data Mgmt and storage in CN

- Data collection (UE->RAN) is in the remit of RAN WGs (see NOTE 1)

- Data transfer / distribution (UE-> Data Mgmt-> CN, RAN-> Data Mgmt-> CN, CN ->Data Mgmt-> CN ) is in the remit of SA2 (in coordination with RAN WGs)

- CN Data Mgmt (incl. storage and processing) is in remit of SA2 in coordination with RAN WGs

- Data access/ exposure (Data Mgmt-> UE,  Data Mgmt-> AF/ UE Server) is in the remit of SA2.

If either Data Producer or Data Consumer is OAM, with data Mgmt and storage in OAM

- Data collection (UE->RAN) is in the remit of RAN WGs (see NOTE 1)

- Data transfer/ distribution (RAN-> Data Mgmt-> OAM ) is in the remit of SA5 (in coordination with RAN WGs and SA2)

- OAM Data Mgmt (incl. storage and processing) is in remit of SA5 in coordination with RAN WGs

- Data access/ exposure (CN-> Data Mgmt->  OAM) is in the remit of SA2 in coordination with SA5

- Data access/ exposure (OAM-> Data Mgmt-> CN) is in the remit of SA5 in coordination with SA2

NOTE 1 : Harmonized mechanism for Data collection/ Data collection configuration to be defined and coordinated by RAN WGs.

If Data Producer is RAN node and Data consumer is UE

- Data access/ exposure (RAN-> UE ) is in the remit of RAN WGs.

Proposal 2:  Subject to the division of roles and responsibilities among WGs for 6G data framework as above, i.e. Green in RAN to lead, Cyan SA2 to lead and yellow SA5 to lead.

- Potential options for high level procedure using 6G data framework

Option 1: data source (e.g. UE, RAN, etc.) provides service data (e.g. measurements data for sensing and AI service) to data framework function(s) in 6G CN first, then data framework function(s) forward service data to service function (e.g. SF or AI function).

Option 2: data source (e.g. UE, RAN, etc.) provides service data (e.g. measurements data for sensing and AI service) to service function (e.g. SF or AI function) first, then service function provides service data to data framework function(s) later.

Both options are two potential procedures from high level, which will be discussed during solution phrase:

  - Whether and how 5GC DCCF can be reused as data framework function is solution oriented, which shall be discussed during study phase;

  - Whether and how the common protocol stack e.g. RAN-CN is used for data collection shall also be discussed during study phase

Proposal 3: WT scope has not ruled out any option, but no need to start solution discussion/evaluation at this moment.

2. proposal

It is proposed to approve the following WT scope in TR23.801-01.

**** First Change ****

A.X 	Work Task X: 6G data framework in SA2

This work task aims to design end to end 6G system architecture for data framework in SA2 that can be used for handling of data generated in the 6G system to support services deployed over the 6G network such as AI and sensing.

The 6G data framework in SA2 can handle the following data that:

- Data producer: UE, 6G RAN, 6G CN NF and AF

- Data consumer: UE, 6G RAN, 6G CN NF and AF

- Controlled and stored by: 6G CN

The scope of the WT includes the following aspects:

- Design end to end 6G system architecture for data framework to support efficient and scalable data handling functionalities, e.g.:

- data collection

- data transfer and distribution

- data processing within 6G CN e.g. labelling data for origin, purpose, context and quality/validity as well as handling metadata such as type and structure of data parameters produced by NFs

- data storage within 6G CN

- data exposure (including data access and authorization) within 6G system and to outside of 6G system

NOTE 1:	Coordination with SA5 may be required to determine whether and how to align with the data handling mechanisms supported in the management domain (i.e., OAM) and charging aspects of data exposure.

NOTE 2:	Coordination with RAN WG is required for data collection from or via RAN.

NOTE 3:  Consideration on MNO’s requirements of visibility and controllability for standardized data during data handling processes is required. And data handling processes can be service-differentiated.

NOTE 4:	The outcome of 5GS studies on data collection and data transfer will be considered as starting point on those aspects.

- Study from SA2 architectural and system level on user consent, privacy, security and data governance to support the 6G data framework.

NOTE 5:	Coordination with SA3 is required for user consent, privacy, security and data governance related topics.

- Study whether and how to support quality of data which is processed and provided based on the requirements from data requester.

**** Second Change (all new text)****

5.X	Key Issue #X: 6G data framework in SA2

This key issue will investigate the following aspects:

- How to design 6G architecture to support end to end data framework for efficient and scalable data handling, which may include:

- identifying new 6G CN level functionalities

- investigating requirements on interface(s)

- enhancing CP/UP (incl. whether to dimension beyond the current CP/ UP)

- How to define related procedures to enable efficient and scalable data handling, including:

- data collection from data producer(s), e.g. UE, 6G RAN, 6G CN NF and AF

- data transfer and distribution from data producer(s) to data consumer(s), e.g. UE, 6G RAN, 6G CN NF and AF

- data processing with 6G CN

- data storage within 6G CN

- data exposure within 6G system and to outside of 6G system

NOTE 1:	Coordination with SA5 may be required to determine whether and how to align with the data handling mechanisms supported in the management domain (i.e., OAM) and charging aspects of data exposure.

NOTE 2:	Coordination with RAN WG is required for data collection from UE or RAN.

NOTE 3:  Consideration on MNO’s requirements of visibility and controllability for standardized data during data handling processes is required. And data handling processes can be service-differentiated.

NOTE 4:	The outcome of 5GS studies on data collection and data transfer will be considered as starting point on those aspects.

- How to define procedures on data governance, user consent and/or privacy from architecture and system level to enhance privacy protection for 6G data framework, which may need to meet the following requirements:

- dynamic and on-demand data privacy protection for users

- granular levels in real-time for different 6G services

- governance of data exposure to 3rd party and cross-domain governance

NOTE 5: Coordination with SA3 is required for user consent, privacy, security and data governance related topics.

- Whether and how to support quality of data which is processed and provided based on the requirements from data requester, including.

- defining new metrics and corresponding policy control mechanism to ensure data quality requested by data consumer

NOTE 6:	Coordination with WT#1.2 is expected.

**** End of Changes ****