3GPP TSG-WG SA2 Meeting #171	S2-2508240

Wuhan, CN, 13th – 17th Oct, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	KI#5: Conclusion

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: Conclusion on KI#5 on Sensing Result Exposure is proposed in this document.

# 1. Introduction

Based on the discussions and analyses in S2-2508235 and the architectural conclusion of KI#1 in S2-2508236, this paper provides the conclusion on KI#5 on Sensing Result Exposure to Support Sensing Service.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-14.

# * * * * First change * * * * all new text

### 7.1.5	Agreed Principles for KI#5 Sensing Result Exposure

The following principles are agreed for KI#5:

a)	The Sensing Control Function provides the Sensing Results containing the list of object(s) sensed, the object characteristics (e.g., location, velocity) and the associating geographical coordinates within the Target Sensing Service Area to the Sensing Service Consumer.

NOTE 1:	Other use cases (e.g., intruder detection, object tracking) based on this can be further determined in the normative phase.

b)	Sensing results provided by the Sensing Control Function can be one time, event-triggered or periodic.

c)	The results exposed can permit for the Application Function to request a subsequent sensing service (e.g., tracking) associated to at least one object that is contained in a Sensing Result.

d)	Existing NEF and SBI interface are reused to support Sensing Result Exposure.

The Figure 7.1.5-1 depicts the procedure for exposure.

Figure 7.1.3-1: Procedure for exposure.

1.	The AF sends a Sensing Service Request to the Sensing Control Function via NEF as described in KI#2 and above.

2.	The NEF send the Sensing Service Request to the selects the Sensing Control Function(s) as described in KI#2 and above

3.	The SCFperforms the authorisation as described in KI#2, selects the gNB and send the request as described in KI#2, KI#3 and KI#4;

4.	Sensing Control Function sends the Sensing Service Request to the selected SPC as described in KI#2. The request is defined in KI#4. This step is left to the implementation in this realease.

5.	gNB send the sensing data to the Sensing Control Function as defined in KI#4

6.	The SPF send the response including the Sensing Results to the SCF. This step is left to the implementation in this realease;

7.	Sensing Control Function sends the Sensing Service Response to AF via NEF as described above

8.	The NEF forward the Sengin Result to the AF as described above

# * * * * Second change * * * *

# 8	Conclusions

Editor's note:			This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

There should be a Topics for further consideration clause per Key Issue. It is recommended that this is used e.g. to capture common issues that need to be resolved for multiple solutions.

Principles in clause 7.1.5 are agreed as the conclusion.

# * * * * End of changes * * * *