3GPP TSG-SA WG2 Meeting #171                                                                   S2-2508243

Wuhan, China, October 13 – 17, 2025

Source:	vivo

Title:	[WT#1.4, Voice Services for 6G] Key issue of Voice Services for 6G

Document for:	Approval

Agenda Item:	20.6.1.4

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: this contribution introduces the key issues of Voice Services for 6G which is the sub work task of WT#1.4 (Voice Services for 6G) based on the baseline tdoc S2-2508128.

1. Justifications

During SA2#170 meeting, the baseline paper S2-2508128 contains following contents:

5.P. Key Issue #P: Voice Services for 6G

This Key Issue will investigate how to support voice services for the 6G system (using 5GS as the starting point for discussion) – the following aspects will be studied:

- How a UE supporting 6G RAN selects and connects to a network for voice service.

- Whether and how to update the IMS specification to support an IPCAN in the 6GS.

- How the 6GS supports IMS based voice over the 6G access network.

Editor’s Note: It is TBD if voice services for 6G NTN will be studied as part of this Key Issue or by a Key Issue related to WT7.

Editor’s Note: It is TBD if interworking, migration, and service continuity (including the issue of “fallback”) for voice services will be studied as part of this Key Issue or by a Key Issue related to WT2.

Editor’s Note: It is still to be determined if other capabilities will be considered as part of this KI.

Based on the baseline paper, we have following proposals:

- The relationship between WT 1.4 and WT 2

The second Editor’s Note is about the relationship between WT 1.4 and WT 1.2, we have following proposals for this Editor’s Note:

In 5GS, as specified in clause 4.13.6.1	EPS fallback for IMS voice of TS 23.502:

Figure 4.13.6.1-1: EPS Fallback for IMS voice

1.	UE camps on NG-RAN in the 5GS and an MO or MT IMS voice session establishment has been initiated.

2. Network initiated PDU Session modification to setup QoS flow for voice reaches the NG-RAN (see N2 PDU Session Request in clause 4.3.3).

3.	NG-RAN is configured to support EPS fallback for IMS voice and decides to trigger fallback to EPS, taking into account UE capabilities, indication from AMF that "Redirection for EPS fallback for voice is possible" (received as part of initial context setup, handover resource allocation or path switch request acknowledge as defined in TS 38.413 [10]), network configuration (e.g. N26 availability configuration) and radio conditions. If NG-RAN decides not to trigger fallback to EPS, then the procedure stops here and following steps are not executed.

5.	NG-RAN initiates either handover (see clause 4.11.1.2.1), or AN Release via inter-system redirection to EPS (see clause 4.2.6 and clause 4.11.1.3.2), taking into account UE capabilities. The SMF+PGW-C reports change of the RAT type if subscribed by PCF as specified in clause 4.11.1.2.1, or clause 4.11.1.3.2.

Based on above description in step 2 and 3, the “fallback” procedure is triggered during the IMS session negotiate and QoS flow for voice setup procedure. Based on above description in step 5, the “fallback” can be handover or redirection.

For the 6G, the same logic and be used to separate WT 1.4 and WT 2:

- WT 1.4 studies how to trigger the “fallback” procedure,

- WT 2 studies how to perform interworking from 6GS to 5GS/EPS.

Proposal 1: WT 1.4 studies how to trigger the “fallback” procedure, WT 2 studies how to perform interworking from 6GS to 5GS/EPS.

- How the 6GS supports IMS based voice over the 6G access network

When LTE was first commercially launched, CS Fallback was used to support voice services. Similarly, when 5G Standalone (SA) was initially commercially deployed, EPS Fallback was adopted for voice support. It is challenging to natively support voice services when a new "G" (generation of mobile communication technology) is first launched. This is because voice services require high Quality of Service (QoS) performance, and achieving native voice support would delay the commercial rollout. Therefore, using "fallback" methods to enable voice services is a reasonable approach.

Due to the varying speeds of 5G deployment among different operators, their timelines for supporting VoNR also differ. If the VoNR are still not support when the 6G is commercially launched, the “fallback” to EPS will be the best way to support voice services. So, to satisfied different requirement for different operators, it is reasonable to study how to perform “fallback” from 6GS to EPS

Proposal 2: WT 1.4 should study how to satisfied different requirement among different operators.

2. Proposal:

It is proposed to add following proposals to TR 23.801-01 V0.1.0.

**** First Change (based on paper S2-2508128) ****

5.P. Key Issue #P: Voice Services for 6G

This Key Issue will investigate how to support voice services for the 6G system (using 5GS as the starting point for discussion) – the following aspects will be studied:

- How a UE supporting 6G RAN selects and connects to a network for voice service.

- Whether and how to update the IMS specification to support an IPCAN in the 6GS.

- How the 6GS supports IMS based voice over the 6G access network, including:

-	How to supports native IMS voice over PS session in 6GS;

-	How to supports IMS voice over PS session via 5GS fallback;

-	Whether and how to supports IMS voice over PS session via EPS fallback.

NOTE X: This Key Issue should study how to perform the trigger of "fallback", and WT 2 should study how to perform interworking from 6GS to 5GS/EPS.

Editor’s Note: It is TBD if voice services for 6G NTN will be studied as part of this Key Issue or by a Key Issue related to WT7.

Editor’s Note: It is still to be determined if other capabilities will be considered as part of this KI.

**** Second Change ****

5.X. Key Issue #X: Migration and Interworking

Study migration and interworking, including:

NOTE X: The interworking from 6GS to 5GS or from 6GS to EPS may be used to perform "fallback" procedure for IMS voice service when native IMS voice over PS session in 6GS is not support for an operator.

**** End of Changes ****