SA WG2 Meeting #S2-171	S2-2508246

13 - 17 October, 2025, Wuhan, China

Source:	Samsung

Title:	KI#1 Interim Agreements

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 /Rel-20

Abstract of the contribution: This contribution proposes the interim agreements for AIML_CN KI#1.

- Discussion

This pCR proposes the interim agreements with the consideration of RAN-provided input, solution, and discussion so far.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel (Note 7) | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel (Note 7) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer (Note 1) | No AI/ML specific controllability | FFS: level of controllability (Note 5) | Full controllability | Full controllability |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure or FFS other procedures | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS on level of visibility (Note 5) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) |

- Proposal

It is proposed to adopt the following changes into TR 23.700-04.

*** Start of the change ***

7.1.1	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#1. This may be populated directly or e.g. also when a topic in clause 7.2.1 gets resolved and a principle is agreed.

The interim agreements on principles for KI#1 are as follows:

First termination entity of the data transfer:

- A dedicated NF, located inside the CN for UE-side model, is the first termination entity (i.e., the first termination NF) for the UP tunnel for UE-side model data collection.

- The first termination NF supports the UP connection establishment with the UE.

- The first termination NF supports a dedicated protocol for AIML-specific collected UE data transfer

Editor’s Note:	It is FFS whether a new NF (DCF) or another 5GC NF will be the first termination entity and support the data transfer over UP for UE data collection.

AIML-specific collected UE Data Transfer Path via UP and its protocol layer for data transfer:

Path between UE and the first termination NF:

- A UP path is used between the UE and the first termination NF, for transferring standardized collected data from the UE using PDU connectivity service provided by a PDU session.

- Dedicated DNN/S-NSSAI may be used for data transfer over UP for UE data collection for model training. The mechanism of existing URSP rule can be reused with specifying specific traffic descriptor(s) and Route Selection Descriptor(s). The UE establishes a new PDU Session, or modifies an existing PDU Session which has been established for UE data transfer, to transfer the standardized UE data, based on URSP rules.

Protocol aspects for data transfer:

- A dedicated protocol used for AIML-specific UE Data Transfer between the UE and the first termination NF over UP tunnel is assumed.

- The protocol over UP connection for data transfer between the UE and the first termination NF is needed.

- The protocol shall be able to be carried over the user plane transport layers.

How to provision UE with the information required for establishment of the UP transfer path to the first termination NF:

- NAS signaling is used to provide the user plane information required for UE to discover the first termination entity and establish the UP connection to it.

- The user plane information includes the user plane address of the first termination NF (e.g., IP address or FQDN).

- The UP connection establishment may be triggered by DCF or UE.

Path between the first termination NF and the UE-side model training/OTT server:

- The existing exposure framework will be reused for the path between the first termination NF and the UE-side model training/OTT server.

Editor’s Note:	It is FFS how the UE Data is transferred from the first termination NF to the UE-side model training/OTT server.

Controllability of MNO on data transfer:

MNO can have controllability via NAS signaling procedure, which may be initiated by the first termination NF for the following operations:

- Indicate UE to initiate UP connection establishment with UE

- Subject to user consent, the first termination NF controls data transfer over UP, including trigger and terminate data transfer. The DCF may trigger or terminate the data transfer based on its internal logic or request from the UE model training entity server.

NOTE:	The UE can reject the request for the data transfer or cancel the data transfer during the process based on, e.g., user’s input or battery state. The detailed procedure will be specified during normative work.

Editor’s Note:	It requires coordination with RAN regarding the details on how the first termination entity control the data transfer over UP.

How to differentiate the traffic for UE Data Transfer from the UE’s regular traffic (i.e., application traffic over UP) in the 5GC:

- The traffic UE data transfer for UE-side model training may be differentiated from other regular application traffic on a per PDU Session basis (with a dedicated DNN/S-NSSAI) and/or on a per IP flow basis (e.g., based on destination IP address of the data packet).

  - Per PDU Session basis: The use of dedicated DNN/S-NSSAI may be enforced by reusing the existing URSP mechanism.

  - Per IP flow basis (or SDF basis): The destination IP address of the first termination NF may be used for differentiating the UE data collection traffic from other regular application traffic.

How the UE knows about what the standardized data to be transferred are: FFS

Editor’s Note:	It is FFS how the UE knows about what the standardized data to be transferred are and it may require coordination with RAN

Visibility of data content in MNO and Data format

As described in S2-2500025/R2-2411152, visibility of data content signifies that the MNO is able to be aware of, access, and comprehend the content of the collected/reported data without the need of SLA. The full visibility allows the MNO to verify/match the data specified/configured to be collected and the data that is being reported.

In this study, if the collected data at the UE refers to the data content fully standardized by RAN1 and if the data can be transferred via the protocol (used by UE and the first termination NF), then the MNO can access the data transferred to the first termination NF.

If the first termination NF is able to be ware of the data configured to be collected, then the first termination NF can match the data configured to be collected and the data actually transferred from the UE.

Data format is not the scope of SA2.

*** End of the change ***