SA WG2 Meeting #172	S2-2508249

# 13 - 17, October, 2025, Wuhan, China

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1513 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on Historical data and analytics via notification procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2025-9-8 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The descriptions (e.g. step in 1c, step 6b) of Figure 6.2B.3-1 imply that the Analytics Consumer (positioned at the far left in the Figure 6.2B.3-1) can send an Analytics subscription request to the NWDAF. However, the service operation (i.e. Nnwdaf_DataManagement_Subscribe) shown in Step 1b that the Analytics consumer invoke was designed solely for data subscription, not for Analytics subscription. Please see the NOTE of clause 5A.1: NOTE: Nnwdaf_DataManagement service can be used to collect historical data or runtime data. For collecting historical and runtime analytics, Nnwdaf_AnalyticsSubscription service is used. This creates a discrepancies and logical contradictions between steps in clause 6.2B.3. Besides, many steps in 6.2B.3 should be ‘data and/or analytics’, but now the descriptions and figure only include ‘data’ or ‘data or analytics’. On the other hand, the DCCF or NWDAF only receives DataManagement subscription request, they do not provide the requests, so the description at the start of step 1 is wrong. Moreover, the descriptions of ‘store the notifications’ to ADRF seems not very appropriate, the ADRF does not need to store all the information in notification (e.g. the ADRF does not need to store notification correlation ID). It is proposed to change ‘store the notifications’ to ‘store data and/or analytics’. Finally, to implement the call-flow in 6.2B.3, the Analytics consumer needs to send ADRF ID and storage handling information to the NWDAF, and may receive the storage Approach determined by NWDAF, DCCF or ADRF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify Figure 6.2B.3-1 to add Analytics subscription related services, change many places to ‘data and/or analytics’, and other editorial corrections. Refine the descriptions in clause 6.2B.3. Add ADRF ID and Storage Handling information as optional input in clause 7.2.2. Add storage approach as optional input in clause 7.2.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The defined procedure in clause 6.2B.3 can not fully working. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.3, 7.2.2, 7.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

6.2B.3	Historical Data and Analytics Storage via Notifications

The procedure depicted in figure 6.2B.3-1 is used by consumers (NWDAF, DCCF) to store received notifications in the ADRF. The consumer requests the ADRF to initiate a subscription for data and/or analytics. Data and/or analytics provided in notifications as a result of the subsequent subscription by the ADRF are stored in the ADRF.

The consumer may include in the subscription request the DataSetTag attribute defined in Table 6.2B-1.

Figure 6.2B.3-1: Historical Data and Analytics Storage via Notifications

0a-c	NWDAF, DCCF or ADRF are configured with default operator storage policies as described in clause 5B.1.

1a-d.	Based on reception of a DataManagement subscription request and/or Analytics subscription request (e.g. see clause 6.2.6.3.2 and clause 6.2.6.2 for DataManagement subscription request,  clause 6.1.1.1 for Analytics subscription request) or based on local configuration, the DCCF or the NWDAF determines that data and/or Analytics are to be stored in an ADRF. The subscription request(s) may contain Storage Handling information with a requested ADRF storage lifetime and a request to be notified before data are deleted from the ADRF.

2a-b.	The DCCF or the NWDAF determines the ADRF where data and/or analytics needs to be stored and requests that the ADRF subscribes to receive notifications. The determination may be made based on configuration or information supplied by the data consumer as described in clauses 6.1.4 and 6.2.6.3. The request to the ADRF specifies the data and/or analytics to which the ADRF will subscribe by invoking the Nadrf_DataManagement_StorageSubscriptionRequest service operation. The request may include the DataSetTag attribute which the data records are to be associated to when stored by ADRF. If the Storage Policy is not configured on the NWDAF or DCCF, the NWDAF or DCCF sends the Storage Handling Request to the ADRF. The NWDAF or DCCF may provide notification endpoint information to the ADRF for use by the ADRF to send notifications (implicit subscription) alerting the DCCF or NWDAF that data are about to be deleted (see step 12).

3.	[Optional] The ADRF may, based on implementation, determine whether the same data and/or analytics is already stored or being stored, based on the information sent in step 2 by the consumer. If the DataSetTag attribute is included for data and/or analytics already stored or being stored, then ADRF associates the data records with such DataSetTag.

3a-c.	[Optional] Based on Storage Handling information and Storage Policy, the ADRF, DCCF or NWDAF determines the Storage Approach (lifetime for storing data and whether consumer is notified prior to data deletion).

4a-b.	[Optional] If the data and/or analytics is already stored and/or being stored in the ADRF, the ADRF sends Nadrf_DataManagement_StorageSubscriptionRequest Response message to the consumer indicating that data and/or analytics is stored. The ADRF includes the Storage Approach if determined in step 3c.

5a-b.	The DCCF or NWDAF sends a subscription response to the Data or Analytics Consumer. The response may contain the Storage Approach determined by the ADRF, NWDAF or DCCF.

6a-b.	ADRF subscribes to the DCCF or the NWDAF to receive notifications, providing its notification endpoint address and a notification correlation ID. In step 6b, the ADRF uses Nnwdaf_DataManagement_Subscribe to obtain data from the NWDAF or Nnwdaf_AnalyticsSubscription_Subscribe to obtain analytics from the NWDAF.

7.	The DCCF, the MFAF or the NWDAF sends Analytics or Data notifications containing the notification correlation ID provided by the ADRF to ADRF notification endpoint address. The Analytics or Data notifications shall contain timestamp. The ADRF stores the data and/or analytics that provided inside notifications.

Conditional on DCCF or NWDAF determining to stop storing data and/or analytics

8a-b.	The DCCF or the NWDAF determines to stop storing data and/or analytics in the ADRF.

9a-b.	The DCCF or the NWDAF requests that the ADRF unsubscribes to receive notifications.

10a-b.	The ADRF sends a request to the DCCF or the NWDAF to unsubscribe to data and/or analytics notifications.

The NWDAF may interact with the Data Source and the DCCF may interact with the Data Source and/or MFAF. Delivery of notifications from the DCCF/MFAF or NWDAF to the ADRF are subsequently halted. If Nnwdaf_AnalyticsSubscription_Subscribe was used in step 6b, then the Nnwdaf_AnalyticsSubscription_Unsubscribe is used in step 10b.

Conditional on ADRF Managing the Storage Approach

11.	The ADRF determines that the lifetime of the stored data and/or analytics has expired (according to the Storage Approach).

12.	If indicated by the Storage Approach, the ADRF sends a notification alerting the DCCF or NWDAF that data and/or Analytics are about to be deleted.

NOTE:	This is an implicit subscription.

Conditional if the Storage Approach is based on Default Operator Policies provisioned on the NWDAF or DCCF (see step 3a-c and clause 5B.1)

13.	The DCCF or NWDAF indicate in the response to the ADRF whether they will retrieve the data and/or Analytics.

14.	The DCCF or NWDAF may retrieve the data and/or analytics from the ADRF as described in clause 5B.1.

Conditional if the Storage Approach is based on a Storage Handling Request received from the Data or Analytics Consumer (see step 3a-c and clause 5B.1)

15a-b. The NWDAF or DCCF sends a notification to the Data or Analytics Consumer alerting it that the data or analytics are about to be deleted.

16a-b. The Data or Analytics Consumer indicates in the response to the NWDAF or DCCF whether it will retrieve the data and/or Analytics.

17. The DCCF or NWDAF indicate in the response to the ADRF whether the consumer will retrieve the data and/or Analytics.

18.	The Data or Analytics Consumer retrieves the stored data and/or analytics as described in clause 5B.1.

Conditional on DCCF or NWDAF Managing the Storage Approach

19a-b.	The NWDAF or DCCF determines that the lifetime of the stored data and/or analytics has expired (according to the Storage Approach).

Conditional if the Storage Approach is based on Default Operator Policies provisioned on the NWDAF or DCCF (see step 3a-c and clause 5B.1)

20.	The NWDAF or DCCF optionally retrieve the data and/or analytics from the ADRF as described in clause 5B.1.

21.	The NWDAF or DCCF request that the data and/or analytics be deleted from the ADRF.

Conditional if the Storage Approach is based on a Storage Handling Request received from the Data or Analytics Consumer (see step 3a-c and clause 5B.1)

22a-b.	The NWDAF or DCCF sends a notification to the Data or Analytics Consumer alerting it that the data and/or analytics are about to be deleted.

23a-b.	The Data or Analytics Consumer indicates in the response to the NWDAF or DCCF whether it will retrieve the Data and/or Analytics.

24.	The Data or Analytics Consumer retrieves the stored data and/or analytics as described in clause 5B.1.

25.	The NWDAF or DCCF request that the data and/or analytics be deleted from the ADRF.

26.	The ADRF deletes the data and/or analytics if:

-	the Storage Approach in the ADRF indicates alerting the consumer is not required prior to data and/or analytics deletion;

-	in Steps 13 and/or 17 the DCCF or NWDAF indicated data and/or analytics will not be retrieved prior to deletion;

-	data and/or analytics retrieval in steps 14 and/or 18 has completed or

-	A request to delete data and/or analytics was received in steps 21 or 25.

If the ADRF received a response from the NWDAF or DCCF in steps 14 and/or 17 indicating data and/or analytics will be retrieved but retrieval is not initiated before an adequate fixed time has elapsed, the ADRF may autonomously delete the data or analytics.

* * * Next Change * * *

7.2.2	Nnwdaf_AnalyticsSubscription_Subscribe service operation

Service operation name: Nnwdaf_AnalyticsSubscription_Subscribe

Description: Subscribes to NWDAF analytics and optionally Analytics Accuracy Information with specific parameters.

Inputs, Required:

-	(Set of) Analytics ID(s) as defined in Table 7.1-2;

-	Target of Analytics Reporting;

-	Notification Target Address (+ Notification Correlation ID);

-	Analytics Reporting Parameters (including Analytics target period, etc.).

NOTE 1:	Target of Analytics Reporting can be provided per individual Analytics ID.

Inputs, Optional:

-	Analytics Filter Information;

-	Time window for historical analytics;

-	Subscription Correlation ID (in the case of modification of the analytics subscription);

-	Preferred level of accuracy of the analytics;

-	Preferred level of accuracy per analytics subset;

-	Reporting Thresholds;

-	Maximum number of objects requested (max);

-	Preferred order of results;

-	Maximum number of SUPIs requested (SUPImax);

-	Time when analytics information is needed;

-	Analytics Metadata Request;

-	(Set of) NWDAF identifiers used by the NWDAF service consumer when aggregating multiple analytic subscriptions;

-	Dataset Statistical Properties;

-	Output strategy;

-	Data time window;

-	Consumer NF's serving area or NF ID;

-	Information of previous analytics subscription, i.e. NWDAF identifier (i.e. Instance ID or Set ID), Analytics ID(s) (including SUPIs and Analytics Filter Information for UE-related Analytics) and Subscription Correlation ID;

-	Use case context.

-	Analytics Accuracy Request information.

-	Analytics Feedback Information.

-	ADRF ID where historical Analytics to be stored.

-	Storage Handling information as described in clause 5B.1.

-	QoS parameter set(s) and corresponding value(s), optionally a requested QoE.

NOTE 2:	Analytics Filter Information, Reporting Thresholds, maximum number of objects requested (max), maximum number of SUPIs requested (SUPImax), Analytics Metadata Request, Dataset Statistical Properties, Output strategy, Data time window and time when analytics information is needed can be provided per individual Analytics ID.

NOTE 3:	Analytics Feedback Information only can be included in modification request for the existing analytics subscription.

NOTE 4:	Other input parameters specific for different analytics ID can be also needed, as specified in the corresponding clause for each analytics ID.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: First corresponding Analytics report is included, if available and if analytics consumer requested immediate reporting (see clause 4.15.1 of TS 23.502 [3]).

NOTE 5:	When the Target of Analytics Reporting is a SUPI or a GPSI then the subscription may not be accepted, e.g. user consent is not granted and an error is sent to the consumer. When the Target of Analytics Reporting is an Internal Group Id, or a list of SUPIs/GPSIs or any UE, no error is sent, but a SUPI or GPSI is skipped if user consent is not granted.

* * * Next Change * * *

7.2.4	Nnwdaf_AnalyticsSubscription_Notify service operation

Service operation name: Nnwdaf_AnalyticsSubscription_Notify

Description: NWDAF notifies the consumer instance of the analytics and optionally Analytics Accuracy Information that has subscribed to the specific NWDAF service. See also clause 6.1.3 for contents of the Analytics Exposure.

Inputs, Required: Notification Correlation Information: this parameter indicates the Notification Correlation ID that has been assigned by the consumer during analytics subscription.

Inputs, Optional:

-	Set of the tuple (Analytics ID, Analytics specific parameters): this parameter shall be present if output analytics are reported.

-	Timestamp of analytics generation (required when ADRF is deployed).

-	Validity period.

-	Confidence

-	Revised waiting time.

-	Analytics Metadata Information.

-	Analytics Accuracy Information.

-	Stop Analytics Output Consumption indication.

-	Stop Analytics Output Consumption time window.

-	Resume Analytics Output Consumption indication.

-	Termination Request: this parameter indicates that NWDAF requests to terminate the analytics subscription, i.e. NWDAF will not provide further notifications related to this subscription, with cause value (e.g. user consent revoked, NWDAF overload, UE moved out of NWDAF serving area, etc.).

-	Accuracy Information Termination.

NOTE 1:	Validity period can also be provided as part of Analytics specific parameters for some NWDAF output analytics.

-	Subscription Change Notification Correlation ID: this parameter shall be present if the notification is for informing the assignment of a new Subscription Correlation Id by the NWDAF. It is set to the old Subscription Correlation ID.

-	Subscription Correlation ID: this parameter shall be present if the notification is for informing the assignment of a new Subscription Correlation Id by the NWDAF. It is set to a new Subscription Correlation ID assigned by the NWDAF.

NOTE 2:	Other input parameters specific for different analytics ID can be also needed, as specified in the corresponding clause of each analytics ID.

-  Storage Approach

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

* * * End of Changes * * *