3GPP TSG-SA WG2 Meeting #171	S2-2508274

Wuhan, China, 13th Oct 2025 - 17th Oct 2025

Source:	OPPO

Title:	Interim agreement update for KI#1

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes the agreed principles update for KI#1.

- Discussion

In the last meeting, some basic principles have been captured about how the UE can send the collected data to the 5GC NF based on the UP path.

Based on the current study progress, this paper adds more interim principles for KI#1 including which NF in the 5GC should support the full visibility and full controllability of the UE data collection, how to send the 5GC NF UP information to UE to support the data transfer via UP connection, etc.

- Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-04.

* * * Start of Change * * * *

7	Interim agreements

7.1	Agreed Principles

7.1.1	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#1. This may be populated directly or e.g. also when a topic in clause 7.2.1 gets resolved and a principle is agreed.

The interim agreements on principles for KI#1 are as follows:

-	A UP path is used between the UE and a 5GC NF, for transferring standardized collected data from the UE using PDU connectivity service provided by a PDU session.

-	Dedicated S-NSSAI/DNN is used to differentiate the traffic of collected UE data from the UE regular traffic per PDU Session level.

-	The UE establishes a new PDU Session, or modifies an existing PDU Session which has been established for UE data transfer, to transfer the standardized UE data, based on URSP rules.

-	A new network function in the 5GC named Data Collection Function (DCF) is introduced to support the UE data collection.

-	The UE side model training server/UE model training entity server (UMTES) configure the DCF about the UE data collection information including the Event ID and the corresponding data type the UE need to be measured and reported.

-	The DCF provision the UE Data Collection information to the UE via control plane signalling including the Event ID and the corresponding data type the UE need to be measured and reported.

-	The UE side model training server/UE model training entity server sends the UE data collection request to the DCF including the AoI and the Event ID.

-	The DCF selects the UEs to be used for the UE data collection.

-	The DCF provides the UP path information to the UE including the DCF IP address or FQDN via NAS message.

-	The DCF sends the UE data transfer configuration to the UE including the Event ID via UP to tigger the UE to perform the data collection with the RAN.

-	The UE sends the collected data to the DCF via UP.

-	The DCF verify and match the UE reported data based on the UE data collection information stored in the DCF and the data reported from UE.

-	The DCF exposes the collected data to the UE side model training server/UMTES.

* * * End of Change * * * *