3GPP TSG-WG SA2 Meeting #171	S2-2508280

Wuhan, China, Oct 13 - 17, 2025	(revison of S2-2506284)

Source:	OPPO

Title:	Proposal on Interim agreements for KI#4

Document for:	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes Interim agreements for KI#4 for the FS_Sensing_ARC TR 23.700-14.

# 1	Discussion

Solution#1, #3, #5, #18, #21, #22, #23, #24, #25, #26, #34 and #39 are to address KI#4 based on TR 23.700-14 V1.0.0.

Key functionalities are summarized as below:

- Coordination by a Sensing Function (SF)

All solutions involve a logical function (often called the Sensing Function, SF, or split into SCF and SPF) that orchestrates the data collection process. Its responsibilities include selecting sensing entities, configuring them, and managing the data flow

- Sensing result generation

A dedicated function (e.g. SF or SPF) is responsible for the final result generation. See Solution #1 / #3 / #5 / #23 / #25 / #26 / #34.

- Bi-directional Data Flow

Configuration Downlink: The Sensing Function sends the sensing service configuration and parameters to the Sensing Entity. See Solution #1 / #18 / #24 / #34 / #39.

Data Uplink: The Sensing Entity performs the measurement and sends the resulting sensing data and associated information back to a processing function.

- Support for Multi-Entity and Multi-Processing scenarios

The architecture is designed to handle data collection from multiple Sensing Entities (e.g., multiple gNBs) and to distribute sensing data to one or multiple processing functions (e.g. SePFs), enabling complex data fusion and processing chains. See Solution #23 / #24 / #26.

- Collection from the data collection entity

In addition to collect data from the Sensing Entity, sensing data and associated information can also be directly collected from the data collection entity to reuse the data collected for other purpose. See the updated Solution #18.

- Data Storage and Management

Some solutions introduce the concept of storing sensing results for future use, suggesting a need for a Sensing Data Management Function (SDMF) to avoid redundant data collection and processing. See Solution#26.

Observation 1: Convergence can be seen for the above-mentioned features.

Proposal 1: Document Item a) to f) in the Agreed Principles for KI#4.

Additionally, diverse views are identified on the following issues:

- Whether the sensing data is transferred to the data processing function over control plane (i.e. N2, via AMF) or over a direct data connection

- Option 1: Data delivery over N2 (i.e. via AMF)

Enhancements to N2 interfaces for sensing data transmission. See Solution#5.

- Option 2: Data delivery over direct data connection

A direct communication path between the Sensing Entity (e.g. a gNB) and the data processing function (often the Sensing Processing Function, SPF) is assumed to efficiently transfer sensing data. See Solution #1 / #18 / #21 / #22 / #25 / #26 / #34.

Observation 2-a: 2 Options can be considered on sensing data delivery to the data processing function.

- Option 1: over N2 (i.e. via AMF)

- Option 2: over direct data connection

For the determination of which option to be adopted for 5GA R20 sensing, the following factors will be considered:

- Latency: the data delivery path (gNB -> AMF -> SF VS gNB -> SF) may impact real-time or near-real-time sensing applications (e.g., drone tracking). For Option2, the procedure of direct data connection establishment may also impact latency.

- Throughput: N2 is designed to transfer control plane signaling. whether it is suitable for delivery sensing data is determined by the volume of sensing data, which is to be coordinated with RAN WGs.

- Scalability: For Option1, as the number of sensing services and entities grows, the N2/AMF path becomes a severe bottleneck. Scaling would require scaling the AMF itself, which is inefficient for a data-forwarding task. For Option 2, SF/SPF can be scaled independently based on sensing processing load

- AMF load: AMF load on data forwarding will be increased, which may impact the forwarding of control signaling

- Impact to gNB: To be evaluated by RAN WGs

- Stage 3 standardizations: How much stage 3 work is expected.

- Future proof to 6G: more scenarios than object detection and tracking will be considered in 6G, as well as UE based sensing

|  | Latency | Throughput | Scalability | AMF load | Impact to gNB | Stage 3 standardizations | Future proof to 6G |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Option 1 | Higher | Limited | Poor | High impact | TBD | Lower | Yes |
| Option 2 | Lower | High | High | Minimum impact | TBD | Higher | No |

NOTE 1: Items marked with TBD require further evaluations, especially the coordination with RAN WGs

Observation 2-b: Option 2 (direct data connection) has more advantages over Option 1 (N2/via AMF). Option 1 is only valid if sensing data volume is as low as it fits to N2 and the impact to AMF is affordable. Sensing data volume needs to be checked with RAN WGs.

Proposal 2: Option 2 is adopted for sensing data transferring to the data processing function. Further evaluation is needed on whether Option 1 is valid. Coordinate with RAN WGs (LS to RAN WGs) for the conclusion. If no response can be received from RAN WGs by the end of 2025, document the issue as “Topics for further consideration for KI#4”.

- how the direct data connection is established between Sensing Entity (e.g., a gNB) and the data processing function:

- Solution#18: The Sensing Function provides IP address to the Sensing Entity to establish the connection, accompanied by a binding procedure where the Sensing Entity confirms the connection, ensuring data is sent to the correct endpoint, implying a sensing UP layer over TLS, which is similar to LCS-UPP.

- Solution#22: The Sensing Function provides IP address and Tunnel Endpoint ID - TEID to the Sensing Entity to establish the connection. This is analogous to how a PDU session tunnel is set up between a gNB and UPF, explicitly establishing a "direct data tunnel" between the gNB and the Sensing Function. This strongly implies a GTP-U tunnel.

- Solution#25: Introduces the higher-level concept of a "sensing session." This session is an association that contains the data connection. The direct connection can be maintained and reused for multiple subsequent data transmissions.

- Solution #34: References the direct connection and states that if Control/Processing split is applied, the SCF requests the SPF to allocate the data connection address.

Observation 3: 2 Options can be considered on direct data connection establishment between Sensing Entity (e.g. a gNB) and the data processing function.

- Option 1: introducing sensing data connection layer over TLS (LCS-UPP similar)

- Option 2: reusing GTP-U

For the determination of which option to be adopted for 5GA R20 sensing, the following factors will be considered:

- Impact to SF deployment: SF may be split into SCF (centrally deployed without sensing layer) and SPF (distributed with sensing layer supported). In such case, an interface may be introduced between SCF and SPF for sensing service control and management. Sensing layer is introduced for the interaction between gNB and SF for sensing service operation (e.g. NRPPa alike protocol).

- Impact to gNB: To be evaluated by RAN WGs

- Impact to CT: How much stage 3 work is expected.

- Future proof to 6G: consistent solution for sensing data collection from gNB and UE is preferred

|  | Impact to SF deployment | Impact to gNB | Impact to CT | Future proof to 6G |
| --- | --- | --- | --- | --- |
| Option 1 | Easily support | TBD | High | Yes |
| Option 2 | Complicated for SCF-SPF interaction | TBD | Low | No |

NOTE: Items marked with TBD require further evaluations, especially the coordination with RAN WGs

Proposal 3: Further evaluate the issue on how to establish data connection between gNB and SF, and coordinate with RAN WGs (LS to RAN WGs) for the conclusion. If no response can be received from RAN WGs by the end of 2025, document the issue as “Topics for further consideration for KI#4”.

- Whether sensing result is generated by gNB

One of the objectives of KI#4 is to study “Which Sensing Function performs Sensing result generation?”

Based on TS 22.187, “3GPP sensing data” and “sensing result” are defined as below:

3GPP sensing data: data derived from 3GPP radio signals impacted (e.g., reflected, refracted, diffracted) by an object or environment of interest for sensing purposes, and optionally processed within the 5G system. Sensing result: processed 3GPP sensing data requested by a service consumer.

Observation 4: Based on TS 22.187 definition, as long as “data derived from 3GPP radio signals” is processed, it is sensing result. However, it is unclear whether and what level of sensing result can be generated by gNB, which is to be clarified by RAN WGs.

Proposal 4: Coordinate with RAN WGs (LS to RAN WGs) for the conclusion of sensing result generation. If no response can be received from RAN WGs by the end of 2025, document the issue as “Topics for further consideration for KI#4”.

# 2	Proposal

Proposal 1: Document Item a) to f) in the Agreed Principles for KI#4.

Proposal 2: Option 2 (over direct data connection) is adopted for sensing data transferring to the data processing function. Further evaluation is needed on whether Option 1 (over N2 via AMF) is valid. Coordinate with RAN WGs (LS to RAN WGs) for the conclusion. If no response can be received from RAN WGs by the end of 2025, document the issue as “Topics for further consideration for KI#4”.

Proposal 3: Further evaluate the issue on how to establish data connection between gNB and SF, and coordinate with RAN WGs (LS to RAN WGs) for the conclusion. If no response can be received from RAN WGs by the end of 2025, document the issue as “Topics for further consideration for KI#4”.

Proposal 4: Coordinate with RAN WGs (LS to RAN WGs) for the conclusion of sensing result generation. If no response can be received from RAN WGs by the end of 2025, document the issue as “Topics for further consideration for KI#4”.

It is proposed to include the following changes in TR 23.700-14 V1.0.0.

* * * * Start of Changes * * * *

### 7.1.Y	Agreed Principles for KI#4

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

Sensing data (and the associated information, if any) is transferred from gNB to the sensing data processing function in the core network for final result generation.

The sensing data processing function is responsible for generating sensing result based on the collected sensing data. One or multiple sensing data processing function may be selected for sensing result generation of the service request. When multiple sensing data processing functions are selected, they are chained to generate the final result, i.e. multiple steps may be needed for final result generation.

A logical function (i.e. Sensing Function) is responsible for data collection configuration, i.e.  configuring gNB and the sensing data processing function for data connection.

The logical function for data collection configuration and the logical function for data processing may be the same NF or different ones.

Editor’s note: Whether a standard interface will be defined between the logical function for data collection configuration and the logical function for data processing when they are separate NFs is FFS.

.

Based on the configuration, the Sensing Entity performs the measurement .

Editor’s note: What are the sensing data and the associated information is FFS, and will be coordinated with RAN WGs.

The sensing data and the associated information are transmitted over a direct data connection from gNB to the Sensing Function for result generation.

Editor’s note: How the direct data connection is established is FFS, and will be coordinated with RAN WGs.

Sensing data can be collected from one or multiple gNBs for the same service request.

* * * * Next Change * * * *

### 7.2.Z	Topics for further consideration for KI#4

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#Z. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.Z) and can either be then marked as not pursued or postponed to a future Release.

- Whether a standard interface will be defined between the logical function for data collection configuration and the logical function for data processing when they are separate NFs?

- What are the sensing data and the associated information?

Editor’s note: What are the sensing data and the associated information is FFS, and will be coordinated with RAN WGs.

- Whether transferring sensing data to the data processing function over N2 is acceptable?

Editor’s note: The volume & format of the sensing data and whether it is acceptable to transfer sensing data to the data processing function over N2 will be checked with RAN WGs.

- How the direct data connection between gNB and the data processing function is established?

Editor’s note: How the direct data connection between gNB and the data processing function is established will be coordinated with RAN WGs.

- Whether gNB can generate sensing result?

Editor’s note: Whether or what level of result generation can be performed by gNB will be checked with RAN WGs.

* * * * End of Changes * * * *