SA WG2 Meeting #171	S2-2508284

13 – 17 October 2025, Wuhan, China

Source:	LG Electronics

Title:	KI#3: Interim agreements

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This paper proposes interim agreements for KI#3.

Discussion

For KI#3 "Sensing Entity and Sensing Function Discovery and (Re-)Selection", 10 solutions have been proposed. An analysis and summary of these solutions are provided in the following table.

| Sol.# | Sensing Entity(ies) discovery/(re-)selection entity(ies) | Sensing Entity(ies) discovery/(re-)selection criteria | Sensing Function(s) discovery/(re-)selection entity(ies) | Sensing Function(s) discovery/(re-)selection criteria |
| --- | --- | --- | --- | --- |
| #1 | Sensing Function (SnF) | Based on sensing service request | Not mentioned | Not mentioned |
| #3 | Sensing Function (including Sensing Control Function (SCF), Sensing Processing Function (SPF)) | e.g., sensing service requirements, sensing capability | NEF | Configuration parameters from AF (No details) |
| #5 | AMF (optionally SF) | Sensing service parameters; sensing capabilities | NEF | Sensing service request parameters |
| #13 | Sensing Management Function (SEMF); (with assistance from AMF) | Sensing service request requirements (i.e., Target sensing area, sensing QoS, sensing service type); sensing capability (e.g., sensing mode, sensing area) | NEF (by querying to the NRF) | Sensing service request requirements (i.e., Target sensing area, sensing QoS, sensing service type) |
| #14 | Not mentioned | Not mentioned | Trusted AF: AF; Untrusted AF: NEF | Selection criteria (e.g., sensing service area of interest, sensing service time period of interest) |
| #15 | Sensing Function (SF) | SE capabilities (e.g., Sensing transmitter/ receiver, supported sensing area) | NEF | SF capabilities (supported area, service type). |
| #16 | Sensing Function (SF) | SE information (e.g., sensing entity ID, sensing capability information, served sensing area); requested KPIs; confidence level | Sensing Entity (SE) or AMF | No details (FFS) |
| #17 | Sensing Management Function (SeMF) | SE capabilities; SE location; requested service area (e.g., Cell IDs, TA) | NF service consumer (e.g. AF, NEF) | Target service area; QoS info.; latency; supported applications; SeMF load; event reporting |
| #35 | Sensing Service Function (SSF) | NG-RAN info. (sensing service capability, Tx/Rx, supported area) | Not mentioned | Not mentioned |
| #36 | Sensing Function (SF) | Event conditions (object approaching Non-Allowed Area, object out of coverage); sensing capability info (e.g., accuracy, resolution, latency, Tx/Rx support). | Not mentioned | Not mentioned |

Summary

- 9 solutions (Sol#1, Sol#3, Sol#5, Sol#13, Sol#15, Sol#16, Sol#17, Sol#35, Sol#36) agree on SE discovery and (re-)selection being performed by a sensing-related function (e.g., SF, SEMF, SSF). - 6 solutions (Sol#13, Sol#15, Sol#16, Sol#17, Sol#35, Sol#36) define explicit criteria for SE discovery and (re-)selection (e.g., sensing capabilities, target area, QoS, service type, KPIs). - In most solutions (Sol#3, Sol#5, Sol#13, Sol#14, Sol#15 Sol#17), NEF or AF handles SF discovery and selection, often with NRF support.

Proposal

It is proposed to agree the following changes into TR 23.700-14.

* * * * Start of 1st Change * * * *

!! All New Texts !!

7.1	Agreed Principles

7.1.Y	Agreed Principles for KI#3

The interim agreements on principles for KI#3 are as follows:

-	The NF service consumer (e.g., AF, NEF) performs discovery and (re-)selection of Sensing Function(s) by querying the NRF, taking into account defined selection criteria (e.g., target sensing area, sensing type).

-	The Sensing Function performs discovery and (re-)selection of Sensing Entity(ies) based on Sensing Entity capabilities, sensing service requirements, and/or other information (e.g., target sensing area, sensing service type).

7.2	Topics for further consideration

7.2.Y	Topics for further consideration for KI#3

Topics for further consideration for KI#3 are as below:

-	How to define selection criteria for Sensing Function(s).

-	How to define selection criteria for Sensing Entity(ies).

* * * * End of Changes * * * *