3GPP TSG-SA WG2#171	S2-2508292

Wuhan, China, 13 – 17 October, 2025	(revison of S2-250xxxx)

Source:	OPPO

Title:	Proposal on Interim agreements for KI#1

Document for:	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes Interim agreements for KI#1 for the FS_Sensing_ARC TR 23.700-14.

1	Discussion

Based on TR 23.700-14 V1.0.0, Key functionalities and architectural options are summarized below:

​Observation #1:​​ The scope of Rel-20 5G-Advanced sensing is limited to gNB-based sensing.

​This aligns with the Rel-20 5G-A RAN ISAC study scope as per the down-scoping agreement at SA#108 (SP-250833).

​Observation #2:​​ A new Core Network function (e.g. the Sensing Function (SF)), is introduced to support the sensing service. The network function provides the following key functionalities:

- NF profile registration to enable its discovery by a centralized gateway function or NEF.

- Receiving, authorizing and revocating sensing service requests from the sensing service consumer.

- Discovery, Selection and re-selection of the Sensing Entity.

- Provisioning of sensing configuration parameters to the selected Sensing Entity.

- Collecting sensing data from the Sensing Entity.

- Processing of sensing data and generation/exposure of the sensing result.

​Observation #3:​​ The Sensing Entity (SE) provides the following key functionalities:

- Performing Sensing Service on Radio interfaces and Collecting sensing data according to the sensing request received from the SF.

- Reporting the sensing data to the SF.

- Registering its Capabilities to the CN NF for SE discovery and selection.

​Observation #4:​​ A centralized network function to handle the AF request provides the following key functionalities:

- Handling of sensing service requests from the Application Function (AF).

- Discovery and selection of the SF.

- Authorization of the AF which sends sensing service request.

Observation #5: Direct data connection should be used for sensing data transport between the Sensing Entity and Sensing Function.

Observation #6:​​ Optionally the AMF can be involved for SE discovery and selection and control plane configuration.

​Proposal​:	Based on the observations above, it is proposed to capture Observations #1 through #6 as the interim agreements for KI#1.

2	Proposal

Proposal: Based on the observations above, it is proposed to capture Observations #1 through #6 as the interim agreements for KI#1.

It is proposed to include the following changes in TR 23.700-14 V1.0.0.

* * * * Start of Changes * * * *

7.1.Y	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

The following interim agreements on KI#1: System Architecture to Support Sensing, are made:

- The scope of Rel-20 5G-Advanced sensing is limited to gNB-based sensing.

- A new Core Network function, the Sensing Function (SF), is introduced to support the sensing service.

- The Sensing Function (SF) is responsible for NF profile registration, authorization of service requests, discovery/selection of the Sensing Entity, provisioning of configuration parameters, and processing of sensing data.

- The Sensing Entity (SE), which is a gNB, is responsible for performing sensing operations, collecting sensing data, and reporting the data to the SF.

- The Sensing Entity (SE) registers its capabilities with a core network function to enable discovery and selection.

- A centralized network function handles sensing service requests from the Application Function (AF), including AF authorization and SF discovery/selection.

- A direct data connection shall be used for transport of sensing data between the Sensing Entity and the Sensing Function.

- The Access and Mobility Management Function (AMF) may be optionally involved for assistance in SE discovery and selection and for control plane signaling.

* * * * End of Changes * * * *