SA WG2 Meeting #171	S2-2508299

Oct. 13 – 17, 2025, Wuhan, China

Source:	OPPO

Title:	[WT#1.1] Support for Control Signalling

Document for:	Agreement

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper proposes the update on the WT#1.1 description and new KI for the support for control signalling aspect in 6G system.

# 1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

  - Motivation

The volume of UEs is expected to be significantly boosted due to wide use of Robots and Automobiles in the 6G era, thus the NAS signaling may be overloaded in 6G system. If all NAS signaling piggybacked in Access and Mobility Management NAS messages goes over a centralized AMF in cloud, the AMF will become a bottleneck for NAS routing.

Furthermore, routing all NAS messages over a centralized AMF between the UE and a localized NF (e.g., I-SMF, local SMF, UE-PCF, LMF) will lead to a certain level of message routing delay.

For the NAS messages for core functionality (e.g., RM, MM, SM and UE Policy Management, etc), they can still be routed over Control Plane, but always routing the NAS messages over a centralized AMF in cloud doesn’t seem to be a scalable solution for 6GS, thus a de-centralized NAS routing mechanism is expected to be on board.

For the MNO services (e.g., LCS, AI and Sensing, etc.), the control signaling interaction between UE and the corresponding NFs is expected to be more frequent and have a bigger message size than the NAS signaling for core functionality (e.g., RM, MM, SM and UE Policy Management, etc.), thus it may cause more burden for Control Plane. In order to reduce the burden for NAS transfer over Control Plane, the signaling for MNO services can be routed over the User Plane instead of Control Plane.

  - Technical Impact

New functions may need to be introduced into 6GS to support an evolved control signalling transfer, which will require new interfaces and corresponding interaction among the UE and network functions.

**** First Change (all new text) ****

It’s proposed to update the Work Task#1.1 as following:

# Annex A.X. WT#1.1 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#1: Define the overall 6G architecture as collection of capabilities and high level functionalities considering the following sub work tasks and other work tasks to support 6G access network:

NOTE  1:	The duplication of functionality in RAN and CN will be avoided, while maintaining the existing RAN and CN functionality split.

1.1.	Study the support for control signalling for 6G, including at least the following:

- a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

- b.	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

- c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support  services.

**** Second Change (all new text) ****

# 5.X. Key Issue #X:  Support for Control Signalling

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

This key issue proposes to study how to support the control signalling interaction between UE and the according NFs includinginvestigating the following aspects:

- How to route the NAS signalling between UE and the according NFs with less NAS signalling routing delay, and the corresponding procedures (e.g., SM, MM, CM, RM, security, UE Policy provisioning, network function selection, etc.).

- How to route the control signalling for services (e.g., LCS, Sensing, AI, etc.) between UE and the according NFs.

- The temporary UE ID (e.g., 6G-GUTI) structure and allocation aspects.

- The UE context storage aspects.

- Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

**** End of Changes ****