SA WG2 Meeting #171	S2-2508349

Wuhan, China, October 13 - 17, 2025

Source:	vivo

Title:	[WT#1.2, Policy framework] UE input for AM policy

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper focus on promoting policy framework in 6G to design a policy framework that reflect to study the UE input for AM policy.

1. Justifications for UE input for AM policy

1.1 Architectural Motivation

In 5G policy framework, AF input for the AM policy (e.g., RFSP, service area restriction, UE-AMBR, Slice-MBR, slice replacement) influence is designed as described in clause 4.15.6.9.2 of the TS 23.502. The AF influence on Access and Mobility related policy control refers to the AF capability to request a service area coverage, the indication that high throughput is desired for a UE or an AF requested network slice replacement. According to the AF requests the PCF adjusts the AM policy (e.g., the restriction area, RFSP, Allowed NSSAI).

Observation 1：Existing 5G policy framework has designed mechanism/operations that allows AF to provide input to influence the AM policy.

It can be understood that these AM policies will be finally used on the UE which control the aspects of the UE access and mobility management. Compared with the input of AF, it is understandable that as the object (i.e., UE) controlled by the policy, UE, using the input of UE to influence the AM policy can make the AM policy more reasonable and accurate. This is because as an object (i.e., UE) controlled by the policy, UE has more specific and rich information that AF cannot obtain, and this information can more effectively reflect the true demands of UE. On the other hand, future 6G Terminals probably has AI capability and well perceivably understand the user needs, and then reflect these needs to the network for more precise AM policy.

Observation 2：Compared with the input from AF, the input of UE to influence the AM policy can make the AM policy more reasonable and accurate, and with the AI capability, the UE can well perceivably understand the user needs, and then reflect these needs to the network for more precise AM policy.

1.2 Gap Analysis

In 5G policy framework, AF input for the AM policy influence is designed as described in clause 4.15.6.9.2 of the TS 23.502. Now the input for AF only on the service area coverage, high throughput is desired for a UE or an AF requested network slice replacement, these inputs are too scarce to provide sufficient support for the accuracy of AM policy making from UE side, whether there are more input/information from UE needs to be further studied.

Another aspect, the procedure for AF input for the AM policy influence can not be used by UE input directly, thus it also needs to study the procedure of UE input for AM policy.

1.3 Technical Impact:

Whether and how to support the UE input for AM policy decisions at the Core Network for 6G, including.

-	What information from UE is useful to influence the AM policy

-	How to provide the UE input for the AM policy

-	How the CN makes the AM policy taking the UE input into account

**** First Change (Baseline WT from S2-2508123) ****

Annex A.X. WT#1.2 [Policy] Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

Take the WT proposal in S2-2508123 as the baseline, additionally add the following sub-WT: X. Whether and how to support the UE input for AM policy decisions at the Core Network for 6G.

**** Second Change (all new text) ****

5.X. Key Issue #X: Study 6G Policy framework

To support Policy Framework in 6GS, the Policy and charging control Framework defined in TS 23.503 for policies (i.e., SM Policy, UE Policy, AM Policy) is considered as starting point for discussion, following aspects are to be studied in this key issue:

X.	Whether and how to support the UE input for AM policy decisions at the Core Network for 6G, including:

-	What information from UE is useful to influence the AM policy

-	How to provide the UE input for the AM policy

-	How the CN makes the AM policy taking the UE input into account

**** End of Changes ****