SA WG2 Meeting #171	S2-2508355

Wuhan, China, 13 – 17 October 2025

Source:	vivo

Title:	[WT#8, 6G IoT] Unified design for 6G IoT and eMBB

Document for:	Agreement

Agenda Item:	20.6.8

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper proposals a unified design for 6G IOT and eMBB in 6G.

1. Justifications

  - Architectural Motivation & Gap Analysis:

In the era of 6G, the mobile system will provide its services to a myriad of devices, from low-end IoT devices to high-end advanced eMBB devices.

Table-1 Three device types with mandatory baseband and RF capability

|  | Device type A (Smart phone, immersive eMBB device, CPE) | Device type B (Reduced capability eMBB device, wearable/XR) | Device type C (6G IoT device) |
| --- | --- | --- | --- |
| Typical Device/service | Smartphones, immersive eMBB, CPE, | Reduced capability eMBB, e.g. wearable/XR, etc | IoT |
| Downlink peak data rate | ~10Gbps | 200Mbps ~ 1Gbps | ~10Mbps |
| Uplink peak data rate | ~2Gbps | 50Mbps ~ 200Mbps | ~5Mbps |
| Supported maximum downlink channel bandwidth | At least 200 MHz | 100 MHz | ~5MHz for FDD ~20MHz for TDD |
| Supported maximum uplink channel bandwidth | [100/200] MHz | 100 MHz | ~5MHz for FDD ~20MHz for TDD |
| Supported maximum Downlink MIMO layer | At least 4 layers | 1~2 layers | 1~2 layers |
| Supported maximum Uplink MIMO layer | At least 2 layers | 1 layer | 1 layer |

Three device types shown in Table 1, from high-tier device type A including Smartphones, immersive eMBB and CPE, to low-tier device type C for IoT, and device type B in-between for reduced capability eMBB, can be considered. It is noted that, ambient IoT is a different type of IoT evolved in a separate track, thus is not considered here.

For each device type, a minimum set of mandatory features for baseband and RF can be defined, e.g., channel bandwidth, number of MIMO layer as well as peak data rate as shown in Table 1.

Considering lower cost and complexity, and also extreme coverage, type C device (6G IoT device) can be categorized as the following aspects:

- bandwidth around 5MHz: According to cost reduction analysis in LTE and NR, bandwidth around 5MHz for low band in FR1 (i.e. FDD bands) achieves sufficiently low cost and complexity, and further reduction of bandwidth would only bring marginal benefit. Meanwhile, with 5MHz bandwidth for device type C, common design for common channels and signals compatible with all three UE types can be easily supported. For higher band in FR1 (i.e. TDD bands), if supported for IoT, minimum bandwidth around 20MHz would be reasonable choice, scaling with the subcarrier spacing, to facilitate unified design for different bands in FR1.

- single Tx & Rx: Similar as in LTE and NR IoT device as well as Redcap/eRedcap, type C device will typically support single Tx & Rx antenna (and optionally two Rx antennas), and may have smaller antenna gain due to complexity and smaller device form factors restriction. Thus, downlink MIMO layer no larger than 2 is supported by type C device.

- up to 10 Mbps: To meet various service requirements/scenarios, e.g., from smart cities, smart metering to immersive multimedia, different device types supporting different data rates shall be considered in the first release of 6GR. With limited bandwidth and limited number of Rx/Tx antennas, type C device can support up to 10 Mbps.

Proposal 1: 6GS shall consider the 6G IoT device as above.

<Lessons on different device types definition in 5GS>

In 5GS, the new device types (e.g., (e)Redcap, (e)MTC, NB-IoT) are not supported in 1st release(release 15) of 5GS and has been introduced in later releases, causing too many different sub-categories serving similar service, resulting in serious dilemma in deployment e.g.:

- NB-IoT with category NB1 & NB2, and MTC with category M1 & M2.

- DRX and eDRX.

- CIoT service request and common Service Request, CM idle and CM suspend etc.

- CIoT UP and common UP.

- CP CIoT PDU Session and common PDU Session.

Prpposal2: 6G IoT device should be introduced together with 6G eMBB device in the first release to avoid compatibility issues in later releases.

In 5GS, the definition of IoT and eMBB stem from different technical origins, follow independent standardization paths, and there is no in-depth collaboration between the core network and access network. e.g.:

- Separate definition will cause fragmented standardization and restricted technical evolution.

- Separated definition will cause fragmented RATs and Core network, and will cause high network deployment/operation costs and low resource utilization.

- Separate definition will cause fragmented terminal chipset, and will causes high terminal deployment cost.

Observation: This "non-unified" design has exposed a series of drawbacks in the implementation process of 5G.

Prpposal3: Single 6GC compatible to both 6G IoT device and eMBB devices and the unified design is necessary for both devices in first releases.

<benefit of unified design>

- The technologies of 6G IOT (such as PSM, eDRX, small data transmission, bigger coverage, and narrowband transmission) can feed back into the eMBB device in perspective scenarios e.g., the eMBB device is in low-power mode. When the eMBB utilize the 6G IoT features, the service and IP continuity should be guaranteed to minimize the impact to the ongoing services.

- Single RAT and Single Core to save the network deployment/operation costs and maximum resource utilization, in case the operator is willing to serve both 6G IoT device and eMBB devices.

- Single terminal chipset to save terminal deployment costs.

- One standardization to easily for maintenance.

- Better coexistence and forward compatibility to migrate to future systems.

1.2. Technical Impact

To achieve a unified design adaptive to both 6G IOT and eMBB devices, the following aspects needs to be considered:

- Maximum common design, e.g., the following:

- Support Common NAS protocol

- Unified DRX

- Unified Service Request

- Unified UP& PDU Session

- Small data transmission.

- Allow 6G IOT specific enhancement:

- Power saving mechanism.

- Coverage enhancement.

- The 5GS CIoT feature can be treated start point for the study, but it is not CIoT features anymore, which can be used by any types of the UEs. (e.g. allow eNBB UE to utilize the technologies of 6G IOTs in specific scenarios:

- The traditional IoT features, such as PSM, eDRX, small data transmission, bigger coverage, and narrowband transmission can feed back into the eMBB devices in perspective scenarios, e.g., the eMBB device is in low-power mode; or the eMBB device is in idle but small packets (e.g. heart beat packets) transmission still happens.

- Keep the service/IP continuity when eNBB devices to utilize the technologies of 6G IOT feature in specific scenarios

- The 5GS CIoT feature , but it is not CIoT features anymore, which can be used by any types of the UEs.  5GSs marked as “YES”

|  | Feature name for 5GS | Reference to existing Feature description | Impacts | Day 1 |
| --- | --- | --- | --- | --- |

| 17 | Support for Cellular IoT |  | UE AS | UE NAS | RAN | CN |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 17.1 | Preferred and Supported Network Behaviour | TS 23.501 clause 5.31.2 |  | X |  | X | Yes (Small data transmission related basic features) |
| 17.2 | Selection, steering and redirection between EPS and 5GS | TS 23.501 clause 5.31.3 |  | X |  | X | No necessary any more because of unified design |
| 17.3 | Control Plane CIoT 5GS Optimisation | TS 23.501 clause 5.31.4 | X | X | X | X | Yes (Small data transmission related basic features) |
| 17.4 | Non-IP Data Delivery (NIDD) | TS 23.501 clause 5.31.5 |  | X |  | X | Yes |
| 17.5 | Reliable Data Service | TS 23.501 clause 5.31.5 |  | X |  | X | No |
| 17.6 | Extended Discontinuous Reception (DRX) for CM-IDLE and CM-CONNECTED with RRC-INACTIVE | TS 23.501 clause 5.31.7.2 | X | X | X | X | Yes (Power Saving related basic features) |
| 17.7 | Paging for extended DRX for RRC_INACTIVE in NR connected to 5GC | TS 23.501 clause 5.31.7.2.4 |  |  | X | X | Yes (Power Saving related basic features) |
| 17.8 | MICO mode | TS 23.501 clause 5.31.7.3-5 |  | X |  | X | Yes (Power Saving related basic features) |
| 17.9 | High latency communication | TS 23.501 clause 5.31.8 |  | X |  | X | No |
| 17.10 | Support for Monitoring Events | TS 23.501 clause 5.31.9 |  |  |  | X | No |
| 17.11 | NB-IoT UE Radio Capability Handling | TS 23.501 clause 5.31.10 |  |  |  | X | No necessary anymore because of unified design |
| 17.12 | Inter-RAT idle mode mobility to and from NB-IoT | TS 23.501 clause 5.31.11 |  |  |  | X | No |
| 17.13 | Restriction of use of Enhanced Coverage | TS 23.501 clause 5.31.12 |  | X |  | X | Yes, but depends on RAN (Coverage Enhancement related basic features) |
| 17.14 | Paging for Enhanced Coverage | TS 23.501 clause 5.31.13 |  |  | X | X | Yes, but depends on RAN (Coverage Enhancement related basic features) |
| 17.15 | Support of rate control of user data | TS 23.501 clause 5.31.14 |  | X |  | X | No |
| 17.16 | Control Plane Data Transfer Congestion Control | TS 23.501 clause 5.31.15 |  | X |  | X | No |
| 17.17 | Service Gap Control | TS5 23.501 clause 5.31.16 |  | X |  | X | No |
| 17.18 | Inter-UE QoS for NB-IoT | TS 23.501 clause 5.31.17 |  |  | X | X | No |
| 17.19 | User Plane CIoT 5GS Optimisation | TS 23.501 clause 5.31.18 | X | X | X | X | Yes (Small data transmission related basic features) |
| 17.20 | QoS model for NB-IoT | TS 23.501 clause 5.31.19 |  | X |  | X | No |
| 17.21 | Category M UEs differentiation | TS 23.501 clause 5.31.20 | X |  | X | X | No |

Observation1: small data transmission (e.g. CP CIoT, UP CIoT), power saving related (€DRX, paging, MICO), NIDD, coverage enhancement can be considered as start point for unified design.

Observation2: small data transmission, (e)DRX, paging, coverage enhancement has RAN dependency and will be coordinated with RAN WGs.

**** First Change ****

A.8	WT#8: Study whether and how to support cellular IoT enablers in 6G

Study whether and how to support cellular IoT enablers in 6G, including:

- Analyze which 5GS IoT features and solution can be treated as the start point for the study discussion.

- Potential enhancement in 6G to the selected 5GS IoT features and solutions are not precluded.

Features driven by this WT are expected to be generic and can apply to any UE in 6G.

NOTE y:	The detailed scope for WT#8 will be coordinated and aligned with RAN. Ambient IoT and non-3GPP access are not in the scope of the study.

**** Second Change (All new text)****

5.X. Key Issue #X: 6G IoT

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

The study for 6G IoT includes the following aspects:

- #1: Study how to support a unified features design adaptive to any types of UEs, including the following aspects:

-   How to maximum common design for any types of UEs (e.g., common NAS, etc.)

-   How to support Non-IP Data Delivery (NIDD), including new study and analysis on whether 5GS NIDD can be treated as start point.

-   How to support small data transmission, including new study and analysis on whether 5GS CP/UP CIoT can be treated as start point.

-   How to support UE power saving mechanism, including unified DRX, MICO, Paging, etc.

-   How to enable coverage enhancement.

NOTE1:	Unified design doesn’t exclude different level value for different UE status (e.g. for DRX).

NOTE2:	Use 5GS IoT features doesn’t exclude enhancement and new solutions.

NOTE3:	It needs the coordination with RAN WGs.

**** End of Changes ****