3GPP TSG-SA WG2 Meeting #171	S2-2508360

Wuhan, China, October 13 – 17, 2025

Source:	NTT DOCOMO

Title:	[WT#5] Data framework for 6G network

Document for:	Approval

Agenda Item:	20.6.5

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper proposes scope and key issues of the “WT#5: Study data framework for all aspects related to efficient and scalable data handling including” of the 6G SID.

# 1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT and KI.

Data type studied in SA2 data collection framework
TSG SA provided guidance on the division of work between SA2 and SA5 in SP-251261. According to this guidance, SA2 and SA5 are requested to consider the 5G work scope as the starting study scope in each WG. In our understanding, SA2 is responsible for data processed/used in the CN (e.g., sensing, positioning, data for AI in CN), while SA5 is responsible for management data. For the 6G study, the same task split should apply. However, it remains unclear whether data collection for UE model training should fall under SA2 or SA5. In Rel-20 5G-Advanced, both SA2 and SA5 are currently studying the feasibility of UE data collection, with a conclusion expected in Dec. 2025. Under this circumstance, it is preferable to wait for the feasibility study outcome in Rel-20 SA2/SA5 and then decide which WGs should handle such data in their study of data collection framework.

Requirement of data collection framework
In 6G, many use cases require data collection in the CN (e.g., sensing, positioning, AI in CN). Depending on the use case, the architectural requirements for data collection vary. To design the 6G data framework, SA2 should identify the requirements of data collection per use case and assess whether existing mechanisms can meet them. Based on this analysis, necessary enhancements to the 6G data collection framework can be derived.
Different WTs (e.g., WT#3, WT#4) discuss use cases that may require data collection. Hence, it is reasonable for each WT to discuss the data collection requirements that correspond to their considered use cases. On the other hand, WT#5 focuses on the data collection framework. Therefore, WT#5 can identify which KPIs (e.g., data collection latency, data volume, data collection frequency) should be considered so that WT#5 can design the 6G data framework appropriately.

Common data collection mechanism
In TR 22.870, SA1 has stated that the 6G system shall support a common data framework that can address the needs of multiple use cases. In this regard, SA2 should study whether, and how, a common data collection mechanism across multiple use cases can be introduced in 6G. However, data collection mechanisms need to differ depending on the data source domain (UE, RAN, CN). Therefore, the possibility of a common data collection mechanism across multiple use cases should be studied separately for each domain.

MNO visibility
MNO visibility is an important aspect of data collection. When data is collected for use/processing in the CN, the CN needs to be aware of what data is being collected. Furthermore, when data is exposed to third parties from the network, the MNO must know what data is being exposed, from security, user privacy, and proprietary perspectives. Such MNO visibility is required regardless of whether the data is standardized or not.

Data governance, privacy protection
There is an inherent trade-off between maximizing data utilization and ensuring robust data governance and privacy protection. Overly strict governance may hinder the efficient use of collected data for service innovation and network optimization, whereas insufficient governance may lead to serious risks, including privacy breaches, security vulnerabilities, and loss of user trust. Hence, an appropriate balance must be achieved.

Fundamentally, data governance and privacy protection are subject to regulatory requirements. The 6G system shall, at a minimum, comply with such requirements while also enabling convenient and effective data usage from a service perspective. As these requirements differ across regions, the 6G data framework should support governance and privacy protection mechanisms that are sufficiently flexible to accommodate national and regional regulations, while still facilitating efficient data utilization. This principle is aligned with TR 22.870, which specifies: “Subject to national or regional regulatory requirement, the 6G system shall provide user privacy protection, location privacy, identity protection for UEs accessing 6G network for services (e.g. communication, sensing, AI inferencing), and for the corresponding information exposure to an authorized 3rd party”.

KI split
The current KI baseline description results in a significant workload imbalance, as all functionalities of the data framework are concentrated into a single KI.
To ensure more efficient management and clearer scope allocation, it is suggested to consider splitting the first KI into multiple KIs in future discussions.

# 2. Proposal

It is proposed to approve the following Key issues in TR 23.801.

**** First Change ****

# A.X 	Work Task X: 6G unified data framework

The following is WT#5 in the SA2 6G SID:

WT#5: Study data framework for all aspects related to efficient and scalable data handling including, for example, data collection, distribution, processing, storage, data access and data exposure, with consideration of access control/user consent and privacy where relevant. The example of data may include data for AI and Sensing. This WT can also study any potential enhancements on system and procedure needed for user consent framework.

NOTE 6:	The work split with SA3, SA5 and RAN WGs will require TSG coordination

Data categories can be handled/supported by 6G data framework studied in SA2

  - Data from Core network and 3rd AF

  - Data from UE (e.g. for Sensing, Positioning, AI in CN, etc.)

  - Data from RAN (e.g. for Sensing, Positioning, AI in CN, etc.)

- Editor’s Note: SA2 will revisit whether data collection for UE-sided model training is included in SA2 study scope or not after endorsing WID of FS_AIML_CN_Ph2.

The scope of the WT includes the following aspects:

- Identify what KPI (e.g., data collection latency, data volume, data collection frequency) needs to be considered for the architectural requirements of 6G data framework.

NOTE 1: Other WT (e.g., WT#3, WT#4) will study the requirements of data collection for each use case based on the KPI identified in WT#5.

- Design end to end 6G system architecture for data framework, e.g. defining new NF(s), new data plane/enhancing CP/UP and interfaces, to support efficient and scalable data handling functionalities, e.g.:

- data collection

- data transfer and distribution

- data processing e.g. labelling data for origin, purpose, context and quality/validity as well as handling metadata such as type and structure of data parameters produced by NFs

- data storage within 6G system

- data access

- data exposure within 6G system and to outside of 6G system

NOTE 2:	Coordination with SA5 may be required to determine whether and how to align with the data handling mechanisms supported in the management domain (i.e., OAM) and charging aspects of data exposure.

NOTE 3:	Coordination with RAN WG is required for data collection from or via RAN.

NOTE 4: Consideration on MNO’s requirements of visibility and controllability for data during data handling processes is required. And data handling processes can be service-differentiated.

NOTE 4: Consideration on interoperability in each data handling functionality.

- Study whether and how to support common data framework over use cases (e.g., sensing, positioning, AI in CN) within the data collection from the same domain (e.g., UE, RAN, CN).

- Study from SA2 architectural and system level on user consent, privacy, security and data governance to support the 6G data framework satisfying national or regional regulatory.

NOTE 5:	Coordination with SA3 is required for user consent, privacy, security and data governance related topics.

- Study how to support quality of data which is processed and provided based on the requirements from data requester.

**** (Next Change) ****

# 5.X. Key Issue #5.1: Data framework for handling of data generated in the 6G system

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

The key issue investigates how to design end to end 6G system architecture for data framework that can be commonly used for handling of data generated in the 6G system to support services deployed over the 6G network such as AI and sensing.

-	Target data:

  - Identify what data domain(s) and categories can be handled, e.g., sensing data, AI data, UE context data, via the data framework

NOTE 1:	The identification of data domain and categories needs coordination with RAN WGs and SA5.

  - Minimum Standardized Data Set per NF: Identify and whether and how to define the minimum standardized sets of operational and analytical data that each network function must generate and expose.

-	Requirement of data framework

  - Identify what KPI (e.g., data collection latency, data volume, data collection frequency) needs to be considered for the architectural requirements of 6G data framework.

NOTE 2: Other WT (e.g., WT#3, WT#4) will study the requirements of data collection for each use case based on the KPI identified in WT#5.

-	Common functionalities for data framework

  - Identify data handling functionalities, which can be commonly utilized for data-driven services such as sensing and AIML data collection. The following functionalities (not exhaustive list) will be investigated in terms of whether a common framework can be used across the services: data source discovery, data source configuration for data collection, transfer of the collected data, specific data distribution handling method such as data publishment, processing/reformatting of data, data labelling, data storage, feature engineering, data access control, data collection coordination (avoiding duplication) and data exposure (to outside of 6G system)

-	Data collection: How to support scalable and efficient data collection (i.e., data source discovery/selection, data source configuration for collection, transfer of the collection data, data collection coordination), including whether and how to generate and assign labels for data or metadata such as type and structure of data parameters, for various use cases (e.g., AIML data/model transfer and Sensing) with the consideration of the followings.

  - Whether and how a new data/service plane is needed to handle the anticipated end-to-end data collection, distribution and processing, or whether and how to enhance the existing user plane architecture

    - Whether and how to differentiate the traffic for data framework and the UE regular traffic.

  - Whether and how to support a common data collection request mechanism to avoid potential duplication of data source configuration and data transfer

  - Whether and how to support efficient data discovery or data source discovery considering that various type of data and data source exist in different domain (e.g., UE, RAN, Core Network)

  - Whether and how to support efficient and flexible data transfer involving UE, RAN, CN NFs

  - Whether and how to support enable data controllability and visibility (e.g., MNO controllability, and MNO visibility) on the data collection process.

  - Whether and how the existing data collection mechanisms in 5G can be reused or enhanced., including the following aspects:

    - whether and how to generalize the Data Collection Coordination Function (DCCF) and exposure functionality already defined in TS 23.288 to enable all 6G CN NFs to request data from a data source and act as data consumer for data exposed by CN NFs.

-	6G system data distribution within 6G system:

  - Whether and how to support distribution (among UE, RAN, CN) to one or multiple data consumers within 6G system

  - Whether and how to enable service-differentiated data transfer, processing and storage mechanisms beyond the MNO domain, e.g., also serving request for data transfer from third party AFs.

-	6G system data processing: Identify what kind of processing/formatting of the collected data needs to be supported, and whether and how to enable them

-	Data Storage within 6G system: How to support data storage that can be commonly used across the data-driven service in the 6G system for efficient data reuse, including the following aspects:

  - Whether and how to define a new NF or enhance existing ADRF/UDR/UDSF to perform data storage (e.g. unstructured data storage for RAG, stored dataset update, etc.).

-	Data Exposure (including data transfer across different domain): How to support data exposure with the secured data access control (e.g., authorization of data collection request, data service access authorization mechanism including the authorization to the service initiator, consumer, and the target object/environment) with the consideration of privacy protection. Data exposure to 3rd party AF/AS will be considered. The followings can be further considered:

  - Identify data type for exposure

  - How to support the data exposure (e.g., to multiple data consumers)

- Whether there is a need for additional data exposure for data collection from new and modified CN NFs based on the same principles for data collection as defined in TS 23.288.

NOTE 3:	In this key issue, the data generated in the 6G system will be the target of the data framework. UE, RAN, and NF (potentially also AF) will be considered as data sources for data-driven services.

NOTE 4:  Coordination/dependency with SA5 is needed for the following aspects: charging aspect, data lifecycle, audit management, whether and how to align with the data handling mechanisms supported in the management domain, whether and how to utilize OAM-generated data.

NOTE 5:	Coordination with WT#1.1 (UE-CN interaction), WT#1 (RAN-CN interaction, Management of context data stored in NF), WT#3 (AI-related aspects), WT#4 (Sensing data), and WT#6 (Computing service related data) is expected.

NOTE 6:	The data framework studied in this key issue Data framework for handling of data generated in the 6G system will comply with data governance and privacy protection (including user consent enforcement), which are studied under Key Issue #5.2.

NOTE 7:	Coordination with RAN WGs is needed for the identified issues during the study phase when the data source is RAN or UE for the collected data defined by RAN WGs (e.g., measurement data by UE or RAN).

NOTE 8: Consideration on the interoperability in each operation of data framework.

# 5.X Key Issue #5.2: Data governance, privacy protection, and user framework

-	How to enhance privacy protection for the data handling in the 6G system subject to national or regional regulatory requirements, including enhancements on system and procedure for the data governance, user consent and/or privacy framework. The followings can be considered:

  - Identify mechanisms to enable dynamic, granular and revocable user consent for data processing to satisfy each national/regional regulatory requirement.

  - Whether and how to design User-Centric Privacy Control to enable dynamic Privacy Protection with minimal privacy exposure based on user’s control data privacy and usage for sensitive data at granular levels in real-time for different 6G services based on user preference for data privacy protection. For example, Anonymization and Pseudonymization techniques, Enhanced encryption framework for supporting encryption for diverse 6G data types and adapt encryption levels based on data sensitivity and service requirements.

  - Whether and how to enable network provisioning of dynamic, on-demand data privacy protection for users before sharing data to third parties or using in network-related operations, supporting automated privacy enforcement based on user preferences.

  - Study all data security related aspects that covers data ownership, data privacy, data access control, data governance including governance of data exposure to 3rd party and cross-domain governance.

NOTE 1:	The aspects on data security, privacy protection and user consent need coordination and work split with SA3.

**** End of Changes ****