3GPP TSG-SA2 Meeting #171	S2-2508368

Wuhan, CN, Oct 13 – 17, 2025                                   (was S2-2506172)

Source:	Nokia

Title:	KI#2: proposed agreement of principles

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract: This document proposes to agree principles for KI#2

# 1.	Introduction

In the scope of KI#2 there is the notion that the network may at any point in time modify the data transmission service offered to a UE to achieve some energy goals.

Various options are possible:

- Use estimates of energy consumption at various granularity of the UP-energy consumption from EIF. These estimates may be at UE level granularity or across all UEs of a slice. Action is taken at the granularity of the estimates that trigger the action. The fairness of this type of actuation and its effectiveness may be linked to the accuracy and confidence level on the measurement. From that perspective it could be useful to have information samples for the UE repeated over a number of periods T before the action is taken and also it is beneficial to have some estimates of the relative energy consumption at the desired granularity over the population of the ongoing measurements as proposed in solution 6.2. So, in general the robustness of the input may be improved by the number of measurements and some analysis of the relative consumption as in sol 6.1.

- The SM PCF may configure the UE with Alternative QoS Policies (AQP) for Energy (Energy Saving QoS profiles as AQP for GBR and corresponding parameters for non-GBR) that can be used upon certain configurable energy criteria being detected at the gNB. The advantage of this approach is that there is no need of a centralized decision and each gNB can trigger service adjustment based on operator defined policy for energy that apply to the UE based on gNB logic. Charging subsystem should receive indication of change of QoS when it happens. Additionally, AQPs sent with different values considering energy aspects, would not really indicate to the gNB that these AQPs are defined due to energy aspects. So, it is proposed to add additional indication that specific AQPs are generated based on energy inputs.

- Policy Data managed by the Operator OAM is used by the PCF for policy decisions. Such a policy data can be additionally provisioned to have energy saving policy for a UE, which can be activated by the AF.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-67.

# * * * * First change (all new text) * * * *

### 7.1.2	Agreed Principles for KI#2

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

The following principles are agreed for KI#2:

-	Subscribe to the additional information for events (Energy consumption percentile/ Energy consumption per bit) as per solution 6.1 to allow PCF to receive additional categorization information and implement policies by considering multiple samples of the information from the same UE.

-	Re-use Alternative QoS profiles (AQP) framework with additional Energy Saving QoS profile (GBR) / parameters (non-GBR) for energy saving operation handling that can be provided to the NG-RAN by the SMF to cause service adjustment based on information locally available at the NG-RAN. These are generated by the PCF and provided to the SMF. The PCF can take AF input for the alternative QoS parameters (non-GBR) and profiles (AQP for GBR) with energy saving service requirements. NG-RAN should provide indication of the applied QoS to the SMF for SMF to include the corresponding information to the CHF for charging purposes.

-	PCF considers energy saving policy provisioned and available in the UDR activated by an AF for SM policy decisions.

-	PCF informs SMF and AMF with the information (for e.g., reason for service adjustment, information received from EIF such as categorization information) that caused service adjustments, and accordingly SMF and AMF includes this information when communicating with the CHF for CHF to implement rating and reporting as required.

-	Without impact to the TS, the PCF with the current specifications, could consider the location (gNB change) (with PRA) and/or time as the criteria and could implement policies that are configured on PCF with the awareness that specific gNB at certain time periods use certain type of energy (e.g., renewable).

# * * * * Next change (all new text) * * * *

### 7.2.2	Topics for further consideration for KI#2

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#Z. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.Z) and can either be then marked as not pursued or postponed to a future Release.

-	Whether, how and which renewable aspects (renewable energy or the ratio of renewable energy) needs to be considered based on the SA5 conclusions is to be established.

# * * * * End of change * * * *