SA WG2 Meeting #171	S2-2508371

13 - 17 Oct, 2025, Wuhan, China

Source:	China Mobile, ZTE, SK Telecom

Title:	Update for KI#2 Interim Agreements and Conclusions

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 /Rel-20

Abstract of the contribution: This contribution proposes to update the interim agreements and conclusions for AIML_CN KI#2.

- Discussion

In TR 23.700-04 V0.3.1 (2025-09), some principles are agreed for KI#2. However, there are still several topics for further consideration depicted as 7.2.2 in TR 23.700-04.

The following topics are for further consideration for KI#2:

- Whether the UPF can be the consumer of the user plane traffic pattern and behaviour analysis or not.

- Whether new UPF or SMF events are needed or not for NWDAF data collection.

- How to reduce the reporting load of input data sources and control plane signalling overhead? For example by configuring appropriate Analytics Filter Information, combining notification to NWDAF at UPF for multiple events, instructions from NWDAF to UPF for pre-processing in the UPF, etc.

- Consumer actions are FFS.

- This proposal discusses these topics and concludes the aspects for KI #2.

- Proposal

It is proposed to adopt the following changes into TR 23.700-04.

*** First change ***

7	Interim agreements

7.1	Agreed Principles

7.1.2	Agreed Principles for KI#2

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.2 gets resolved and a principle is agreed.

7.1.2.1	General

No impact on RAN and UE operation.

To reduce the reporting load of input data sources (e.g. UPF), the analytics consumer may include the Analytics Filter Information for input data filtering to UPF.

7.1.2.2	Agreed Principles for Use Case #1

- The service consumer of the analytics service may be SMF, PCF, OAM, UPF and AF. The consumer may take the output Analytics into account.

- To provide analytics to the consumers, the NWDAF may provide the output analytics.

- To derive the output analytics, the following input data may be collected from the UPF to support the NWDAF-based analytics:

-	Information to identify the abnormal traffic

-	Information of traffic characteristics including the UL/DL/round trip of Data Burst Size, traffic patterns, traffic volume or the number of data packets, traffic rate (e.g. packet rate, bit flow rate, etc.), delay, loss rate, throughput, packet size distribution, jitter, (average) packet interval, etc.

- The SMF may take the following actions upon the detection of the abnormal traffic:

-	UPF reselection to distribute the load across UPF instances upon detecting abnormal traffic patterns (e.g. RTT fluctuations, abnormal PPS).

-	Notifying UPF to execute traffic gating or shaping, enforcing bandwidth parameters (e.g., rate limiting to 5Mbps) or adjusting QoS parameters.

- The UPF may take the following actions:

-	Controlling the number of DLDR (Downlink Data Report) messages sent to SMF by e.g., adjusting message frequency.

-	Downlink traffic suppression (e.g., selective packet dropping) for AS-initiated traffic bursts.

-	Apply operator-configured policies (e.g. drop, rate-limit, deny), adjust packet processing resources, mirror suspicious/attack/fraud/phishing packets for security audits, and/or regard as error/partial failure to trigger UPF-initiated PFCP Session Release per clause 5.18.2 of TS 29.244.

- The PCF may take the following actions:

-	Policies provision to SMF, e.g. executing traffic gating or shaping, enforcing bandwidth parameters (e.g. rate limiting to 5Mbps) or adjusting QoS parameters.

7.1.2.3	Agreed Principles for Use Case #2

- The service consumer of the analytics service is PCF.

- To provide analytics to the consumers, the NWDAF may provide the output analytics on traffic pattern information.

- To derive the output analytics, the following input data may be collected from NFs to support the NWDAF-based analytics:

-	Information identifying the associated application/service data flow of the collected QoS monitoring information.

- The consumer actions may take the output Analytics into account, e.g., generate or update PCC rules.

*** Second change ***

8	Conclusions

The following principles are concluded for KI #2.

- NWDAF collects the following data from the UPF to train ML models and perform the inference for the user plane analytics:

-	Information to identify the abnormal traffic.

-	Information of traffic characteristics including the UL/DL/round trip of Data Burst Size, traffic patterns, traffic volume or the number of data packets, traffic rate (e.g. packet rate, bit flow rate, etc.), delay, loss rate, throughput, packet size distribution, jitter, (average) packet interval, etc.

- The service consumer of the analytics service may be SMF, PCF, OAM, UPF and AF, which may subscribe/request the analytics from NWDAF and take actions by taking the output analytics from NWDAF into consideration.

- The SMF may take the following actions upon the detection of the abnormal traffic:

-	UPF reselection to distribute the load across UPF instances upon detecting abnormal traffic patterns (e.g. RTT fluctuations, abnormal PPS).

-	Notifying UPF to execute traffic gating or shaping, enforcing bandwidth parameters (e.g., rate limiting to 5Mbps) or adjusting QoS parameters.

- The UPF may take the following actions:

-	Controlling the number of DLDR (Downlink Data Report) messages sent to SMF by e.g., adjusting message frequency.

-	Downlink traffic suppression (e.g., selective packet dropping) for AS-initiated traffic bursts.

-	Apply operator-configured policies (e.g. drop, rate-limit, deny), adjust packet processing resources, mirror suspicious/attack/fraud/phishing packets for security audits, and/or regard as error/partial failure to trigger UPF-initiated PFCP Session Release per clause 5.18.2 of TS 29.244.

- The PCF may take the following actions:

-	Policies provision to SMF, e.g. executing traffic gating or shaping, enforcing bandwidth parameters (e.g. rate limiting to 5Mbps) or adjusting QoS parameters.

*** End of the changes ***