SA WG2 Meeting #171	S2-2508377

# Wuhan, China, October 13 - 17, 2025	(revision of S2-2507044)

Source: 	vivo

Title: 	Interim principles for KI3

Document for: 	Approval

Agenda Item: 	20.4.1

Work Item / Release:	FS_EnergySys_Ph2/ Rel-20

Abstract of the contribution: This contribution tries to provide the interim principles for KI3.

- Discussion

This paper is trying to evaluate the solutions captured for KI#3(5GS enhancements for network energy saving and efficiency), provide the conclusions for the KI#3 based on the captured solutions.

From the current solution organized, following 4 part can be understood:

- NF profile extension including the energy related information (Sol#20,#21)

  - Solution#20 proposes to extend the NR profile to include the Energy-related NF profile attributes, which includes EnergySaving /Not EnergySaving State, Schedule of EnergySaving/not EnergySaving, Energy Priority Information, Schedule of Energy Priority Information.

- NF selection based energy related requirement and information (Sol#19, Sol#20,#28,#29)

  - Solution 19 proposes the AMF selection takes the Energy-related NF profile attributes

  - Solution 20 gives a common solution including the add the Energy-related NF profile attributes and the NF selection based on the added Energy-related NF profile attributes

  - Solution#28 considers an array of energy priority may be stored and NF discovery from NRF by inputting a energy policy which quired from the UDM.

  - Solution#29 proposes that the AMF can take Energy Priority Information or the Schedule of Energy Priority Information into account for N3IWF or TNGF reselection.

- UP path adjustment based on energy related information of DNAI/UPF (Sol#21,22, 23)

- NF Status to control NF traffic before activating energy saving state (Sol#30)

For the NF profile extension including the energy related information, EnergySaving /Not EnergySaving State and Schedule of EnergySaving/not EnergySaving can effectively reflect the energy status of the NF. For the “Energy Priority Information” or “Schedule of Energy Priority Information”, it is not clear on the differences to the exist “priority” attribute in the NF profile. Maybe based on implementation, the OAM can directly consider some energy related influences in the exist “priority” attribute in the NF profile, so the new introduced attribute “Energy Priority” seems redundant and not necessary. Moreover, if both the “Energy priority” and “Priority” attributes exist and without the same value in the NF profile, how to use the two attributes needs further clarification. Renewable energy can reflect whether the NF uses the renewable energy or not, if this attribute is considered for NF discovery and selection, it can achieve the purpose of the energy saving.

For the NF selection based energy related requirement and information, the intention of extend NF profile to include the energy related information is not only applied to UP plane (i.e., UPF), CP plane function (e.g., AMF) is also needed to be considered.

For the UP path adjustment based on energy related information of DNAI: UP path adjustment can effectively improve the energy efficiency and decrease the energy consumption, the AF, SMF or PCF can obtain the energy related information per DNAI or UPF from OAM (e.g., energy consumption information or renewable energy information) and adjust the UP path (UPF reselection) accordingly.

NF Status to control NF traffic before activating energy saving state as in solution 30, the intention is to avoid discovering an NF in the 5G core, which is to enter an energy saving state (according to TS 28.310 in the near future. It seems the Schedule of EnergySaving/not EnergySaving attribute can reflect this intention.

2	Proposal

It is proposed to capture the following solution in TR 23.700-67.

Start of Changes

7.1.3	Agreed Principles for KI#3

The following principles are agreed for KI#3:

-	For NF selection, the NF profile is enhanced to include the parameters in the table below. This is a feature requiring the software to be upgraded in NRF and its consumers.

| Information | Description |
| --- | --- |
| EnergySaving /Not EnergySaving State | Value of the energy state as specified in TS 28.310 [9], e.g. notEnergySaving state, Energy Saving state. (NOTE 1) |
| Schedule of EnergySaving/not EnergySaving | The schedule of the EnergySaving/not EnergySaving states, includes the value of EnergySaving or not EnergySaving state which applies at each time interval in the schedule. If this is present, the EnergySaving /Not EnergySaving State attribute is not present. |
| Renewable energy | Indicates whether the energy source of the NF is Renewable energy. |
| NOTE 1: EnergySaving/not EnergySaving refers to states of a network function which are defined in TS 28.310 [9]. |  |

NOTE:	The renewable energy information has dependency on SA5 and can be further updated based on SA5 feedback in the normative phase.

-	For UP path selection and reselection, the UPFs are selected/reselected by using NF discovery and selection procedures which may be Energy-aware if NF profile is updated as per bullets above.

-	UP path of PDU session may be adjusted based on the energy related information of a list of DNAI(s) or UPFs from the OAM.

End of Changes