3GPP TSG-SA WG2 Meeting #171	S2-2508389

Wuhan, China, October 13 – 17, 2025

Source:	CATT, Toyota

Title:	KI#3 Solution#21 update

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes updates to Solution #21 "UP path adjustment based on energy related information" for KI#3.

1. Introduction

There are the following editor's notes in Solution #21 "UP path adjustment based on energy related information" for KI#3:

Editor's note:	The triggers for UP path adjustment are FFS.

Editor's note:	How to select the UPF in case that a DNAI refers to multiple UPFs is FFS.

Editor's note:	Which NF, i.e. the AF, PCF or SMF, takes into account of energy related information per DNAI for DNAI selection is FFS.

1)	Triggers for UP path adjustment

As described in clause 6.21.1, “The energy related information to be taken into account for DNAI (re)selection can include the following:

-	Total energy consumption of the UP path associated with the DNAI, i.e. sum of the energy consumption of each UPF (including PSA UPF terminating N6, I-UPF and UL CL/BP UPF if any) involved in the UP path, per PDU Session or per Service Data Flow, and

-	Transmitted data volume over this UP path per PDU Session or per Service Data Flow; or

-	Energy efficiency of the UP path associated with the DNAI, i.e. the transmitted data volume over this UP path divided by total energy consumption of this UP path associated with the DNAI, per PDU Session or per Service Data Flow; or

-	Renewable energy consumption ratio of the UP path associated with the DNAI.”

A threshold for any of the above criteria can be configured in the AF/PCF/SMF according to operator policy. Then based on the notifications from the EIF, which can include the energy related information (e.g. energy consumption and corresponding data volume, energy efficiency, and/or renewable energy consumption ratio) per DNAI per Service Data Flow or per DNAI per PDU Session, the AF/PCF/SMF decides whether any of the configured threshold is reached and therefore UP path adjustment needs to be performed for the Service Data Flow or for the PDU Session.

2)	How to select the UPF in case that a DNAI refers to multiple UPFs

DN Access Identifier (DNAI) is an identifier of a user plane access to one or more DN(s) where applications are deployed. When multiple UPFs (i.e. PSA UPFs) can access to the same DNAI, or multiple UPFs are involved in the UP path towards the same DNAI, one of the PSA UPFs is selected along with other UPF(s) involved in the UP path as specified in clause 4.3.5 of TS 23.502 taking into account the energy related information related to DNAI (re)selection as described in 1), e.g. to maximize the energy efficiency (or renewable energy consumption ratio) of the UP path associated with the DNAI.

3)	Which NF, i.e. the AF, PCF or SMF, takes into account of energy related information per DNAI for DNAI selection

As specified in clause 5.51.2.2 of TS 23.501, “The Energy Information Function (EIF) collects the UE related Energy Consumption information including Node-level energy consumption information, Node-level data volume from OAM and data volume of the required granularities (i.e. S-NSSAI, PDU Session and/or Service Data Flow) from UPF (via SMF).” The collected UE related Energy Consumption information can further be used by the EIF for calculating the energy related information per DNAI per Service Data Flow or per DNAI per PDU Session as described in clause 6.21.1, and sending such information to the AF, PCF or SMF according to the subscriptions to assist in possible DNAI (re)selection.

Here it is proposed that the EIF calculates the energy related information per DNAI per Service Data Flow or per DNAI per PDU Session, instead of the AF, PCF or SMF performing the calculation, considering that:

-	the EIF is responsible for energy information calculation and exposure, and the EIF can calculate such information based on the collected energy related information;

-	it is hard for the AF or PCF to perform the calculation without sufficient information of UPFs and relevant energy related information;

-	the impacts to the AF, PCF or SMF can be minimized.

Then it is possible for the AF, PCF or SMF to take into account of energy related information per DNAI for DNAI (re)selection based on EIF notifications. Which NF takes such an action depends on the following conditions/decisions:

-	Operator policy or deployments.

-	Whether the SMF is enhanced to subscribe to energy related information notifications from the EIF.

-	Whether the PCF is enhanced to subscribe to energy related information notifications from the EIF.

Based on the above discussion, updates to KI#3 Solution #21 "UP path adjustment based on energy related information" are proposed, to resolve the ENs and etc.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-67.

* * * Start of Change * * * *

6.21	Solution #21: UP path adjustment based on energy related information

6.21.0	High-level solution principles

This solution proposes the following principles to be agreed:

-	The AF, PCF or SMF takes into account of energy related information for DNAI (re)selection to adjust the UP path of the PDU Session, during the AF influence on traffic routing procedure.

-	The AF, PCF or SMF subscribes from the EIF for energy related information (e.g. energy consumption and corresponding data volume, energy efficiency, and/or renewable energy consumption ratio) per DNAI per Service Data Flow or per DNAI per PDU Session, which is calculated by the EIF based on the collected Energy Consumption information from the OAM and UPF (via the SMF).

-	The AF or PCF selects/updates the DNAI list or the SMF selects the target DNAI taking into account the energy related information per DNAI per Service Data Flow or per DNAI per PDU Session, and the SMF reconfigures the user plane of the PDU Session towards the selected DNAI.

NOTE:	The adjustment of UP paths is determined based on operator policy, and frequent adjustment of UP paths for the PDU Session should be avoided.

6.21.1	Description

This solution addresses Key Issue #3.

Transmitting data traffic via different UP paths may have different energy efficiency and/or energy consumption, as different UP paths involves different 5GC NFs (e.g. different PSA UPFs, I-UPFs and/or UL CL/BP UPFs) which may have different energy performance. Therefore, one possible way of improving energy efficiency and/or reducing energy consumption in the network is to adjust the UP paths for transmitting the service data flows (i.e. UP paths of the PDU Sessions) based on energy related information.

The DNAI identifies a user plane access to one or more DN(s) where applications are deployed, and different DNAIs can correspond to different UP paths for transmitting the service data flows of the applications. Thus the existing mechanism of DNAI (re)selection mechanism, e.g. AF influence on traffic routing, can be extended by taking energy related information into consideration to achieve UP path adjustment for energy efficiency and energy saving.

As specified in clause 5.6.7 of TS 23.501 [2] and clause 4.3.6 of TS 23.502 [3], the AF may influence the DNAI selection by providing the list of DNAI(s) used by specific application(s), or the NEF can provide the mapping between the AF-Service-Identifier and a list of DNAI(s) based on local configuration when the DNAI(s) being used by the applications are statically defined. The list of DNAI(s) is then provided to the PCF (except in the case of HR SBO where Traffic Influence information is provided directly from V-NEF to V-SMF bypassing the PCF), and the PCF may update the list of DNAI(s), e.g. based on service experience analytics per UP path as defined in TS 23.288 [12]. Finally the PCF provides the list of DNAI(s) to the SMF and the SMF determines the target DNAI, using the target DNAI to trigger PDU Session modification or establishment to add, replace or remove UPF(s) (acting as UL CL/BP or PSA) in the UP path.

To support DNAI (re)selection based on energy related information, it is proposed to enhance the AF influence on traffic routing as following:

-	The AF may decide the list of DNAI(s) for the application(s) taking into account of energy related information, and provide the selected DNAI(s) with corresponding traffic steering policy identifier(s) to the NEF/PCF.

-	The PCF may update the DNAI(s) in the PCC rule taking into account of energy related information and operator policy.

-	The SMF may determine the target DNAI based on energy related information, and local configuration and/or the PCC rule (if available).

The energy related information to be taken into account for DNAI (re)selection can include the following:

-	Total energy consumption of the UP path associated with the DNAI, i.e. sum of the energy consumption of each UPF (including PSA UPF terminating N6, I-UPF and UL CL/BP UPF if any) involved in the UP path, per PDU Session or per Service Data Flow, and

-	Transmitted data volume over this UP path per PDU Session or per Service Data Flow; or

-	Energy efficiency of the UP path associated with the DNAI, i.e. the transmitted data volume over this UP path divided by total energy consumption of this UP path associated with the DNAI, per PDU Session or per Service Data Flow; or

-	Renewable energy consumption ratio of the UP path associated with the DNAI.

NOTE 1:	UP paths towards different DNAIs of the same DNN and UP paths towards the same DNAI can be considered when evaluating the energy related information per DNAI.

The 5GC NF (i.e. AF, PCF or SMF) can subscribe to the above information (i.e. energy related information per DNAI per PDU Session or per DNAI per Service Data Flow) from the EIF, which calculates the information based on the collected Energy Consumption information from the OAM and UPF (via the SMF) as described in clause 5.51.2.2 of TS 23.501 [2].

NOTE 2:	How the 5GC NF (i.e. AF, PCF or the SMF) decides to perform UP path adjustment based on the above information is up to implementation.

Compared to NF (re)selection based on energy related information, UP path adjustment considers the overall energy consumption and/or energy efficiency in the CN for transmitting the service data flow, instead of considering energy consumption and/or energy efficiency of only one NF.

6.21.2	Procedures

Figure 6.21.2-1: Procedure UP path adjustment based on energy related information

0a.	The AF, PCF or SMF subscribes to energy related information (e.g. energy consumption and corresponding data volume, energy efficiency, and/or renewable energy consumption ratio)  per DNAI per Service Data Flow or per DNAI per PDU Session as described in clause 6.21.1, from the EIF using the Neif_EventExposure_Subcribe service.

The AF, PCF or SMF includes Notification Target Address, Energy Consumption as Event ID, parameters reflecting the requested granularity (e.g., DNN/S-NSSAI, Application ID, Flow description(s)), and reporting parameters such as reporting threshold/period.

0b.	The AF, PCF, or SMF receives notifications of energy related information with corresponding DNAI and PDU Session ID from the EIF for the requested granularity (e.g., per PDU Session, per application ID, per traffic flow).

The AF, PCF, or SMF may decide to reselect the DNAI for specific Service Data Flow(s) or PDU Session(s), taking into account the received energy related information as described in clause 6.21.1, e.g. the Energy efficiency or renewable energy consumption ratio of the UP path associated with the DNAI for the Service Data Flow(s) or the PDU Session(s) is lower than the corresponding preconfigured threshold. Then the AF, PCF or SMF takes actions as described in the following steps.

1-7. An AF influence on traffic routing procedure is performed as described in clause 4.3.6.2 of TS 23.502 [3], with the enhancements as described as below:

-	Step 1, same as step 1 of figure 4.3.6.2-1 of TS 23.502 [3] with the enhancement that the AF may decide the list of DNAI(s) for the application(s) taking into account of energy related information per DNAI per Service Data Flow.

-	Steps 2-4 are the same as steps 2-4 of figure 4.3.6.2-1 of TS 23.502 [3].

-	Step 5, same as step 5 of figure 4.3.6.2-1 of TS 23.502 [3] with the following enhancement:

-	The PCF takes the energy related information per DNAI per Service Data Flow into account to determine whether the DNAI is optimal, e.g. by comparing energy efficiency of the path to a certain threshold (e.g., configured at the PCF), and updates the list of DNAI(s) in the traffic routing information of the traffic flow(s) in the PCC rule.

-	The PCF sends a Npcf_SMPolicyControl_UpdateNotify request to the SMF with the updated PCC rule.

-	Step 6, same as step 6 of figure 4.3.6.2-1 of TS 23.502 [3] with the enhancement that the SMF may (re)select the target DNAI taking into account of energy related information per DNAI per PDU Session.When multiple UPFs (i.e. PSA UPFs) can access to the same DNAI, or multiple UPFs are involved in the UP path towards the same DNAI, one PSA UPF is selected along with other UPF(s) involved in the UP path and the procedure of adding, replacing or removing a UPF in the UP path is performed as specified in clause 4.3.5 of TS 23.502 taking into account the energy related information related to DNAI (re)selection, e.g. to maximize the energy efficiency (or renewable energy consumption ratio) of the UP path associated with the DNAI.

-	Step 7 is the same as step 7 of figure 4.3.6.2-1 of TS 23.502 [3].

NOTE:	Which NF, i.e. the AF, PCF or SMF, takes into account of energy related information per DNAI for DNAI selection depends on operator policy or deployments, and TR conclusions (e.g. whether the SMF or PCF is enhanced to subscribe to EIF notifications).

6.21.3	Impacts on services, entities and interfaces

AF:

-	Subscribes to energy related information (e.g. energy consumption and corresponding data volume, energy efficiency, and/or renewable energy consumption ratio) per DNAI per Service Data Flow from the EIF.

-	Decides the list of DNAI(s) for the application(s) taking into account of energy related information per DNAI per Service Data Flow.

PCF:

-	Subscribes to energy related information (e.g. energy consumption and corresponding data volume, energy efficiency, and/or renewable energy consumption ratio) per DNAI per Service Data Flow or per DNAI per PDU Session from the EIF.

-	Updates the list of DNAI(s) for the Service Data Flow taking into account of energy related information per DNAI per Service Data Flow.

SMF:

-	Subscribes to energy related information (e.g. energy consumption and corresponding data volume, energy efficiency, and/or renewable energy consumption ratio) per DNAI per PDU Session from the EIF.

-	Determines the target DNAI for the PDU Session taking into account of energy related information per DNAI per PDU Session.

EIF:

-	Calculates the energy related information per DNAI per Service Data Flow or per DNAI per PDU Session and provide the information to the AF, PCF or SMF, based on the collected energy related information from the OAM and UPF (via the SMF) and according to the subscriptions from the AF, PCF or SMF.

* * * End of Change * * * *