3GPP TSG-SA WG2 Meeting #171	S2-2508390

Wuhan, China, October 13 – 17, 2025										(revision of S2-2507748)

Source:	CATT, NTT DOCOMO

Title:	KI#2 interim agreements

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes interim agreements for KI#2.

1. Introduction

Based on S2-2507748 which has been discussed in SA2#170 meeting, the interim agreements for KI#2 “Support of service adjustments for the UE” is proposed.

The main change on top of S2-2507748 is adding to the Agreed Principles that the PCF subscribes to energy related information (i.e. Energy Consumption information at UE, S-NSSAI, PDU Session and Service Data Flow granularity) from the EIF and taking this energy related information into account for making policy decisions. This enables that, the knowledge of energy related information (i.e. Energy Consumption information at UE, S-NSSAI, PDU Session and Service Data Flow granularity) in the EIF can be utilized by the PCF for more resilient and accurate policy control (e.g. SM policy control per specific PDU Sessions, PDTQ policy negotiation per specific services) according to the latest network and service status.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-67.

* * * Start of Change * * * *

7.1.2	Agreed Principles for KI#2

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#2. This may be populated directly or e.g. also when a topic in clause 7.2.2 gets resolved and a principle is agreed.

The interim agreements on principles for KI#2 are as follows:

-	The PCF may subscribe to notifications of the following energy related information from the EIF:

-	Energy Consumption information at UE, S-NSSAI (e.g. per UE per Network Slice), PDU Session and Service Data Flow (e.g. per UE per application) granularity.

-	The PCF may make the following policy control decisions for network energy saving based on subscription information, AF request and/or operator policy, and taking into account energy related information from the EIF:

-	SM policy control.

-	PDTQ policy negotiation with the AF, e.g. adjusting the time window and/or QoS for data transfer.

* * * Next Change * * * *

7.2.2	Topics for further consideration for KI#2

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#2. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.2) and can either be then marked as not pursued or postponed to a future Release.

The following topics/principles are for further consideration:

-	The PCF may subscribe to notifications of the following energy related information from the EIF:

-	renewable energy consumption ratio at UE, S-NSSAI (e.g. per UE per Network Slice), PDU Session and Service Data Flow (e.g. per UE per application) granularity.

-	The PCF may obtain the following energy related information from the OAM:

-	Energy Consumption information at Network Slice granularity.

-	renewable energy consumption ratio at Network Slice granularity.

Editor’s note:	Whether and how the renewable energy related information can be collected relies on SA WG5, so feedback from SA WG5 is needed to conclude renewable energy aspect.

-	The PCF may generate PCC rules with energy related QoS parameters sets. The SMF may generate energy related QoS profile based on PCC rules with energy related QoS parameters sets or based on local policy, and provide the energy related QoS profile to the NG-RAN. NG-RAN selects the energy related QoS profile only if NG-RAN decides to downgrade the QoS due to energy saving. NG-RAN notifies the selected energy related QoS profile to SMF, SMF will further inform the selected energy related QoS profile to PCF and to UPF for charging purpose.

NOTE:	One or more energy related QoS Parameter Sets can be provided in a prioritized order.

Editor’s note:	Whether and how the energy related QoS profile can be used by NG-RAN for energy saving needs feedback from RAN WGs.

-	The PCF may make the following policy control decisions for network energy saving:

-	UE policy control, e.g. generating and updating URSP rules.

-	The PCF add "reason for a policy change" in charging info of the associated PCC rule when service adjustment is applied for an SDF. SMF sends the reason for a policy change together with existing charging info to CHF.

-	The AF may indicate to the network the preferred or expected network energy saving behaviors per application. The PCF may perform energy saving SM policy control based on the AF provided service parameters (e.g. activate different UE subscribed energy saving service levels which lead to different service experiences).

* * * End of Change * * * *