SA WG2 Meeting #171	S2-2508399

13 – 17 Oct, 2025, Wuhan, CN

Source:	China Telecom

Title:	[WT#1.2, QoS framework] KI Proposal for QoS

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes to study the key issue UE/application and network QoS collaboration, to better support the 6G services. It aims to address: how the UE and application provide service related information to the network for QoS collaboration and how the network maps the information to the QoS characteristics or parameters, etc.

Annex A.X WT#1.2.X: Evolution of QoS Framework for 6G

Editor’s Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

- Investigate if any new functionality in 6G QoS framework considering the emerging new traffic characteristics and application needs.

NOTE 1: The definition of 6GS QoS concepts in the 6G System provider perspective and in the 6G System usage perspective respectively should have clear boundary (enable clear & measurable connectivity service descriptions for SLAs).

- Whether and how to enhance QoS control mechanisms to

  - better support QoS targets than the best effort approach of existing non-GBR and less resource intensive than the existing GBR, Delay Critical-GBR to improve capacity utilization in the network.

  - to improve QoS for applications with dynamic requirements.

- Whether and what to monitor in relation to QoS Framework.

- Study whether and how to support the UE and network QoS collaboration to improve awareness in the 6GS of connectivity performance needs.

  - UE provided information to the network for QoS collaboration

  - Application and Network QoS collaboration including what information shall be exchanged by means of ‘application/UE and network collaboration’ [see WT#1.2 User Plane].

NOTE 2: Applications typically consist of an Application Client in the UE and application on the server side.

- Study how to ensure interworking with 5GS.

NOTE 3: The content of the 5GS QoS Framework that will be used as a starting point for discussion on 6G QoS will be determined by moderated discussion process.

NOTE 4: Aspects of traffic detection are assumed to be handled by the WT#1.2 User Plane.

NOTE 5: Impacts on QoS originating from new services will be considered based on and when relevant input is identified and provided by the relevant WTs.

NOTE 6: This WT will coordinate with SA WG4 and RAN WGs.

**** First Change ****

5.X. Key Issue #X: UE/application and network QoS collaboration

5.X.1	Description

This key issue will address:

- Whether and how the UE provides information to the network for QoS collaboration.

- To provide QoS decision assistance information to the core network to make QoS decision based on this information. The QoS decision assistance information is related to the services.

- To provide QoS policy negotiation information to the core network to make QoS decision based on this information. The QoS policy negotiation information can include the recommended values for QoS parameters such as MFBR, Session-AMBR, etc.

- Whether and how the application provides information to the network for QoS collaboration.

- To provide information either by NAS or AF, where the information can be related to the connectivity services or beyond-connectivity services.

- To map the information related to services to QoS characteristics or parameters, where the QoS characteristics or parameters can be related to the connectivity services or beyond-connectivity services.

**** End of Changes ****