SA WG2 Meeting #171	S2-2508402

13 – 17 October, 2025, Wuhan, China	(revision of S2-25xxxxx)

Source:	China Telecom

Title:	KI#1: Interim agreements for System Architecture to Support Sensing Service

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This paper proposes interim agreements for KI#1 to address Sensing Service system architecture design.

1. Discussion

According to TR 23.700-14, KI#1 address the general system architecture and function enhancements required to provide end-to-end support for Sensing. The aspects that need to be concluded include the introduction of the new NF, the information interaction between the Sensing Entity and the new NF, and the general description of the features listed in KI#2, KI#3, KI#4, KI#5 and KI#6.

Based on the solutions, the following aspects need to be addressed for KI#1.

- New network function to support Sensing Service:

All TR solutions suggest that a new Sensing Function (SF) shall be introduced in the 5GC to support Sensing Service, rather than reuse the existing 5GC NF. In addition, some companies propose that the Sensing Function can be splitted into Sensing Control Function (SCF) and Sensing Processing Function (SPF), the definition copied from solution #3 are as following:

Sensing Control Function: Indicating the logical function which is involved to support Sensing Service and responsible for providing the related configuration information.

Sensing Processing Function: responsible for receiving the 3GPP Sensing Data and for performing the elaboration for determining the Sensing Result.

Observation#1: Based on the 3GPP Sensing Data, the generation of Sensing Result demands significantly more computation resources than the sensing signaling handling. With more gNBs deployed for data collection, the volume of 3GPP Sensing Data will significantly increase, while the sensing signaling only increases a little bit.

Proposal#1: It is proposed to separate the sensing signaling control function and the sensing data processing function of the Sensing Function, introduce the SCF to responsible for sensing task management and control and the SPF to responsible for sensing data collection and sensing result calculation.

- Interface between gNB Sensing Entity and the New network function:

With the introduction of the Sensing Function (SF) and the function split, according to the existing solutions, the gNB and SPF can establish the direct connection for sensing data transmission, the sensing signaling can be delivered from the SCF to the gNB directly or via the AMF.

Observation#2: The AMF is generally deployed in regional data center which is far from the sensing campus. Therefore, for Sensing Service in a localized campus, if the sensing signaling is delivered from the SCF to the gNB via the AMF, it will experience higher latency than the direct connection and the AMF needs to be upgraded, which increases the deployment complexity of Sensing Service.

Observation#3: However, for large-scale commercial deployment scenario, the direct connection will introduce too many interface between the SCF and the large number of gNBs.

Proposal#2: For 5G-A gNB based sensing, it is proposed that the 3GPP Sensing Data is transmitted from the gNB to the SPF directly, and the sensing signalling is delivered from the SCF to the gNB directly and optionally via the AMF.

Observation#3: The SCF is supposed to connect to the SBI bus and register itself in the NRF so that can be discovered by other CN NFs, while the SPF only need to be discovered by the SCF and can be deployed near to the gNB to reduce the sensing data transmission latency.

Proposal#3: It is proposed that the SCF register itself in the NRF and is configured with the SPF information. It is feasible to adopt reference point level association setup, update, release and report between the SCF and SPF, the interface between SCF and SPF is not necessary to be standardized.

- Sensing result storage and management:

Observation#4: When perform the sensing service for UAV tracking, the sensing result in a certain peroid of time may need to be stored and reused for predicting future flight trajectory. To satisfy the regulation requirements, sometimes it’s also required to store/retain the flight data within a period of time.

Proposal#4: Considering time limitation, the sensing result storage can be implemented as the internal logic of the SCF/SPF, a dedicated sensing result storage NF and the interface between the sensing result storage NF and the SCF/SPF can be studied in 6G sensing.

Therefore, the overall 5GS architecture to support sensing is proposed as following:

2. Proposal

It is proposed to agree the following changes into TR 23.700-14 V1.0.0.

# * * * * First change (All Text New) * * * *

7	Interim agreements

7.1	Agreed Principles

7.1.Y	Agreed Principles for KI#1

The following principles are agreed for KI#1 System Architecture to Support Sensing.

1. The introduction of new CN function:

- A new core network function, Sensing Function (SF), is introduced to support Sensing Service in 5G system architecture, and the Sensing Function can further be splitted to two logical functions:

- Sensing Control Function (SCF): responsible for the sensing task management and control, i.e., receiving the sensing service request(s), performing the sensing service authorization and revocation, providing the related configuration information to the gNB and the SPF, and storing the Sensing Result.

- Sensing Processing Function (SPF): responsible for receiving the 3GPP Sensing Data, aggregating the 3GPP Sensing Data from multiple gNB sensing entities, and generating the Sensing Result.

NOTE 1:	The interface between the SCF and the SPF does not need to be standardized. Reference point level association setup, update, release and report between the SCF and SPF can be defined.

2. The basic procedure to support end-to-end Sensing Service:

The overall 5GS architecture to support sensing is proposed as following:

Figure 7.1.1-1: 5G System Architecture to Support Sensing

- The SCF registers itself in the NRF, so that the SCF can be discovered and selected by other CN NFs.

- The SCF receives the sensing service request(s) from the Sensing Service Consumer.

NOTE 2:	In 5G-A, the Sensing Service Consumer only refers to the trusted AF and untrusted AF. If the Sensing Service Consumer is an untrusted AF, the NEF performs the authorization of the AF and the selection of the SCF, so that the sensing service request can be delivered to the SCF via the NEF.

- The SCF performs the authorization of sensing service request and the revocation of an on-going sensing service when triggered by some conditions. The detailed authorization and revocation procedures are described in KI#2 interim agreements.

- The SCF discovers and selects one or multiple gNB(s) as Sensing Entity based on the target sensing area and the gNB(s) capability information. The detailed gNB discovery and selection procedures are described in KI#3 interim agreements.

- The SCF sends the sensing service request and the sensing configuration parameters to the gNB(s). The SCF can communicate with the gNB via a new direct interface for sensing signalling delivery, and optionally via the AMF (e.g, for large scale commercial usage). The detailed sensing configuration parameters provision procedures are described in KI#6 interim agreements.

NOTE 3:	The protocol stack used between the SCF and the gNB will be determined by RAN WG3.

- Based on the received sensing service request(s) and the sensing configuration parameters, the gNB(s) perform object detection/object tracking to obtain the 3GPP Sensing Data. And then establishes a new data connection towards the SPF for data transfer. The detailed sensing data collection and transfer procedures are described in KI#4 interim agreements.

NOTE 4:	The protocol stack used between the SPF and the gNB, and the data format will be determined by RAN WG3.

- The SPF receives 3GPP Sensing Data from one/multiple gNB(s) and performs sensing result generation. When generated the sensing result, the SPF sends the sensing result to the SCF, and the SCF further exposes the sensing result to the Sensing Service Consumer. The detailed sensing result generation and exposure procedures are described in KI#5 interim agreements.

NOTE 5:	If the Sensing Service Consumer is an untrusted AF, the SCF exposes the sensing result to the AF via the NEF.

# * * * * End of changes * * * *