SA WG2 Meeting #171	S2-2508417

Wuhan, China, October 13 - 17, 2025

Source: 	vivo

Title: 	Interim agreements on KI#4: Sensing Data and the Associated Information Collection and Transport

Document for: 	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This contribution provides interim agreements on KI#4: Sensing Data and the Associated Information Collection and Transport.

1	Discussion

According to the TR 23.700-14 V1.0.0, there are 12 solutions (#1, #3, #5, #18, #21 - #26, #34, #39) related to the KI#4 Sensing Data and the Associated Information Collection and Transport.

|  | AMF or not | How to transfer sensing request | How to transfer sensing data | Who generate result | associated information for Sensing result generation |
| --- | --- | --- | --- | --- | --- |
| #1 | No | Direct interface over direct connection | Direct interface over direct connection (by UP address, transport protocol and required security information over CP or UP connection) | Sensing Function |  |
| #3 | No | Direct interface? | the path for 3GPP sensing data reporting. (establishment is triggered by SE) | SF (incl. control & processing) |  |
| #5 | Yes | CP (via AMF) | CP (via AMF) | SF |  |
| #18 | Yes & No | Direct data connection? | direct data connection (by connection info [address or end point info of SF] and binding information [e.g. Sensing Entity ID, connection ID or specific binding ID] over CP or direct interface) | SF |  |
| #21 | No | Direct interface | Direct interface | SF | correlation ID |
| #22 | No | Direct interface | Data tunnel (by exchange [SF tunnel info and sensing service ID] and [SE tunnel info and operation execution result indication]) | SF | the Sensing service ID |
| #23 | No | Direct interface? | Direct interface? | SePF (SeMF) | Sensing Service Request (SSR) Identifier, response code |
| #24 | Yes | CP | Data connection (by SDCF endpoint information (e.g. SDCF ID/ FQDN over CP) | SSMF (SDCF) |  |
| #25 | No | Direct interface? | Direction connection (by SCF identifier and address, sensing session ID, sensing service identification and SPF connection configure information) | SPF (SCF) Can be one |  |
| #26 | No | new interface defined between the SCF and the gNB. | data reporting tunnel (e.g. the IP address of the SPF, the Sensing Task ID) | SPF (SCF) Can be one | the Sensing Task ID |
| #34 | No | New interface | data connection | SPF (SCF) Can be one |  |
| #39 | Yes | CP (an enhanced version of NGAP or a new protocol for sensing defined between the endpoints as AMF and gNB can be used.) | CP | SF |  |

Based on above summary, it proposes the following interim agreements for KI#4:

- Sensing data and the associated information, if any, are transferred from the Sensing Entity to the Sensing Function via the dedicated data tunnel. The data tunnel is established by exchange of tunnel information (e.g. IP address, terminal endpoint info) between the Sensing Entity and the Sensing Function. Sensing service request is transferred from the Sensing Function to the Sensing Entity to trigger sensing data and the associated information collection and transport.

- The SF performs sensing result generation based on the received sensing data from the Sensing Entity.

- The associated information is required by the Sensing Function for sensing result generation. The required associated information includes correlation ID (e.g. sensing service ID) to bind the generated sensing result and the requested sensing service.

2 	Proposal

It is proposed to capture the following contents into TR 23.700-14 V1.0.0.

# * * * * First change* * * *

7	Interim agreements

7.1	Agreed Principles

7.1.4	Agreed Principles for KI#4

The following interim agreements on KI#4: Sensing Data and the Associated Information Collection and Transport, are made:

- Sensing data and the associated information, if any, are transferred from the Sensing Entity to the Sensing Function via the dedicated data tunnel.

- The data tunnel is established by exchange of tunnel information (e.g. IP address, terminal endpoint info) between the Sensing Entity and the Sensing Function.

- Sensing service request is transferred from the Sensing Function to the Sensing Entity to trigger sensing data and the associated information collection and transport.

- The SF performs sensing result generation based on the received sensing data from the Sensing Entity.

- The associated information is required by the Sensing Function for sensing result generation. The required associated information includes correlation ID (e.g. sensing service ID) to bind the generated sensing result and the requested sensing service.

NOTE 1:	The detailed protocol used for the data tunnel between the Sensing Entity and the Sensing Function are in scope of RAN WG3.

NOTE 2:	Other associated information required by the Sensing Function for sensing result generation will be determined in the normative phase.

# * * * * End of change* * * *