SA WG2 Meeting #171	S2-2508453

13 – 17 Oct., 2025, Wuhan, China

Source:	OPPO, CATT.

Title:	Update of solution#13: IMS call setup procedure without pre-condition for voice over GEO satellite

Document for:	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC/Rel-20

Abstract of the contribution: This contribution is to update the solution to make the description better organized and clearer.

1	Discussion

Some description in solution#13 is not fully clear or aligned. This contribution is to update the solution to make the description better organized and clearer.

2	Proposal

It is proposed to include the following changes in TR 23.700-19 V0.2.0.

* * * * Start of Changes * * * *

6.13	Solution #13: IMS call setup procedure without pre-condition for voice over GEO satellite, AS as B2BUA

6.13.0	High-level solution Principles

This solution is to address Key Issue #2: IMS enhancement for GEO NB-IoT NTN access.

This solution is to realize IMS call setup without pre-condition for MO/MT UE connected via GEO, so as to avoid some SIP messages exchanged via GEO access and reduce the call setup time.

6.13.1	Description

Procedure in this solution is basically using IMS Call Setup without pre-condition for MO/MT UE via GEO, with the following clarification/change:

1)	Capability of Optimized voice is negotiated during IMS REGISTER. For a UE supporting GEO voice, it needs to support Codec for GEO voice (e.g. Codec-2) and processing SIP INVITE with no pre-condition. For the IMS network allowing GEO voice to UE, it needs to support transcoding and B2BUA service when either MO or MT UE is connecting via GEO.

2)	SigComp capability negotiation during IMS REGISTER.

3)	For "no pre-condition" during IMS Call Setup,

Option-1: the "Authorizing QoS Resource" is skipped and UE uses an existing IP-CAN bearer (e.g. default or pre-established) for media based on the media parameters received in the SDP offer/answer. The Establishment of EPS bearer for IMS voice is skipped.

NOTE 1:	Since the NB-IoT device has at most 2 EPS bearers over UP, and the GBR bearer is not supported over GEO Satellite access, it is proposed to transmit both IMS signalling and Media via one or two non-GBR EPS bearer.

Option-2: Dedicated bearer is established in resource reservation during call setup.

For IMS signalling, UE uses a default bearer to transmit the signalling message.

Editor's note:	It depends on KI#1 to decide how to transmit the IMS signalling and media flow via EPS Bearers without Authorizing QoS resource during IMS Call Setup.

4)	B2BUA (B2B User Agent) can be used to support the interaction with normal MO/MT UEs.

HSS provisions iFC to S-CSCF based on RAT type (GEO access) and UE's Optimized voice capability for involving AS acting as B2BUA in the UE's IMS call setup signalling path.

5)	Estimation of call setup time:

-	SIP message size.

According to example in clause 7.2.3 of TS 24.228 [21], the message size would be about:

SIP INVITE: 1.4Kbyte; 200 OK: 0.8KByte.

NOTE 2:	Given the codec is very simple for GEO voice, the SDP offer will be a basic one, the 1.4KB for SIP INVITE, and 0.8KB for 200 OK should be enough (No 183 in no pre-condition case).

Additionally, considering "SigComp" for compressed SIP used for Call setup, the compression efficiency of SIP message is about 2:1 (basic value), thus 0.5*SIP message size is the estimated SIP size after compression.

-	PDB (AN+CN).

Referring the 5QI=10 for satellite and the PDB (include AN and CN) is 1100 ms.

-	Data rate over GEO access.

Referring the [1-3K] for UL and DL, the data rate for GEO in SA1 requirements.

Given the solution assumes to associate IMS signalling and media data into existing bearer, i.e. no establishment during call setup, there is no time consumed for this part. Therefore, the estimation of total latency over GEO is:

SIP INVITE (SDP Offer): latency = 0.5*1400*8/(1~3)Kbps+1.1s=2.97~6.7s

SIP 200 OK (Offer Response): 0.5*800*8/(1~3)Kbps+1.1s=2.17~4.3s

NOTE 3:	The Ring can be sent in parallel with 200 OK which is not increase the total call setup time.

Then roughly estimate the transmission in IMS network side is about 1 second. And the transmission time for TN UE side is 2 second (with pre-condition).

The total E2E latency for call setup time is:

GEO_UE-to-TN_UE = (2.97~6.7s) + (2.17~4.3s) + 1s + 2s = 8.14 ~16s

GEO_UE-to-GEO_UE = 16.28 ~ 32s

Editor's note:	The estimation of call setup time over GEO can be further evaluated.

In the following procedure clause, the call setup has two options:

Option-1: The IMS serving the UE connecting to GEO access acts as B2BUA. In this case, during call setup, the normal UE's IMS procedure will not be impacted.

Option-2: Only the MT SCC AS acting as B2BUA for pre-condition. During call setup, if the MO UE and MT UE have different requirement for pre-condition, e.g. only one side of the call is accessing via GEO satellite, then the MT IMS needs to do B2BUA for pre-condition.

6.13.2	Procedure

6.13.3.1	IMS Register

Figure 6.13.3.1: IMS REGISTER for Optimized voice capability negotiation

The purpose of capability negotiation is to determine the GEO voice service is allowed for the UE. Then accordingly, IMS NFs may perform transcoding configuration and B2BUA during the Call Setup.

Step 1-2:

When UE is connecting via GEO, the UE sends SIP Register to P-CSCF in which the SIP Header includes "Capability of GEO voice". By indicating "Capability of GEO voice", it means the UE supports codec(s) for GEO voice (e.g. Codec-2). The PANI header indicates "GEO access".

Meanwhile, the UE may send capability of "SigComp" in SIP header to P-CSCF for Compressed SIP usage during call setup procedure.

When UE is no longer connecting via GEO, the UE performs (re-)RSGITER update to P-CSCF.

NOTE:	The steps of S-CSCF discovery is not described, but following existing procedure.

Steps 3-7:

The existing procedure for authorization/authentication for IMS layer.

Step 8:

S-CSCF sends Diameter message to HSS with the "GEO access" and "Capability of Optimized voice". The HSS downloads subscription data incl. service parameter and iFC to S-CSCF.

NOTE:  the iFC is configured differently per option-1 and option-2 for IMS call setup procedure in the following clauses.

Step 9:

Based on the subscription data retrieved from HSS. The S-CSCF may perform Service Control. Specifically, S-CSCF determines whether the UE is allowed for GEO voice service, and install the iFC for the B2BUA service in Call Setup procedure.

Then S-CSCF sends 200 OK to P-CSCF with "confirm of GEO voice".

Step 10:

The P-CSCF stored the "Confirm of GEO voice", based on which the P-CSCF may instruct AGW to perform transcoding actions described in the Call Setup procedure. Meanwhile, if the UE sends SigComp capability in step-1, the P-CSCF also needs to responds to indicate applying SigComp for Compressed SIP message for Call Setup procedure.

The P-CSCF sends 200 OK to UE indicating the "Confirm of GEO Voice".

Steps 11-12:

If the iFC is installed in S-CSCF, the S-CSCF sends REGISTER to AS to check the availability of B2BUA.

After the IMS register, the call setup is performed either in option-1 and option-2 where the AS acts as B2BUA.

6.13.3.2	Option 1: The IMS serving the GEO UE access acts as B2BUA

6.13.3.2.1	IMS Call Setup-MO

Figure 6.13.3.2: IMS Call Setup-MO (with B2BUA)

Steps 4-13 is described as below for B2BUA. These steps can be skipped if no B2BUA is needed.

Step 1:

MO UE connecting via GEO initiates SIP INVIATE with SDP Offer including GEO voice codec and no pre-condition setting parameters (e.g. Support/Require header does not include "precondition").

Steps 2-3:

P-CSCF modifies the codec list in SDP Offer by adding legacy codec such as AMR/EVS, and send the INVITE* to S-CSCF

Step 4:

By checking iFC (Filter Criteria), the SIP INVITE with no pre-condition are routed to AS supporting B2B User Agent.

Step 5:

The AS, as B2B User Agent, terminates the original INVITE and initiate a new INVITE with pre-condition setting, e.g. "Support: precondition" in SIP header, and/or "a=curr:", "a=des:", "a=conf:" in SDP offer.

NOTE:	The modification of codec list and transcoding configuration to MRF may also be performed by B2B User Agent. If so, the P-CSCF does not necessarily do the transcoding actions in step 2.

Step 6:

The AS sends the new INVITE with pre condition setting to S-CSCF, and S-CSCF sends the new INVITE to the MT side.

Step 8-11:

Acting as B2BUA, the AS process the steps for media negotiation and resource reservation with the MT side. All the messages are transmitted via S-CSCF based on iFC.

Step 13:

When receiving final 200 OK, the AS initiate a new SIP: 200 OK with no pre-condition setting.

Steps 14-17:

The 200 OK (SDP Answer) sends to UE, during which the P-CSCF configures the transcoding to Access GW.

Step 18:

The UE uses an existing (e.g. default or pre-established) IP-CAN bearer for media based on the media parameters received in the SDP answer. The P-CSCF does not instruct PCRF to establish a new dedicated EPS bearer for voice.

6.13.3.2.2	IMS Call Setup-MT

Figure 6.13.3.2: IMS Call Setup-MT (with B2BUA)

Steps 2-4, 8-11, 17-18 described below are for B2BUA. Those steps can be skipped if no B2BUA is needed.

Steps 1-2:

When receiving SIP INVITE (Legacy codecs in SDP offer, pre-condition setting) from MO, the S-CSCF forwards the INVITE to AS (B2BUA) based on iFC.

Step 3:

The AS terminates the original SIP INVITE and initiate a new SIP INVITE (no pre-condition setting) for the MT UE.

NOTE:	The modification of codec list and transcoding configuration to MRF may also be performed by B2B User Agent. If so, the P-CSCF does not necessarily do the transcoding actions in step 6.

Steps 4-5:

The new SIP INVITE is sent to P-CSCF via S-CSCF.

Step 6:

The P-CSCF determines to modify the codec list by adding codec for GEO voice (e.g. Codec-2) in SDP Offer. Then it sends the INVITE with updated SDP* to the UE connecting via GEO.

Step 7:

The UE uses an existing (e.g. default, or pre-established) bearer for media based on the media parameters received in SDP offer. The P-CSCF does not instruct PCRF to establish a new dedicated EPS bearer for voice.

Steps 8-11:

In parallel to step 4-7, the AS, as B2B User Agent interact, with MO side for media negotiation and resource reservation. All the messages are transmitted via S-CSCF.

Step 13:

UE sends 180 Ringing when alerting user.

Step 14:

When user pick up the phone, the MT UE sends final 200 OK to S-CSCF/P-CSCF. The P-CSCF configures transcoding to Access GW.

Steps 15-19:

The AS initiates a new SIP 200 OK and sends to the MT side via the S-CSCF.

6.13.3.3	Option 2: The MT SCC AS acting as B2BUA for all cases (i.e. GEO-GEO, GEO-TN, and TN-GEO)

This option-2 proposes to not use precondition for call setup when GEO satellite access is involved, while the IMS network still acquires the status of resource allocation during the call setup to avoid the ghost ring.

On the other hand, the IMS network needs to be capable of acting as "proxy" to handle precondition negotiation as the MO UE and MT UE may have different requirements for precondition.

In this solution, the SCC AS serving MT UE is proposed to act as this kind of proxy (i.e. B2BUA) for all cases (i.e. GEO-GEO, GEO-TN, and TN-GEO) for the following reasons:

-	SCC AS is mandatory node in the mobile networks and at least for terminating domain selection (T-ADS).

-	SCC AS can acquire MT UE's GEO voice capability and access type, which can be used to determine whether precondition can be skipped for the MT UE.

-	Compared with some solutions where P-CSCF or IMS AS serving MO UE acts as B2BUA, this solution can avoid unnecessary signalling interaction. For example, when P-CSCF serving the GEO UE acts as B2BUA, the P-CSCF may have to always add or remove the precondition indication because it has no knowledge of the access type of the other side UE (and it is unnecessary to exchange the access types of the MO UE and MT UE between the originating and terminating network).

-	The SCC AS solution is more easy to be implemented compared with P-CSCF solution, since as a centralized node, the number of SCC AS is much fewer than that of the P-CSCF.

6.13.3.3.1	Both MO UE and MT UE are accessing network via GEO satellite

The Figure 6.13.3.3-1 shows the call setup procedures when both MO UE and MT UE are accessing network via GEO satellite. The P-CSCF1/S-CSCF1/SCC AS1 are the serving nodes of MO UE, and P-CSCF2/S-CSCF2/SCC AS2 are the serving nodes of MT UE.

Figure 6.13.3.3-1: Both MO and MT UE access the network via GEO satellite

The UE1 and UE2 have registered in the IMS network. The UEs support the voice over GEO satellite and will skip the precondition negotiation in the call setup.

1.	UE1 initiates the call without the requirement of precondition. The codec for GEO satellite is included in the SDP.

2.	The serving P-CSCF1 of UE1 triggers the dedicated resource allocation.

3.	The P-CSCF1 forwards the SIP Invite to terminating side via S-CSCF1, and P-CSCF1 updates the codecs in the SDP by adding the codecs supported by the network.

4a.The S-CSCF2 sends the SIP Invite to the SCC AS2 based on the IFC trigger for SCC AS, and SCC AS2 retrieves TADS information and UE's GEO voice capability from HSS with the UDR/UDA procedure. TADS information contains the GEO access type. The IFC trigger is provisioned as subscription data of the UE.

4b. SCC AS2 determines whether the precondition is required for UE2, based on GEO voice capability and GEO access type of UE2 retrieved from IMS HSS and the “no precondition” information in SIP INVITE. .

5.	Since SCC AS2 determines precondition does not apply to UE2, the SCC AS2 forwards the SIP INVITE to P-CSCF2 via S-CSCF2 without changing the received no-precondition information.

6.	If needed, the P-CSCF2 may modify the codec list containing codec of GEO voice.. However, in this case, given the MO UE and MT UE are supporting GEO voice capability, the P-CSCF does not necessarily need to modify the codec list.

7.	UE2 replies with 180 Ringing.

8.	The P-CSCF2 triggers the dedicated resource allocation for UE2.

9.	After receiving the 180 Ringing, SCC AS2 holds the message and waits for the completion of resource allocation of UEs.

10.	UE1 sends the SIP INFO to the network indicating that its dedicated resource has been allocated. SDP is not needed in the SIP INFO message. The message is sent to SCC AS2 via P-CSCF1/S-CSCF1/S-CSCF2.

11.	Similarly, UE2 sends the SIP INFO to SCC AS2 via P-CSCF2/S-CSCF2 when its dedicated resource has been allocated.

12.	SCC AS2 confirms that resource allocation has been successful and forwards the SIP 180 Ringing to the originating side via S-CSCF2. Then Call setup continues.

6.13.3.3.2	Only MO UE is accessing network via GEO satellite

The Figure 6.13.3.3-2 shows the call setup procedures when only MO UE is accessing network via GEO satellite.

Figure 6.13.3.3-2: Only MO accesses the network via GEO satellite

The UE1 and UE2 have registered in the IMS network. The UE1 support the voice over GEO satellite and will skip the precondition negotiation in the call setup. While the UE2 is not accessing via GEO satellite and it requires precondition. SCC AS2 acts as B2BUA for precondition negotiation.

1-4.	Same as the steps 1-4 in clause 6.13.2.1.

5.	In this case, SCC AS2 determines that precondition is required by UE2, then SCC AS2 acts as B2BUA and updates the SIP header indicating precondition is required, then sends the request to P-CSCF2 via S-CSCF2.

6.	The P-CSCF2 may modify the codec list according to its own configuration, and the codec for GEO satellite may be included.

7.	UE2 replies with 183 Session Progress.

8.	The P-CSCF2 triggers the dedicated resource allocation for UE2.

9-10.	SCC AS2 acts as B2BUA for the precondition negotiation with UE2 and waits for the resource allocation status from MO side.

11.	UE1 sends the SIP INFO to the SCC AS2 via originating network and S-CSCF2 indicating that its dedicated resource has been allocated.

12-14.	SCC AS2 confirms that resource allocation for UE1 has been successful and continues the call setup procedure.

6.13.3.3.3	Only MT UE is accessing network via GEO satellite

The Figure 6.13.3.3-3 shows the call setup procedures when only MT UE is accessing network via GEO satellite.

Figure 6.13.3.3-3: Only MT UE accesses the network via GEO satellite

The UE1 and UE2 have registered in the IMS network. The UE2 support the voice over GEO satellite and will skip the precondition negotiation in the call setup. While the UE1 is not accessing via GEO satellite and it requires precondition. SCC AS2 acts as B2BUA for precondition negotiation.

1.	UE1 initiates the call with the requirement of precondition, and the SIP Invite is transferred to from originating network to terminating side SCC AS2 via S-CSCF2.

2a-2b.	SCC AS2 retrieves TADS information and UE's GEO voice capability from HSS based on the IFC trigger, and determines whether the precondition is required for UE2.

3.	In this case, SCC AS2 determines precondition is not required by UE2, then SCC AS2 acts as B2BUA and updates the SIP header indicating precondition is not required, then sends the request to P-CSCF2 via S-CSCF2.

4.	The P-CSCF2 modifies the codec list according to its own configuration, e.g. adding the codec for GEO satellite access.

5.	UE2 replies with 180 Ringing.

6.	The P-CSCF2 triggers the dedicated resource allocation for UE2.

7-10.	After receiving the 180 Ringing, SCC AS2 acts as B2BUA for the precondition negotiation with UE1 and waits for the resource allocation status update from UE2.

11.	UE2 sends the SIP INFO to the SCC AS2 via P-CSCF2/S-CSCF2 indicating that its dedicated resource has been allocated.

12-13.	SCC AS2 confirms that resource allocation for UE2 has been successful and continues the call setup procedure.

6.13.3	Impacts on Services, Entities and Interfaces

The impacts below apply to both option-1 and option-2 unless it is clarified.

MO/MT UE:

-	Support capability negotiation and indicating the GEO access in IMS REGISTER

-	Support Codec for GEO voice.

-  Support skipping precondition negotiation

-  for option-2: reporting resource allocation successful status to network

P-CSCF:

-	support capability negotiation in IMS REGISTER.

AS:

-	Support B2BUA

-  For option-1: Codec list modification (optional)

-  For option-2: SCC AS checks if acting as B2BUA for pre-condition negotiation is required

IMS HSS:

-	Support new subscription parameter for GEO voice.

AGW/MRF:

-	for option-1: Support codec transcoding (optional) .

* * * * End of Changes * * * *