SA WG2 Meeting #S2-171	S2-2508496

13 - 17 October, 2025, Wuhan, China

Source:	Samsung

Title:	KI#4 Interim Agreements

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC /Rel-20

Abstract of the contribution: This contribution proposes the potential interim agreements for FS_Sensing_ARC KI#4.

# Discussion

In TR 23.700-14 V1.0.0 (2025-09), there are 12 solutions documented for KI#4 Sensing Data and the Associated Information Collection and Transport.

This contribution provides comprehensive analysis on all documented solutions on KI#4 and proposes the potential interim agreements for SI conclusions.

Solution#1 proposes that the Sensing Function uses the direct connection to request the sensing service operation at the Sensing Entity(ies) and obtain the Sensing Data and Associated information from the Sensing Entity(ies) to support transporting large amount of data. The Sensing Function uses the direct interface to manage the sensing service operation at the Sensing Entity(ies), and obtaining the Sensing Data and Associated information from the Sensing Entity(ies). The direct connection could be established by providing for example the user plane address, transport protocols and required security information to the identified Sensing Entity, e.g. establish a TLS connection to a provided IP address. The signalling between the Sensing Function and the Sensing Entity can make use of any of the available connections between them, e.g. the control plane signalling or an already established user plane connection.

Solution#3 proposes Sensing Function may be split into Sensing Control Function and Sensing Processing Function. The Sensing Processing Function is responsible for receiving the Sensing Data and for calculating the Sensing Result. Therefore, the control signalling and sensing data will go though different paths. The Sensing Entity may trigger the establishment of the path for 3GPP sensing data reporting once receiving sensing service request message from the Sensing Function.

Solution#5 proposes to enhance the N1/N2 interface to support sensing data transmission. The Sensing Data is transmitted to the SF via the AMF, which processes the data to derive the Sensing Result. The SF may invoke the Namf_Sensing_Subscribe towards AMF for event reporting and the sensing-related parameters. After AMF completes the sensing entity selection, the AMF may transmit the Sensing Subscribe request message, which includes the sensing configuration parameters, to the selected sensing entity/entities. After the sensing entities finish data measurement, the sensing data is transmitted to the AMF. Then the AMF forwards the sensing data to the SF.

Solution#18 proposes to transfer sensing data and associated information between the Sensing Entity and the Sensing Function via data connection. The data connection establishment procedure is carried out on control plane signalling interaction that provides connection information (e.g. IP address) for data connection and binding information (e.g. Sensing Entity ID, connection ID or specific binding ID) from Sensing function to the Sensing Entity. After the Sensing Entity receives the connection information, the Sensing Entity establish data connection with the Sensing Function according to the address of the Sensing Function. If the sensing entity is a gNB, the sensing entity establishes direct data connection with the Sensing Function. If Sensing Function or Sensing Entity determines to utilize the established data connection, messages are transferred between Sensing Entity and Sensing Function for sensing data and the associated information transport via the data connection.

Solution#21 proposes that the SF sends DL Service Message Transfer to the selected sensing entities, NG-RAN, with correlation ID. NG-RAN sends UL Service Message Transfer to SF that includes correlation ID and sensing response.

Solution#22 proposes to establish a direct data tunnel between the Sensing Entity (i.e. gNB) and the Sensing Function via the sensing control signalling interaction by exchange tunnel information (e.g. TEID (Tunnel Endpoint ID) and/or IP address) between sensing entity and sensing function. The direct data tunnel will used for sensing data and the associated information (e.g. Sensing Service ID) transmission.

Solution#23 proposes to derive the sensing results by processing sensing data in a distributed approach in the scenario of multiple sensing functions work in jointly.

Solution#24 proposes to establish data connection between the SE(s) and SDCF for sensing data transfer using the SDCF endpoint information indicated by SSMF in configuration instructions via AMF. The sensing data is reported to the SDCF periodically, aggregated report, or partial sensing data reporting depending on the configuration information.

Solution#25 mainly focuses on supporting UE as a sensing service consumer.

Solution#26 proposes to establish a data reporting tunnel by gNB to transfer sensing data to SPF. The SCF sends the Sensing Task Initiated Request and the corresponding parameters to selected SPF and sensing entities. Then the gNB/gNBs establish the data reporting tunnel with the SPF. After gNB performing sensing data collection, it sends sensing data to SPF via data tunnel.

Solution#34 proposes to establish connection between Sensing entity and SF using the address information received from SF (SCF) for sensing data transfer.

Solution#39 proposes to configure the gNB for sensing data collection by SF via AMF, e.g. the configuration information will be transferred from SF to AMF by invoking a new AMF service operation Namf_Sensing_Configuration. Then the AMF forwards the configuration to the selected sensing entities via enhanced NGAP or a new protocol,

# Proposal

It is proposed to adopt the following changes into TR 23.700-14.

*** Start of the change ***

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.Y	Agreed Principles for KI#4

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

-	Based on request from sensing service consumer, the sensing function (SF) determines the configuration information of the corresponding sensing service.

-	SF may send the determined configuration information in sensing data collection request the selected sensing entities (SEs) via AMF. If there is no direct data connection established between the SF and the corresponding SE(s) for sensing data transfer, the SF may also indicate its address (e.g. IP address/FQDN) to SE.

NOTE 1: sensing entity discovery and selection is addressed under KI#3.

-	A SE may collect sensing data and optional associated information based on the configuration information received from SF.

-	The SE may start to transfer the collected sensing data and optional the associated information to SF upon receiving sensing data transfer request from SF or the criteria for SE sensing data transfer are met.

-	The criteria for SE sensing data transfer maybe configured by SF (e.g. timing information for data reporting, periodic data reporting, event, etc.), or pre-configured by operators.

-	Before transferring sensing data from SE to SF, if there is no direct data connection established between SF and the corresponding SE, the SF or SE may trigger the establishment of direct data connection for sensing data transfer, e.g. using the IP address/FQDN of SF and SE.

*** End of the change ***