3GPP TSG-SA WG2 Meeting #171	S2-2508513

October 13-17, 2025 Wuhan, China

Source: 	ZTE, CMCC

Title: 	Interim conclusion for KI#1 and KI#2 of FS_5GSAT_Ph4_ARC

Document for: 	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes an interim conclusion for KI#1 and KI#2 of FS_5GSAT_Ph4_ARC regarding the evaluation of different PDN connection types and IMS optimization options.

1	Discussion

In TR 23.700-19, 12 solutions are identified to support IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC and 9 solutions are identified to enhance IMS for GEO NB-IoT NTN access, covering several different options regarding the PDN connection and IMS signaling optimization.

For PDN connection, there are two classification dimensions. The first dimension refers to the session number for IMS services: either establishing a single PDN connection for both IMS signalling and IMS voice, or establishing two separate PDN connections—one for IMS signalling and the other for IMS voice. The second dimension distinguishes between IP type PDN connections and non-IP type PDN connections.

For the number of PDN connection:

- The current IMS architecture typically uses a single PDN connection with a common APN, where signaling and voice media are transmitted over separate EPS bearers.

- If two separate PDN connections are used to support IMS signaling and voice, each session requires an independent address, session context, QoS flow, policy control procedures, and resource allocation. This significantly increases the state maintenance burden on both the network side (e.g., P-GW, PCRF) and the UE.

- When signaling and media are associated with different IP addresses, the network must handle the mapping between them, which may cause some issues. For example, during call setup procedure, if the signaling and media IP addresses are not aligned, the session binding maybe failed.

Proposal 1: A single PDN connection shall be used for both IMS signalling and IMS voice.

Proposal 2: Default bearer shall be used for IMS signalling, dedicated bearer shall be used for IMS voice.

For the type of PDN connection:

- IMS services rely on the SIP/UDP/IP protocol for signaling exchange, while media streams are typically transported over RTP/UDP/IP. Both signaling and media require an IP address to establish an end-to-end transport path.

- If the PDN connection type is non-IP, the IMS client in the UE will be significantly impacted due to the absence of a UE IP address.

Proposal 3: The PDN connection type for IMS voice call over GEO shall be IP.

Proposal 4: IMS signaling and voice packets transported inside the IMS Data Network shall include an IP/UDP header.

For IMS signalling, there are two main dimensions of classification: IMS signalling over IP and IMS signalling over non-IP, text-based simplification and binary-based simplification. These two dimensions are orthogonal and independent of other WG.

For the IMS signalling over IP and IMS signalling over non-IP:

- The size of the IP/UDP header is 28–48 bytes. Under a 2 kbps transmission rate, a single signaling message only incurs an additional delay of 0.112s or 0.192s. With simplified procedures, using non-IP packet would at most save 0.224s to 0.384s, which is negligible. However, it introduces significant system complexity.

- According to TS 23.401, NIDD is a data transmission method with PDN Type "Non-IP" and UE is agnostic of the IP address, making it unsuitable for IMS over GEO. Additionally, only UDP/IP encapsulation/decapsulation at the P-GW is standardized, which does not support IP/UDP header insertion and removal at the UE, thus necessitating additional modifications.

- For IP-type PDN connections, the removal or insertion of specific fields should be handled by both the UE and P-GW, which would require modifications to the modem/application on the UE side and to the user plane processing module/protocol stack on the P-GW side.

- The default bearer established for IMS signalling may also be used to convey other types of data, such as DNS. As a result, the UE and P-GW must implement additional per-packet detection mechanisms to handle packets with different formats.

Proposal 5: The NIDD defined in TS 23.401 is not preferred since the UE lacks IP address, which significantly affects IMS client.

Proposal 6: The IMS signalling transport between UE and P-GW shall include an IP/UDP header.

For text-based simplification and binary-based simplification:

- Based on S2-2508234, the 𝐸𝐶𝑆𝑇𝑠𝑎𝑡𝑇𝑜𝑇𝑒𝑟𝑟, which considers state transition, transmission delay and propagation delay for IMS and EPS, is quite similar between text-based simplification (11.5–15s) and binary-based simplification (~14s). This indicates that from a latency perspective the performance gain of binary-based simplification is limited in GEO NB-IoT NTN scenarios.

- Binary-based simplification is designed to support limited IMS service. To support expanded services such as call-hold, multi-call, or additional services, the encoding needs continuous expansion. This approach has poor scalability and severely restricts service capabilities and flexibility.

- Binary-based simplification has significant implications for CT1, requiring the design of a new encoding scheme specifically for IMS over GEO. This poses a risk of not meeting the R20 timeline.

Proposal 7: The text-based simplification shall be adopted for IMS optimization.

Proposal 8: Whether to adopt binary-based simplification requires CT1's confirmation.

2 Proposal

It is proposed to agree and update TR 23.700-19 on FS_5GSAT_Ph4_ARC as follows.

# * * * * First Change* * *

7	Interim Agreements

7.1	Agreed Principles

7.1.X	Agreed principles for KI#1 on support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC

Principle 1: A single PDN connection shall be used for both IMS signalling and IMS voice.

Principle 2: Default bearer shall be used for IMS signalling, dedicated bearer shall be used for IMS voice.

Principle 3: The PDN connection type for IMS voice call over GEO shall be IP.

Principle 4: IMS signaling and voice packets transported inside the IMS Data Network shall include an IP/UDP header.

Principle 5: The NIDD defined in TS 23.401 is not preferred since the UE lacks IP address, which significantly affects IMS client.

Principle 6: The IMS signalling transport between UE and P-GW shall include an IP/UDP header.

7.1.Y	Agreed principles for KI#2 on IMS enhancement for GEO NB-IoT NTN access

Principle 1: The text-based simplification shall be adopted for IMS optimization.

Principle 2: Whether to adopt binary-based simplification requires CT1's confirmation.

# * * * * End of change * * * *