3GPP TSG-SA WG2 Meeting #171	S2-2508586

Wuhan, China, 13 – 17 October 2025

Source:	ETRI

Title:	Solution 6.4 Update: Multiple SeMF Support

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: The proposal enhances the discovery procedure and selection efficiency in multi-coverage, multi-domain sensing environments by introducing a hierarchical/multi-SeMF structure.

# 1. Discussion

The solution #4 addresses a localized sensing management architecture per sensing coverage with multiple SeMF support but details are left for further study. This contribution proposes to update solution 6.4 of TR 23.700-14 by adding support for multiple SeMF deployment, addressing KI#1 (architecture) and KI#3 (discovery).

In the proposed solution, a Main SeMF can discover, select, and coordinate one or more Sub SeMFs to extend sensing coverage and capabilities across multiple domains. This enables more flexible and scalable handling of sensing service requests, especially when a single SeMF cannot cover the required sensing target area or capabilities.

Example scenarios where multiple SeMF support is used include:

- Multiple Sub SeMFs perform localized sensing within their respective coverage areas or domains, where each Sub SeMF manages sensing entities in its dedicated area or domain.

- A Main SeMF coordinates multiple Sub SeMFs to initiate sensing operations and aggregates sensing data or results from the Sub SeMFs to produce a final sensing result.

- During an ongoing sensing operation, the Main SeMF may dynamically re-select Sub SeMFs (e.g., replacing or adding Sub SeMFs) to adapt to target sensing area/coverage changes due to sensing target mobility or to meet sensing quality requirements.

Key benefits include:

- Improved discovery and selection of Sub SeMFs based on their coverage, domain, and capabilities to meet diverse service requirements.

- Enhanced scalability by distributing sensing operations across multiple Sub SeMFs while maintaining centralized coordination through the Main SeMF.

- Flexible aggregation of sensing data or results from multiple Sub SeMFs under a unified policy determined by the Main SeMF.

- Support for multi-coverage and multi-domain operations while minimizing changes to the existing system architecture and interfaces.

# 2. Proposal

It is proposed to add the following text to TR 23.700-14.

* * * Start of Change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 | #6 |
| #1 | X |  | X | X | X |  |
| #2 |  | X |  |  | X |  |
| #3 | X | X | X | X | X | X |
| #4 | X |  | X |  |  |  |
| #5 | X |  |  | X |  |  |
| #6 | X | X | X |  |  |  |
| #7 | X |  |  |  |  |  |
| #8 |  | X |  |  |  |  |
| #9 |  | X |  |  |  |  |
| #10 |  | X |  |  |  |  |
| #11 |  | X |  |  |  |  |
| #12 |  | X |  |  |  |  |
| #13 |  |  | X |  |  |  |
| #14 |  |  | X |  |  |  |
| #15 |  |  | X |  |  | X |
| #16 |  |  | X |  |  |  |
| #17 |  |  | X |  |  |  |
| #18 |  |  |  | X |  |  |
| #19 | X | X |  |  | X |  |
| #20 |  |  |  |  |  | X |

* * * Next Change * * * *

6.4	Solution #4: Localized Sensing Management Architecture per Sensing Coverage

6.4.0	High-level solution Principles

The solution is based on the following general principles to support sensing:

-	The Sensing Management Function (SeMF) is newly introduced to support core sensing capabilities including sensing entity management and sensing data management,

-	The SeMF may be distributed to provide the sensing capabilities using the sensing entities within a specific sensing coverage,

-	The NEF manages sensing services requested by 3rd-parties (e.g. AFs) to support sensing service provisioning and sensing result exposure,

-	Multiple SeMFs may operate in a hierarchical structure to coordinate sensing operations across different coverage areas or domains.

It addresses the Key Issue #1 and #3. Any further details are covered in the other Key Issues.

6.4.1	Description

6.4.1.1	General

This solution addresses Key Issue #1 and #3 by proposing a distributed sensing management architecture per sensing coverage for supporting sensing capabilities within the 5G System. The architecture introduces a new Network Function (NF): the Sensing Management Function (SeMF) which is responsible for managing sensing entities and sensing data within a specific sensing coverage; and leveraging NEF functionalities for sensing service provisioning. The SeMF is responsible for the core functionalities related to managing sensing resources and handling sensing data. Its key roles include:

-	Sensing Entity Management:

This involves the registration of sensing-capable entities (gNBs), their discovery and selection for specific requests (as per KI#3), and their configuration (parameter provisioning, as per KI#6). Sensing entities register their capabilities including coverage, role (e.g. transmitter, receiver, or both), sensing-related performance parameters (e.g. range, resolution, accuracy, latency), resource information (e.g. energy consumption), and features (e.g. pre-processing).

-	Sensing Data Management:

This covers the collection of sensing data from sensing entities (e.g. via N1/N2 or N3), processing the collected data to generate sensing results. It also includes storing the sensing data and/or result for purposes such as enabling analytics for predictions, historical analysis, or re-use for sensing with static objects minimizing resource consumption (as per KI#4).

The NEF manages sensing services requested by 3rd-parties (e.g. AFs). Its primary roles are related to provisioning and exposing sensing results. Its key functionalities include:

-	Sensing Service Provisioning:

This involves authorizing sensing service requests (as per KI#2), enforcing policies and service-specific requirements, discovering and selecting the appropriate SeMF(s) to handle a request (e.g. via NRF, using sensing coverage), and triggering the sensing request to the selected SeMF(s) (as per KI#3). It also includes mapping between sensing target areas (e.g. geographical area) specified in AF requests and network-internal identifiers

-	Sensing Result Exposure:

This involves receiving sensing results from SeMF(s) and exposing them to the requesting entity (e.g. AF as per KI#5). The content of exposed results may include, for example, object type, location, velocity, direction, etc. Results may be provided as one-time reports, periodically, or based on events.

6.4.1.2	Multiple SeMF Support

This subclause specifies support for hierarchical deployment of multiple SeMFs to enhance discovery, (re-)selection, and coordination across multiple sensing coverages or domains.

An SeMF may coordinate with other SeMFs in different roles depending on the relationship:

- A Main SeMF receives sensing service requests from NEF (or trusted AF) and coordinates with one or more Sub SeMFs whose sensing coverage corresponds to subsets of the required sensing coverage area.

- A Sub SeMF manages sensing entities within its dedicated coverage or domain and provides sensing data or results to the Main SeMF that coordinates it.

- A Sub SeMF may itself coordinate other Sub SeMFs, acting as a Main SeMF in that relationship.

Key functional aspects include:

- Discovery and Selection of Sub SeMFs:

A Main SeMF identifies that the requested sensing target area requires multiple Sub SeMFs based on their coverage capabilities. The Main SeMF discovers candidate Sub SeMFs via NRF using NF profile attributes such as coverage, domain ID, capabilities, and load, and selects Sub SeMFs whose coverage collectively matches the required area while fulfilling service requirements and policy constraints.

- Coordination between Main SeMF and Sub SeMFs:

The Main SeMF sends sensing requests to selected Sub SeMFs with sensing-related parameters (e.g., sensing type, target area, required performance), while adapting them to each Sub SeMF’s assigned coverage and capabilities. The Main SeMF may also provide an aggregation policy, which defines both how each Sub SeMF should process and report sensing data (e.g., raw data, pre-processed results, aggregated results), and the quality targets to be met (e.g., accuracy, latency, confidence thresholds).

- Data and Result Handling from Sub SeMFs:

Each Sub SeMF processes sensing data according to the aggregation policy specified by the Main SeMF. The Sub SeMF may perform optional pre-processing operations (e.g., filtering, format conversion, initial analysis) before reporting to the Main SeMF. The Main SeMF then combines the sensing data or results received from multiple Sub SeMFs to produce the final sensing result. Different aggregation strategies may be applied depending on service requirements, such as full integration of all Sub results for comprehensive coverage, selective integration based on domain or quality criteria, or minimal processing for low-latency delivery.

- Dynamic Re-selection of Sub SeMFs:

The Main SeMF may perform Sub SeMF re-selection during an ongoing sensing operation to maintain service quality and coverage. Re-selection may be triggered by various conditions including coverage changes (e.g., sensing target mobility), quality degradation (e.g., accuracy or latency falling below thresholds), load conditions (e.g., Sub SeMF overload), or failure scenarios (e.g., Sub SeMF unavailability). The re-selection procedure involves discovering new candidate Sub SeMFs via NRF, selecting appropriate Sub SeMFs based on current requirements (coverage, load, capabilities), and initiating sensing operations with the selected new Sub SeMFs. The detailed re-selection procedure is described in clause 6.4.2.3.

To support Sub SeMF discovery and selection, SeMF NF profile registered to NRF should include the following additional information:

- Sensing coverage: geographical area, cell ID(s), or tracking area(s) that the SeMF can manage

- Domain identifier: to distinguish administrative or operational domains (e.g., operator ID, security domain)

- Load indicator: current load status or available capacity

- Supported capabilities: sensing modes, data processing capabilities (e.g., aggregation, fusion, pre-processing)

NOTE: The details of NF profile parameters are covered in KI#3.

6.4.2	Procedures

6.4.2.1	General

Figure 6.4.2-1: Overall procedure for handling sensing service request

A high-level procedure for a sensing service request may involve the following steps:

0.	(Assumption) Sensing entities are already registered to SeMF with their sensing capabilities, which may be covered in KI#3.

Editor's note:		The details of sensing entity registration procedure is FFS.

1.	An Application Function (AF) initiates a sensing service request towards the 5G System via NEF. The request includes details such as the sensing type (e.g. object detection, tracking, environment monitoring, etc.), target (e.g. object, area), required service-specific performance (e.g. accuracy, resolution, latency) and potentially assistance information (e.g. object/area details).

2.	The NEF performs authorization based on the request details and sensing service profile information (e.g. interacting with UDR).

Editor's note:	The use of UDR for sensing service authorization is FFS.

3.	Based on the target area/object and required capabilities specified in the request, the NEF discovers and selects one or more suitable SeMFs. This involves mapping the sensing target area (if specified) to sensing coverage and querying the NRF for SeMF(s) matching the sensing coverage and required sensing capabilities.

4.	The NEF triggers the sensing request towards the selected SeMF(s), including the relevant sensing target information and assistance information derived from the AF request.

5.	Upon receiving the sensing request, the SeMF discovers and selects the specific sensing entities (gNBs) under its management that are capable of fulfilling the request for the specified target area/object and service-specific performance.

6.	The SeMF configures the selected sensing entities with necessary parameters for the sensing request.

7.	The SeMF requests and collects sensing data from the configured sensing entities. Transport for the sensing data collection may occur over CP (N1/N2) or UP (N3).

8.	The SeMF processes the collected sensing data to generate the sensing results. The collected sensing data and/or sensing results may be stored by the SeMF for further use.

9.	The SeMF reports the sensing results, potentially with contextual information, back to the requesting NEF.

10.	The NEF receives sensing results from the SeMF and exposes the sensing result to the requesting AF.

6.4.2.2	Procedures for Multiple SeMF Support

This subclause describes the procedure for supporting hierarchical deployment of multiple SeMFs, as introduced in clause 6.4.1.2. The procedure extends the general procedure in clause 6.4.2.1 by adding the discovery, selection, and coordination steps between a Main SeMF and one or more Sub SeMFs.

Figure 6.4.2-2: Procedure for hierarchical SeMF operations

The procedure for a sensing service request with hierarchical SeMF support may include the following steps:

- The AF sends a sensing service request to the NEF, including parameters such as sensing target area, required capabilities, and reporting conditions. (same as step 1 in clause 6.4.2.1)

- The NEF authorizes the request, discovers and selects the appropriate Main SeMF, and triggers the sensing request to the selected Main SeMF. (same as step 2-4 in clause 6.4.2.1)

- The Main SeMF queries the NRF to discover candidate Sub SeMFs that match the service requirements, using NF profile attributes such as sensing coverage, domains, capabilities, load, etc.

- The Main SeMF selects appropriate Sub SeMFs whose coverage collectively matches the required sensing target area and sends sensing requests to the selected Sub SeMFs with adapted sensing-related parameters (e.g., assigned coverage area, aggregation policy, reporting configuration) and an aggregation policy.

- Each Sub SeMF performs localized discovery and configuration of sensing entities in its coverage and initiates sensing measurements. (same as step 5-7 in clause 6.4.2.1)

- The Sub SeMF may optionally pre-process the collected sensing data before reporting to the Main SeMF, in accordance with the Main SeMF’s aggregation policy.

- The Sub SeMF sends either raw sensing data or pre-processed results to the Main SeMF as instructed.

- The Main SeMF aggregates and/or processes the sensing data or results collected from multiple Sub SeMFs to produce the final sensing result.

- The Main SeMF provides the final sensing result to the NEF. (same as step 9 in clause 6.4.2.1)

- The NEF exposes the final sensing result to the AF, following the requested reporting mode. (same as step 10 in clause 6.4.2.1)

6.4.2.3	Procedures for SeMF Re-selection

This subclause describes the procedure for Sub SeMF re-selection during an ongoing sensing operation. Re-selection allows the Main SeMF to adapt to changing conditions such as coverage changes, quality degradation, load conditions, or failure scenarios, as described in clause 6.4.1.2.

Re-selection may be triggered by one or more of the following conditions:

- Coverage change: the sensing target moves outside the current Sub SeMF's coverage or enters a different Sub SeMF's coverage area

- Quality degradation: sensing result quality (e.g., accuracy, latency) falls below required thresholds

- Load balancing: the current Sub SeMF becomes overloaded or another Sub SeMF has better availability

- Failure scenario: the current Sub SeMF becomes unavailable

Figure 6.4.2-3: Procedure for SeMF Re-selection operations

The re-selection procedure may include the following steps:

- The sensing operation between the Main SeMF and the old Sub SeMF(s) is ongoing as configured according to the procedure defined in clause 6.4.2.2.

- The Main SeMF detects a re-selection trigger condition. The trigger may be based on: sensing result quality monitoring (e.g., accuracy or latency degradation), Sub SeMF status reports (e.g., load increase, capability change), sensing target mobility information (e.g., target moving out of current Sub SeMF coverage), or failure indication (e.g., Sub SeMF becomes unavailable).

- The Main SeMF performs SeMF discovery by querying the NRF with NF profile attributes such as sensing coverage, domain ID, capabilities, and load. (same as step 3 in clause 6.4.2.2)

- The Main SeMF selects a new Sub SeMF based on its coverage that matches the current or updated sensing target area, its capabilities that meet the service requirements, and its load status. The Main SeMF sends a sensing request to the selected new Sub SeMF with adapted sensing-related parameters and an aggregation policy. (same as step 4 in clause 6.4.2.2)

- The new Sub SeMF performs localized discovery and configuration of sensing entities in its coverage and initiates sensing measurements. (same as step 5 in clause 6.4.2.2)

- The Main SeMF determines that the new Sub SeMF has successfully initiated sensing operations and decides to terminate the sensing operation with the old Sub SeMF. The Main SeMF instructs the old Sub SeMF to stop sensing measurements and release associated resources.

- The new Sub SeMF may optionally pre-process the collected sensing data according to the aggregation policy. (same as step 6 in clause 6.4.2.2)

- The new Sub SeMF sends sensing data or result reports to the Main SeMF. (same as step 7 in clause 6.4.2.2)

- The Main SeMF aggregates and/or processes sensing data or results from the new Sub SeMF to produce the final sensing result. (same as step 8 in clause 6.4.2.2)

NOTE: The Main SeMF may continue receiving sensing result reports from both old and new Sub SeMFs during a transition period before instructing the old Sub SeMF to terminate. The timing of the termination may depend on service continuity requirements or implementation-specific.

6.4.3	Impacts on Services, Entities and Interfaces

The following NFs and interfaces are impacted:

SeMF:

-	a new network function for sensing entity management and sensing data management under its sensing coverage.

-	new services and interfaces for sensing entity registration, sensing data collection, and sensing request.

-	support for hierarchical operation as Main SeMF or Sub SeMF, where Main SeMF coordinates Sub SeMFs and aggregates sensing data or results from them.

NEF:

-	enhancement for sensing service provisioning and sensing result exposure.

-	new services and interfaces for sensing service request (including subscription) from AF.

NRF:

-	updates to NF profile management for discovery of SeMF.

* * * End of Change * * * *