3GPP TSG-SA2 Meeting #171	S2-2508622

# Wuhan, China, 13th – 17th October, 2025										   (revision of S2-2507624)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1574 | rev | 4 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for PCF involvement in ECN marking for L4S |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM, TEI19 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As it is now described in the Rel-19 version of 23.501 how the SMF can dynamically eatablish a QoS Flow enabled with ECN marking for L4S based on detection of L4S traffic, the misleading statement about the PCF involvement in this functionality should be clarified. The PCF is not aware of the detection of L4S traffic when the SMF based configuration option is used and does not control or influence the QoS Flow establishment or enabling of ECN marking. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The statement on PCF decision making in case of detection of L4S traffic is clarified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The statement regarding PCF decision making in case of detection of L4S traffic is misleading wrt. the PCF involvement when the SMF based configuration option is used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.27.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | There is no stage 3 impact from this CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First changes * * * *

##### 6.1.3.27.1	Exposure of network information

For support of real-time media codec/traffic adaptation to the network conditions, the AF may subscribe for exposure of 5GS network information.

When the AF requests the network to provide QoS for the AF session (as described in clause 6.1.3.22), the AF may also provide the subscription to the following information based on QoS monitoring (as described in clause 5.37.4 of TS 23.501 [2]): the UL and/or DL congestion information, UL and/or DL packet delay, the round-trip delay over one or two service data flows, the UL and/or DL Available Bitrate for the service data flow, the UL and/or DL data rate for the service data flow as described in clause 6.1.3.21. The PCF may generatee QoS Monitoring policies for the above network information in the PCC rule of the service data flow as described in clause 6.1.3.21.

If the AF provides an explicit indication (i.e. Indication of ECN marking for L4S support) that the UL and/or DL of the service data flow supports ECN marking for L4S or the PCF decides, based on local configuration, that the service data flow supports ECN marking for L4S, then the PCF may explicitly, or implicitly (based on PCF/SMF local configuration), indicate to the SMF to enable for ECN marking for L4S. The PCF decision may also be taken based on local configuration in PCF and L4S traffic detection result.

NOTE 1:	The PCF can provide a PCC rule with an application identifier for detection of L4S traffic (see clause 5.37.3.1 of TS 23.501 [2] for the detection criteria) on the QoS Flow associated with the default QoS rule in order to be notified about the start of an IP flow using L4S. Upon notification with the deduced service data flow description (i.e. IP-5-tuple), the PCF can then generate a PCC rule with a service data flow description for that L4S traffic and enable ECN marking for L4S in this PCC rule.

NOTE 2:	The option of local configuration in SMF for L4S traffic dection based establishment of a QoS Flow with enabled ECN marking for L4S is described in clause 5.37.3.1 of TS 23.501 [2].

The PCF shall ensure that only one of QoS monitoring for UL and/or DL congestion information or ECN marking for L4S is activated for the same service data flow (due to the limitation described in clause 5.45.3 of TS 23.501 [2]).

For subscription of UL and/or DL Available Bitrate of a GBR service data flow, the AF also provides a list of thresholds per direction, containing the bitrate values of interest to the AF including one value equal to the Requested Guaranteed Bitrate and one value equal to the Requested Maximum Bitrate (in case it is different). The list of thresholds per direction is to be provided with an increasing order of bitrate values. The PCF then includes the list(s) of thresholds in the QoS Monitoring policy of the corresponding PCC rule (see also clause 5.45.6 of TS 23.501 [2]).

The AF may also provide the value for Averaging Window using AF session with required QoS as described in clause 6.1.3.22.

The AF may subscribe the Data Rate Limitation Information report for a Non-GBR service data flow as described in clauses 6.1.3.18 and 6.1.3.22 (see also clause 5.37.4 of TS 23.501 [2]). The PCF reports the uplink and downlink maximum bitrate authorized for the service data flow to the AF and any change of it.

The AF may subscribe the Data Rate Limitation Information report for a PDU Session as described in clause 6.1.3.18 (see also clause 5.37.4 of TS 23.501 [2]). The PCF reports the uplink and downlink Authorized Session-AMBR to the AF and any change of it by means of the Npcf_EventExposure service.

# * * * * End of changes * * * *