3GPP TSG-SA WG2 Meeting #171	S2-2508660

Wuhan, China, October 13 - 17, 2025	(revision of S2-250xxxx)

Source:	Ericsson

Title:	Interim Agreements for KI#1

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: The contribution discusses and proposes interim agreements for KI#1.

1. Introduction

In TR 23.700-13, there were comprehensive discussions on the support of Topology 2. And there were some interim conclusions reached with unsolved issues in Editor’s Notes. With the decision of adopting RRC-based solution, some of the ENs can be resolved, while some requires further investigation.

In TR 23.700-30, KI#1 for the support of topology 2 was agreed as below:

## 5.1	Key Issue #1: support AIoT services under the RRC-based option for UE Reader connectivity

NOTE 1:	Only CM-CONNECTED with RRC_CONNECTED UE Reader is in the scope.

Only the following aspects will be included:

-	How to perform and revoke authorization of UE Readers.

NOTE 2:	Rel-19 interim conclusion in TR 23.700-13 [7] should be used as basis for UE Reader authorization.

-	Potential impact from how the NG-RAN node and UE reader selection can be performed.

NOTE 3:	The aspect of NG-RAN node and UE reader selection requires coordination with RAN WG3.

-	Whether and how the AIOTF can provide the information of the UE readers to the NG-RAN node to assist NG-RAN for UE reader selection.

NOTE 4:	Rel-19 interim conclusion in TR 23.700-13 [7] should be used as basis for this key issue. No Rel-19 AIoT Device impact from this key issue is expected.

NOTE 5:	Conclusions of this KI will be coordinated with RAN WGs.

[Proposal-1] The interim agreements for KI#1 can be based on interim conclusions in TR 23.700-13.

[Proposal-2] The interim agreements for KI#1 should consider the ENs in the TR 23.700-13 and the agreed KI in TR 23.700-30.

System Architecture

System architecture to support topology 2 hasn’t been included in the interim conclusions in Rel-19. But it is not a controversial topic.

Based on our understanding on the RRC-based option of Topology 2, using Intermediate UE as the UE reader, UE and UDM need to be included. For the other aspects of the system architecture, it is similar as the indirect connectivity of topology 1. The following figure depicts the system architecture. The new entities for topology 2 are in blue.

[Proposal-3] It is proposed to agree on the system architecture figure above in interim agreements.

Also, based on our understanding, the following figure depicts the system architecture reference point representation.  The new entities and the reference points for topology 2 are in blue.

[Proposal-4] It is proposed to agree on the reference point architecture figure above in interim agreements.

Protocol stack aspects

In clause 8.1.3.3 of TS 23.700-13, the protocol aspects of RRC-based option are included:

#### 8.1.3.3	RRC option

The following principles apply:

-	Messages between the UE Reader and the AIoTF are delivered using RRC between UE and RRC and NGAP between gNB and AMF, and using an SBI interface between AMF and AIoTF. The related protocol stack is shown in Figure 8.1.3.3-1.

Editor's note:	How addressing works for UL for Option B is FFS.

Editor's note:	Further details how the RRC option works are FFS.

Figure 8.1.3.3-1: Protocol Stack for the RRC option

In our understanding, the current protocol stack from the TR is no longer accurate, due to the fact that AIoT Reader Control layer have been removed. Instead, “NGAP AIoT Information” is defined between the AIOTF and the NG-RAN, but not as a separate protocol layer. The following figure shows the updated protocol stack.

[Proposal-5.1] It is proposed to agree on principle and the protocol stack figure above in interim agreements.

Regarding the EN of “How addressing works for UL for Option B is FFS”. Using the AIOTF ID and Correlation ID, in indirect connectivity of topology 1, NG-RAN can route UL traffic to the proper AIOTF via the proper AMF. In topology 2, the same principle applies. In addition, the UE traffic can be delivered to the NG-RAN node via RRC. Such aspect is going to be addressed by RAN2. As a conclusion, we do not foresee the issues in the UL message routing.

[Proposal-5.2] It is proposed to resolve EN by not including it in the interim agreements.

Regarding the EN of “Further details how the RRC option works are FFS”. This EN is very generic and we cannot take specific actions on this EN.

[Proposal-5.3] It is proposed to resolve EN without including it in the interim agreements. Concrete ENs can be included when the issues have been identified during the discussion.

Subscription and Authorization aspects

In clause 8.1.3.1 of TS 23.700-13, the subscription aspects have been agreed as below:

-	Subscription aspects:

-	The UE subscription in the UDM will be extended with UE Reader subscription information, which consists of the following:

-	information indicating whether the UE is allowed to operate as a UE Reader.

Editor’s note:	Additional subscription information, e.g. validity information, for the UE Reader is FFS.

-	UE Reader subscription information is available to AMF and AIoTF.

-	If AMF receives, as part of the subscription information, the indication that the UE is authorized to operate as a UE Reader, then AMF informs NG-RAN that the UE is authorized to operate as a UE Reader.

Editor’s note:	Whether and how to enable authorization to the UE is FFS.

In clause 8.1.3.4 of TS 23.700-13, the interim conclusion for authorization has been agreed as below:

The following interim conclusion are agreed for the UE Reader authorization in Topology 2:

-	The subscription data of UE Reader includes the indication that whether the UE is allowed to act as an AIoT Reader.

-	The AMF provides the UE Reader authorization indication to the NG-RAN serving the UE Reader.

Editor's note:	How to authorize the UE Reader will be determined later.

Clause 8.1.3.1 covers the contents in clause 8.1.3.4, so we can remove the duplicated informationin the interim agreements.

[Proposal-6.1] It is proposed to agree on principles on subscription aspects and authorizaiton aspects above in interim agreements.

Regarding the EN of “Additional subscription information, e.g., validity information, for the UE Reader is FFS”, we don’t have a strong view. More discussion is needed. It could be beneficial to have the vaildity period information and/or validity area information to enable NG-RAN to force the authorization in a better granularity.

[Proposal-6.2] It is proposed to keep the EN in the interim agreements to allow more discussion.

Regarding the EN of “Whether and how to enable authorization to the UE is FFS”, the discussion was about whether the UE needs to know its authorization result. In RRC-based option, gNB is the entity to control UE reader. If the UE reader is not authorized, the gNB will not select the UE, allocate radio resources and send AIoT service operation information. From UE side, there wouldn’t make differences no matter whether UE knows its authorization result or not.

[Proposal-6.3] It is proposed to resolve the EN by not including it in the interim agreements.

In the KI#1 description, there is one apsect newly raised:

-	How to perform and revoke authorization of UE Readers.

In our view, as the subscription data stored in the UDM are extended for the indication on whether the UE is allowed to act as an AIoT Reader, it makes sense to allow such indication to be updated dynamically (to grant or revoke the authorization). The UDM can notify the AMF about such change, and then let the AMF inform the NG-RAN, so that the NG-RAN can take proper actions to allow/disallow the UE to work as an UE Reader.

[Proposal-6.4] It is proposed to agree the principle above in the interim agreements.

Radio Resource Allocation

In clause 8.1.3.1 of TR 23.700-13, there are some interim conclusions on radio resource allocation:

-	Radio resource management for UE Reader operation:

-	If the gNB has received the indication that a UE is authorized to operate as a UE Reader, then the gNB may assign radio resources to the UE for UE Reader operation.

-	The UE Reader is assumed to request radio resources for Reader operation (e.g. taking into account assistance information received from the AIoTF) from the gNB.

NOTE 1:	Assistance information is further clarified in other clauses.

NOTE 2:	The details of UE Reader radio resource management will be defined by RAN WG2.

The radio resouce allocation to UE Readers is a pure RAN2 topic. SA2 only needs to provide the UE authorization information and AIoT service operation request to the gNB. It is up to RAN2 to determine when and how the radio resource is allocated to the UE reader. There are no needs for SA2 to document the principles in interim agreements.

[Proposal-7] It is proposed not to document principle on radio resource allocation in the interim agreements.

UE Reader Selection

Regarding UE Reader Selection, it is a controversial topic. RAN3 will study UE Reader Selection as part of objective in RAN WID. The UE Reader Selection has been removed from SA2 WID. SA2 also acknowledged the potential impact from RAN3 by including “potential impact from how the NG-RAN node and UE reader selection can be performed” in the KI description.

In our view, the issues for UE Reader Selection in the AIOTF is to understand how to associate the UEs with its AIoT areas, so that the AIOTF can select UEs based on AIoT area. So far, the CN is only aware of the UE with its serving cell and/or TA, but not AIoT area.

Before coming into the solution proposal, our suggestion is to check the use cases we are going to support to understand whether the issues are valid or not.

Use Cases

Regarding the support of Topology 2, there are two typical use cases to be supported: fixed UE reader deployment and mobile UE reader case.

Fixed UE Reader Deployment

In this deployment, it is assumed the UE readers are deployed as fixed in the related area (e.g. in a warehouse or room) with careful planning and configuration including the position of the UEs, to ensure the full coverage of the room, like RAN readers in Rel-19. There is no mobility for those UE readers.

It is also fair to assume that the UE readers can be configured in the NG-RAN as well as the AIOTF. Similar as NG-RAN configuration information for RAN readers in Rel-19, for each UE Reader, the AIoT area(s), the serving NG-RAN node and the serving AMF can be configured.

Using Inventory Procedure as an example for the discussion:

- In step 1, the AF sends AIoT service operation request with an External Target Area information, which is the same as Rel-19.

- In step 2, the NEF translates the External Target Area information to AIoT area list, which is the same as Rel-19.

- In step 4, it is unclear whether the AIOTF needs to perform UE reader selection. Technically, if the AIOTF needs to perform UE reader selection, it is feasible based on the local configuration in the AIOTF, as RAN Readers in Rel-19.

- In step 7, the AIOTF sends the Inventory Request to the NG-RAN node which contains Requested Service Area Information. The AIOTF may only provide the AIoT area list in the Requested Service Area Information, assuming NG-RAN can perform UE Reader selection based on AIoT area list.

- In step 9, the NG-RAN node delivers the inventory request to its selected/determined UE readers and allocates radio resources between the UE readers and AIoT devices for the UE readers. Such aspects are under the remit of RAN WGs.

[Proposal-8.1] It is proposed to agree on that, for fixed UE reader deployment, if UE reader selection is performed by the AIOTF, the AIOTF selects UE readers based on local configuration.

[Proposal-8.2] It is proposed to agree on that, for fixed UE reader deployment, the AIOTF includes AIoT area list in the Requested Service Area Information in the Inventory Request.

Mobile UE Reader Case

In this case, the UEs are handheld UE. When the person holding the UE come to the intended area, the UE communicates with the AF to indicate the inventory can be started. The mobility of UE readers needs to be considered.

Still using Inventory Procedure as an example for the discussion:

- In step 1, the AF sends AIoT service operation request with external UE IDs (e.g., GPSIs), which is different from Rel-19.

- In step 2, the NEF translates the external UE IDs (e.g., GPSIs) to internal UE IDs (e.g., SUPIs), which are different from Rel-19. But how the NEF can discover and select the proper AIOTF (issue#1), as there is no AIoT area information provided for the AIOTF discovery?

- In step 4, as the UEs are determined by the AF, there are no needs for the AIOTF to perform UE reader selection.

- In step 7, the AIOTF includes the UE Reader IDs in the Requested Service Area Information, without having AIoT area list, assuming NG-RAN should use UEs in the UE reader list (or determine UEs based on the UE reader list). But what are the UE Reader IDs to be included in the Requested Service Area Information (issue#2)?

- In step 9, the NG-RAN node delivers the inventory request to UEs (or its determined UE readers) and allocates radio resources between the UE readers and AIoT devices for the UE readers. Such aspects are under the remit of RAN WGs.

[Proposal-8.3] It is proposed to agree on that, for mobile UE reader case, the AIOTF does not need to perform UE Reader selection.

[Proposal-8.4] It is proposed to agree on that, for mobile UE reader case, the AIOTF includes UE Reader list in the Requested Service Area Information in the Inventory Request, similar as RAN Reader list in Rel-19.

On Issue#1, the typical solution is to perform AIOTF discovery via NRF or via UDM.

- Option-1: To be able to know which AIOTF should be discovered/selected via NRF, the AIOTF needs to update the served UE as part of the NF profile in the NRF, and then the NEF can utilize the UE ID (e.g. SUPI) to discover the AIOTF from the NRF.

- Option-2: To be able to know which AIOTF should be discovered/selected via UDM, the AIOTF needs to update the serving AIOTF into subscription data, and then the NEF can query the UDM to get the serving AIOTF from the UDM.

Both options are workable and require the AIOTF to be invoked by the AMF to update NRF or UDM, after the UE is authorized to work as a UE reader.

[Proposal-9.1] It is proposed to agree on that the AIOTF to be invoked after the authorization in the AMF.

Usually, the NRF discovery is based on the NF profile data which are comparable static, or some information (e.g., load information) which is updated periodically. The per UE update from the AIOTF may overload the NRF, if there are quite a lot of UEs deployed. On this aspect, we slightly prefer the UDM option.

[Proposal-9.2] It is proposed to adopt the UDM option in the interim agreements.

On Issue#2, there is no common UE reader ID between the AIOTF and the NG-RAN node. The AIOTF knows SUPI, and the NG-RAN node knows RAN/CN UE NGAP ID.

It would be a simple solution if the SUPI can be sent to the NG-RAN by the AMF, so that the AIOTF and the NG-RAN can use SUPI as a common UE Reader ID. However, SUPI is a sensitive privacy information. And from security design point of view, it is better to keep it as much as possible in the core network (contrary to sending to many RAN nodes) so that the possibility of SUPI leakage is minimized.

It could be an alternative solution to let the AMF translate between SUPI to RAN/AMF UE NGAP ID. However, it does not align with the design intention of Namf_AIoT_MessageDelivery, which is expected to be a message delivery service operation like NonUEN2MessageDelivery. The AMF is expected to deliver the AIoT Message transparently. It may not be a good idea to involve AMF too much in AIoT specific logic.

Then, the option left is to introduce a new UE Reader ID which is common between the AIOTF and the NG-RAN. As proposed in [Proposal-9.1], the AIOTF is invoked by the AMF after the UE is authorized, then the AIOTF can allocate a UE Reader ID. In the response message from the AIOTF to the AMF, the AIOTF can inform the AMF this UE Reader ID. The AMF can include the UE Reader ID in the UE context information and send it to the NG-RAN.

[Proposal-9.3] It is proposed let the AIOTF allocate UE Reader ID and return to the AMF, and the AMF informs the NG-RAN as part of UE context.

After SA2 AIoT Conference Call, the following changes are made (highlighted in yellow in the proposal)

-	Include N52 (between the NEF and the UDM) in the reference point architecture.

-	In the proposal, it is proposed to remove the radio resource allocation aspects.

-	In AIOTF discovery and selection, add the fixed UE Reader deployment, which is the same as Rel-19 (i.e., query NRF to discover the AIOTF based on target area information)

-	In AIOTF discovery and selection, for mobile UE Reader case, propose to let the AMF register serving AIOTF to the UDM

-	In UE Reader ID allocation, clarify that for fixed UE reader deployment, the UE Reader ID is configured in the UE Reader configuration information. The UE Reader ID allocation needs to be performed for the mobile UE reader case.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-30:

* * * Start of Change * * * *

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.1	Agreed Principles for KI#1

Figure 7.1.1-1 depicts the AIoT System Architecture for Topology 2.

Figure 7.1.1-1: AIoT System Architecture for Topology 2

Figure 7.1.1-2 depicts the AIoT system architecture, using the reference point representation.

Figure 7.1.1-2:  AIoT System Architecture for Topology 2 in reference point representation

The following aspects and principles are considered and agreed as the interim agreements for the support of Topology 2:

-	Protocol aspects:

Messages between the UE Reader and the AIOTF are delivered using RRC between UE and NG-RAN and NGAP between NG-RAN and AMF, and using an SBI interface between AMF and AIOTF. The related protocol stack is shown in Figure 7.1.1-3.

Figure 7.1.1-3:  Protocol Stack for Topology 2

-	Subscription aspects:

-	The UE subscription in the UDM will be extended with UE Reader subscription information, which consists of the following:

-	information indicating whether the UE is allowed to operate as a UE Reader.

Editor’s note:		Additional subscription information, e.g. validity information, for the UE Reader is FFS.

-	UE Reader subscription information is available to AMF and AIOTF.

-	UE Reader subscription information in the UDM can be updated to grant or revoke the UE authorization.

-	Authorization aspects:

-	If AMF receives, as part of the subscription information, the indication that the UE is authorized to operate as a UE Reader or the authorization is revoked, then AMF informs NG-RAN that the UE is authorized to operate as a UE Reader or the revocation of the authorzaition.

-	UE Reader selection aspects:

-	For fixed UE Reader deployment, the target area information is provided by the AF. If UE Reader selection needs to be performed, the AIOTF can select UE Readers based on UE Reader configuration information. The AIOTF includes AIoT area list in the Requested Service Area Information in the Inventory Request to the NG-RAN.

Editor’s note: 	Whether UE Reader selection is needed require further discussion and the input from RAN3.

-	For mobile UE Reader case, the UE IDs are provided by the AF. The AIOTF uses those UEs without UE Reader selection. The AIOTF includes UE Reader ID list in the Requested Service Area Information in the Inventory Request to the NG-RAN.

-	AIOTF discovery aspect:

-	For fixed UE Reader deployment, the AF provides External Target Area information.

-	The NEF performs AIOTF discovery and selection as Rel-19.

-	For mobile UE Reader case, the AF provides UE IDs.

-	The NEF queries the UDM for their serving AIOTFs.

-	After successful UE Reader authorization, the AMF selects and invokes the AIOTF. The AMF registers the serving AIOTF to the UDM in UE subscription data.

-	UE Reader ID allocation aspect:

-	After the AMF invokes the AIOTF,

-	For fixed UE Reader deployment, the AIOTF gets UE Reader ID from UE Reader configuration information

-	For mobile UE Reader case, the AIOTF allocates a UE Reader ID for the UE

-	The AIOTF informs the AMF of the UE Reader ID.

-	The AMF informs the NG-RAN of the UE Reader ID.

## 7.2	Topics for further consideration

### 7.2.Z	Topics for further consideration for KI#Z

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#Z. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.Z) and can either be then marked as not pursued or postponed to a future release.

* * * End of Change * * * *