3GPP TSG-WG SA2 Meeting #171	S2-2508697

13th – 17th October 2025, Wuhan, China	(is revision of 2506923)

Source:	Lenovo, China Telecom

Title:	KI#1 New Solution: UE reader revocation

Document for:	Approval

Agenda Item: 	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: This paper proposes the solution on how to support the UE reader revocation.

1	Introduction

This paper proposes the solution to address the following two editor’s notes in the TR 23.700-13 about topology 2, and enhance the UE reader management. The existing two editor’s notes in the clause 8.1.3 (Architecture to support Topology 2) are copied below:

Editor’s note:	Additional subscription information, e.g. validity information, for the UE Reader is FFS.

Editor’s note:	Whether and how to enable authorization to the UE is FFS.

From our perspective, it is essential for the UE to be authorized as an AIoT reader first and then interact with the AIoT Devices to perform the AIoT service operation, and the authorization should be under the full control of the MNO that the UE reader is currently connected and registered with.

What’s more, the authorization is not a one time for all thing and should be associated with the validity information, which can be stored as part of the UE subscription data in the UDM/UDR. The validity information (e.g., time based, or location based) can not only be used by the AMF and the AIOTF to determine whether to authorize the UE as an AIoT reader but also can be used by the AMF and the AIOTF as one of the triggers/factors to revoke the UE reader authorization, i.e., UE will stop acting as the AIoT reader. This kind of revocation can ensure the operator has the control and flexibility over the UE reader and make full use of the resources.

2. Text Proposal

It is proposed to capture the following changes to TR 23.700-30.

# * * * * First change * * * *

## 6.X	Solution #X: Support of UE reader Revocation

### 6.X.0	High-level Solution Principles

The solution is based on the following general principles to support the UE reader revocation:

- The UE subscription data stored in the UDM is enhanced with UE reader allowed indication indicating that UE is allowed to act as AIoT Reader, and it is associated with validity information that contains the allowed time/duration, and allowed area/location for the UE as an AIoT reader.

- The AMF performs the UE reader authorization by checking the UE subscription data stored in the UDM.

- AMF can determine to revoke the UE reader authorization, based on local policies and configuration, UE reader associated validity information, and subscription data update from UDM, and sends the UE reader revocation indication to the NG-RAN (that serves the UE reader) and optionally the UE. NG-RAN will release the radio resources configured for that UE to act as an AIoT reader.

- AIOTF can also determine to revoke the UE reader authorization based on local configuration that is related to the AIoT service, or based on the request from AF.

It addresses the KI#1.

### 6.X.1	Description

This solution is aiming for the case when UE is acting as an AIoT reader and gives the procedure on how to revoke the UE authorization as an AIoT reader, which is enabled at both the AMF and AIOTF.

### 6.X.2	Procedures

#### 6.X.2.1	UE reader revocation procedure

Figure 6.X.2.1-1. UE reader revocation procedure

Step 1 to Step 2 is basically the general UE registration procedure as described in clause 4.2.2.2.2 of TS 23.502. UE sends the UE registration request towards the AMF, that further includes the reader indication capability. After receiving the UE registration request with a reader indication, the AMF will send the UE ID and use ambient IoT reader or ambient IoT as the subscription data type to check the UE subscription data related to the AIoT service, which is stored in the UDM.

Step 3: UDM sends the UE ID, UE reader allowed indication (or not), together with the associated reader validity information to the AMF. The validity information can include the validity time and validity area information. The AMF will also subscribe to the UE reader subscription update towards the UDM implicitly.

Step 4: AMF sends the authorized UE reader information to the AIOTF, that includes the UE ID, UE reader allowed/authorization indication, UE reader location and the current serving NG-RAN information of the UE reader. The selection of AIOTF at AMF can be based on AMF local configuration or based on query from NRF.

Step 5: Based on the received UE reader information from AMF, AIOTF can also subscribe to the UDM about the UE subscription data update.

Step 6: AMF sends the UE registration response to both the UE and the NG-RAN that serves the UE, that contains the UE ID, UE reader allowed/authorization indication. The UE reader allowed indication shall be sent to the NG-RAN and can be optionally sent to UE.

Step 7: Based on the UE reader allowed indication, the NG-RAN will allocate the radio resources for the UE to act as an AIoT reader.

Step 8 and step 9: NG-RAN forwards the UE reader registration response to the UE, optionally the UE reader allowed indication. The UE can in turn perform the AIoT service operation towards the AIoT Devices.

Revocation procedure

AMF determination:

Step 10-a: Based on the local policy and configuration, the validity information, or UDM notification that UE is no longer allowed as an AIoT reader, AMF determines that the UE reader authorization needs to be revoked.

Step 11-a: AMF sends the revoked UE reader information/notification to the AIOTF, that contains the UE ID and a revoke indication.

AIOTF determination:

Step 10-b: Alternatively, based on use cases like local configuration (related to SLA), or request from 3rd party AF that requests to revoke specific UE reader (s), AIOTF can also determine that UE reader authorization needs to be revoked, and sends the UE reader revocation information to the AMF in step 11-b, including the UE ID and revocation indication.

For both option a and b, AIOTF will also delete the reader information stored locally of the revoked UE.

Step 12: For both options, AMF sends the UE reader revocation indication, and the UE ID to both the NG-RAN (that serves the UE) and the UE, to notify about the UE reader revocation.

Step 13: After receiving the UE reader revocation indication, NG-RAN releases the radio resources for the UE in Step 7.

Step 14 and step 15: After UE receives the UE reader revocation indication from the NG-RAN (optionally), UE stops acting as a reader to interact with the AIoT Devices.

6.X.3	Impacts on Services, Entities and Interfaces

AMF:

- Perform the UE reader authorization

- Determine that the UE reader authorization needs to be revoked

- Send UE reader revocation indication to NG-RAN and optionally UE.

- Inform the UE reader revocation information to AIOTF.

AIOTF:

- Determine that the UE reader authorization needs to be revoked

- Send UE reader revocation indication to AMF, NG-RAN and optionally UE

- Subscribe to UDM about UE reader subscription data update

NG-RAN:

- Release the allocated resources for UE acting as the AIoT reader based on received revocation indication

- Optionally send UE reader revocation indication to UE.

# * * * * End of changes * * * *