3GPP TSG-WG SA2 Meeting #171	S2-250xxxx

Wuhan, Oct 13 – 17, 2025	(revision of S2-250xxxx)

Source:	Nokia

Title:	New solution: AIoT Reader selection

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2 / Rel-20

Abstract: a new solution for AIoT reader selection is proposed.

# 1. Introduction/Discussion

A new solution is proposed to enable 5GC support AIoT reader selection considering both topology 1 and 2.

# 2. Text Proposal

It is proposed to approve the following changes in TR 23.700-30.

FIRST CHANGE (All text is new)

## 6.X	Solution #X: AIoT Reader Selection in Coherent Deployment of Topology 1 and Topology 2

### 6.X.0	High-level solution principles

The key technical principles proposed in this solution are summarized below:

- AIoT Reader Awareness: The AIOTF maintains knowledge of available AIoT readers, including their coverage areas, type (RAN or UE), and operational status, to enable informed reader selection.

- Flexible Reader Selection Policy: The AIOTF may select an AIoT reader based on local policy considerations, such as reader stability, load, and latency requirements, ensuring that the selected reader meets the service requirements of the targeted AIoT devices.

- Radio-Aware Reader Selection: The AIOTF can leverage probing information, such as signal strength (RSRP) and signal quality (SNR), to dynamically select the reader that provides the most reliable and efficient communication with AIoT devices.

- Device-Preference-Based Reader Selection: AIoT devices may be configured with a preference for a specific reader type. When multiple readers are available, the device selects which reader to respond to, enabling optimized operation based on application-specific requirements or environmental conditions.

- Redundancy and Reliability Management: Unselected readers can be used as backup paths to enhance reliability or recover from failures, ensuring resilient AIoT communications.

- Topology Coexistence Support: Procedures support the coexistence of Topology 1 (RAN readers) and Topology 2 (UE readers) within the same target area.

### 6.X.1	Description

The introduction of Topology 2, in which a UE functions as an AIoT reader in Release 20, on top of Topology 1, where the NG-RAN serves as an AIoT reader in Release 19 would result in scenarios where a given target area may be simultaneously covered by both types of AIoT readers as depicted in the figure below. This solution addresses the selection of the most appropriate AIoT reader type when a target AIoT device is reachable via both Topology 1 and Topology 2, to avoid duplications and enhance efficiency.

### 6.X.2	Procedures

Step 0: Using existing procedures, the AIOTF would be made available with the reader information, i.e., AIoT serving area of each reader.

Step 1: The AF requests for delivery of message targeted to a specific area to the AIOTF (via NEF).

Step 2: The AIOTF determines the available readers based on the requested target area from the AF and the reader information obtained in Step 0. The AIOTF could identify that there are two types of readers available in the target location.

At this point, the AIOTF may take three different approaches:

[Option 1: Local policy at the AIOTF]

Step 3: This approach relies on local policy configured in the AIOTF. When both a RAN reader and a UE reader are discovered to cover the same target area, the AIOTF selects one based on the following considerations:

- Reader stability: A RAN reader is typically fixed and stable, offering predictable performance, whereas a UE reader can be mobile and dynamic. If stability is critical for the requested service, the RAN reader may be preferred.

- Reader availability and load: The AIOTF can assess the current load or availability of each reader and avoid assigning too many AIoT devices to a reader that is already heavily utilized.

- Latency: The AIOTF may choose the reader that minimizes latency to the AIoT device, for example, by considering the distance between the device and each reader.

Steps 4 to 6: After the reader is selected, AIoT communications between the AIOTF and the AIoT devices are carried out using the selected reader only. However, if a failure occurs or redundancy is required to enhance reliability, the unselected reader can also be utilized.

[Option 2: AIOTF selecting the reader based on the probing information]

Step 7: In this approach, the AIOTF selects the reader based on some probing information which may include the signal quality (using SNR) and signal strength (using RSRP) between the AIoT device and each reader. As better signal often leads to faster and more reliable data exchange, the AIOTF selects the reader which has better radio condition. The AIOTF sends the probing request to both RAN reader and UE reader that are available in the target area.

Step 8: The AIOTF collects the radio status of the readers which can be also reported periodically. The radio status may include e.g., signal strength and signal quality.

Step 9: The AIOTF processes the information about the probing and selects the final reader to deliver the AIoT request.

Steps 10 to 12: After the reader is selected, AIoT communications between the AIOTF and the AIoT devices are carried out using the selected reader only. However, if a failure occurs or redundancy is required to enhance reliability, the unselected reader can also be utilized.

[Option 3: Preference set at the AIoT device]

Step13: In this approach, the AIOTF does not select a single reader but instead requests all available readers to deliver the AIoT request to the devices located in the target area.

Step 14: The UE reader and RAN reader include an indication in the AIoT request sent to the AIoT device, specifying their respective reader types (i.e., UE reader or RAN reader). This information can be conveyed either through a separate parameter or embedded within the reader ID—for example, by using a fixed prefix to indicate the type of reader.

Step 15: The AIoT device determines which type of reader to respond to, based on a preference pre-configured within the device. This preference may be set by the application provider, for example, Topology 1 might be preferred in more static environments, while Topology 2 could be favored in more dynamic scenarios.

Step 16: The AIoT device responds to the AIoT request only through the reader determined in step 15, based on its configured preference, which is further forwarded to the AIOTF via the selected AIoT reader.

Step 17: The AIOTF identifies which reader is selected by the AIoT device and stores this information for use in subsequent communications.

### 6.X.3	Impacts on Services, Entities and Interfaces

AIoT Device:

  - Receives AIoT requests from multiple readers in some approaches (Option 3) and may need to select the reader based on a pre-configured preference.

AIoT Reader (NG-RAN or UE):

  - Selected readers carry the main traffic load, while unselected readers may act as backup for redundancy or failure recovery.

  - Radio conditions (signal strength/quality) may be monitored and reported to AIOTF for dynamic selection decisions.

AIOTF:

  - Responsible for managing reader selection based on local policy, probing results, or device preference.

  - Needs to maintain reader availability information, including coverage areas, load, and radio conditions.

  - Stores the selected reader information for each AIoT device for subsequent sessions.

  - Facilitates reliability and redundancy mechanisms by leveraging unselected readers when required.

END OF CHANGES