3GPP TSG-SA WG2#171	S2-2508747

Wuhan, China, 13-17 October 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated

Title:	[KI#1, New Solution] UE Reader Selection

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-2

Abstract of the contribution: Solution for UE Reader selection.

# 1.	Discussion

​This proposal addresses KI#1, and specifically the following:

-	Whether and how the AIOTF can provide the information of the UE readers to the NG-RAN node to assist NG-RAN for UE reader selection.

NOTE 4:	Rel-19 interim conclusion in TR 23.700-13 should be used as basis for this key issue. No Rel-19 AIoT Device impact from this key issue is expected.

NOTE 5:	Conclusions of this KI will be coordinated with RAN WGs.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-xxx:

# >>>>BEGINNING OF CHANGES<<<<

## 6.X	Solution #X: AIoT UE Reader Selection Procedures

### 6.X.0	High-level solution Principles

The following principles apply:

-	Upon receiving the Area Information from AF in Inventory Request, the AIOTF can select UE reader(s) for inventory within the indicated area (e.g., via interaction with AMF/LMF)

-	AIOTF can receive a list of UE Reader(s) from AF in Inventory Request (e.g., upon a worker pressing a button in handheld UE reader). Upon receiving a list of UE Reader(s) in Inventory Request, the AIOTF simply “uses” those UE reader(s) for inventory, and there is no UE reader selection in this case.​ AIOTF then can send the Inventory Request to the selected UE reader(s) (e.g., via the AMF and the gNB)​

### 6.X.1	Description

This solution addresses KI#1 on how to enable the AIOTF to provide the information of the UE readers to the NG-RAN node to assist NG-RAN for UE reader selection.

### 6.X.2	Procedures

#### 6.X.2.1	Selecting UE Reader(s) Within An Area

Figure 6.X.2.1-1: Selecting UE Reader(s) Within An Area.

- The AF sends an Inventory Request to the AIOTF providing an Area Information. The Inventory Request may also contain a Reader Type to indicate whether the AF requires to perform inventory using UE Readers of a specific type (e.g. UE Reader(s) only, gNB Reader(s) only, both UE Reader(s) and gNB Reader(s) and/or  Reader(s) supporting certain capabilities (e.g., supporting DO-DTT/DO-A, supporting devices 1/2a/2b)b​. ​

2.	Upon receiving an Inventory Request with Area Information from AF, the AIOTF requests the AMF to provide a list of UE Reader(s) within a certain area (this can be the same Area Information indicated by AF or can be translated into areas understandable by AMF e.g., in terms of tracking areas) and their location. If received from the AF, the AIOTF also provides the Reader Type.

3.	The AMF determines the list of “suitable” UE Reader(s) by selecting those UE Reader(s) in its context which are located on a cell or tracking area or registration area within the indicated area, are authorized, have UE Reader’s capabilities, and are in CM_CONNECTED state. AMF may also consider the Reader Type.

The AMF may determine UE Reader’s location upon receiving UE Location Information (ULI) from gNB or having obtained UE’s location from other sources (e.g., from LMF if UE Reader is performing LCS procedures).

4.	The AMF provides the list of “suitable” UE Reader(s) in the requested area and their location information to AIOTF.

5.	The AIOTF may down select UE reader(s) among the obtained list for performing inventory operation.

6.	The AIOTF sends an the Inventory Request along with the selected UE reader ID(s) to an AMF. If received, the AIOTF may also include the Reader Type.

7.	The AMF forwards the Inventory Request to NG-RAN.

#### 6.X.2.1	AF-Requested Reader(s) Selection

Figure 6.X.2.2-1: Selecting UE Reader(s) Based on AF Request.

1.	The AF sends an Inventory Request to the AIOTF providing a list of UE Reader(s) to be used for the Inventory Request (e.g., upon worker(s) pressing a button in handheld UE readers).

2.	Upon receiving an Inventory Request with a list of UE Reader(s) from AF, the AIOTF does not perform UE Reader selection and simply uses the UE Reader ID(s) indicated by the AF.

3.	The AIOTF sends an Inventory Request along with the selected UE reader ID(s) to an AMF.

4.	The AMF forwards the Inventory Request to NG-RAN.

### 6.X.3	Impacts on Services, Entities and Interfaces

AIOTF:

-	Support the reception of Area Information, Reader Type, and list of UE Reader ID(s) from AF.

-   Supports selecting a list of UE Readers based on the knowledge of UE Reader’s location with respect to the requested Area Information from AF.

AMF:

-	Provide location of UE reader(s) if requested by the AIOTF.

# >>>>NEXT CHANGE<<<<

# >>>>NEXT CHANGE<<<<

# >>>>END OF CHANGES<<<<