3GPP TSG-WG SA2 Meeting #171	S2-2508772

Wuhan, China, Oct 13 – 17, 2025

Source:	China Mobile

Title:	Update on KI#6 Solution 20 Configuration Parameter Provisioning for Sensing Entities

Document for:	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes an update to KI#6 on Configuration Parameter Provisioning of Sensing Entities for the FS_Sensing_ARC TR 23.700-14.

1	Discussion

There are four cases for configuration parameter provisioning towards sensing entities:

- The first case may be handled by the sensing operation server, which could belong to the operators or have an SLA with the operators.

- The second situation may be caused by multiple sensing services.

- The OAM handles the third case within the operator’s domain.

- The SCF may carry out the fourth case based on the sensing result.

2	Proposal

It is proposed to include the following changes in TR 23.700-14.

* * * * Start of Changes * * * *

## 6.20	Solution #20: Solution for Configuration of Parameters for Sensing Entities

### 6.20.1	High level principles

Different from sensing service request to trigger sensing service operation, configuration parameter provisioning aims to provision general configuration parameters towards the sensing entities, not for individual sensing service operations.

There may be four cases for the configuration of parameters for sensing entities.

- The first case may be conducted by a sensing operation server, which could belong to the operators or have an SLA with the operators.

- The second case may be triggered by multiple sensing services.

- The third case is triggered by the OAM within the operator's domain.

- The SCF may perform the fourth case based on the sensing result.

- Editor's note:	OAM initiates the configuration of parameters for sensing entities is FFS.

The high-level principles of the solution are:

1.	For the untrusted sensing operation server, it sends a configuration of parameters request to the NEF. After authentication/authorization are complete, the NEF forwards the configuration of parameters request to the SCF(s). For the trusted sensing operation server, it sends the configuration of parameters request directly to the SCF(s).

2.	The SCF determines the targeted sensing entity(s) and performs the configuration of parameters for sensing entity(s).

NOTE:	The SF (Sensing Function) splits into SCF (Sensing Control Function) and the SPF (Sensing Processing Function.

### 6.20.2	Description

This solution addresses Key Issue #6, which concerns the configuration of parameters for sensing entities.

### 6.20.3	Procedures

#### 6.20.3.1	Sensing operation server initiated Configuration of Parameters for sensing entity procedure

Figure 6.20.3.1: Configuration of Parameters procedure

1a.	The untrusted sensing operation server sends the configuration of parameters request to the NEF with the following parameters: AF ID, sensing-related parameters, external target area information, and so on.

1b.	The trusted sensing operation server sends the configuration of parameters request to the SCF with the following parameters: AF ID, sensing-related parameters, target area information, and so on.

Editor's note:	What are the provisioned configuration parameters is FFS.

NOTE:	Sensing operation server is an application function.

2a.	The NEF authenticates and authorizes the untrusted sensing operation server to allow to perform the operation.

3a.	The NEF converts the external target area information to internal target area information.

4a.	The NEF obtains the SCF based on internal target area information by NRF query or local configuration. Then the NEF forwards the request to the SCF.

5.	The SCF determines the targeted sensing entity(s) based on internal target area information.

6.	The SCF performs the sensing configuration of parameters on sensing entity(s).

7a.	The SCF notifies the NEF response to the configuration of parameters.

7b.	The SCF notifies the untrusted sensing operation server with a configuration parameter response.

8a.	The NEF forwards the configuration parameters response to the trusted sensing operation server.

#### 6.20.3.2	Multiple sensing services initiate the Configuration of Parameters for sensing entities

- Sensing entities are performing sensing services.

- The Sensing Operation Server sends the sensing service request to the SCF either directly or indirectly via the NEF.

- The SCF verifies and authorizes the sensing service request and selects the target sensing entity(s) as described in KI#3.

- The SCF can detect the concurrent sensing services and properly deal with the concurrent sensing service(s) by merging the concurrent sensing services or triggering the configuration parameters update (e.g., sensing service Quality of Service).

NOTE:  The SCF may merge concurrent sensing services based on sensing entities' status, ongoing sensing quality of service, target area, sensing service type, requested sensing quality of service, AF ID, or other factors.

- Configuration parameters update between the SCF and sensing entity(s).

- The SCF sends the sensing service response to the sensing operation server.

#### 6.20.3.3	Sensing Result initiates Configuration of Parameters for sensing entities procedure

In some scenarios, the configuration of the parameter procedure may be triggered by the sensing result. The SCF will initiate the configuration of the parameters procedure (e.g., sensing service quality of service) to the sensing entity(s) based on the sensing result.

Figure 6.20.3.3: The SCF initiates the Configuration of Parameters for sensing entities based on the sensing result

- The sensing entity(s) send the sensing data to the SPF.

- The SPF calculates the sensing result based on the sensing data

- The SPF will notify the SCF of the sensing result. The SCF decides whether to initiate the configuration of parameters for sensing entities procedure based on the sensing result.

- The SCF sends the configuration of parameters for the sensing entity(s).

NOTE:  Steps 1 to 4 may be executed repeatedly.

### 6.20.4	Impacts on services, entities, and interfaces

This solution impacts the following entities.

NEF:

-	Supports the selection of the SCF based on internal target area information.

SCF:

-	Supports configuration parameter provisioning procedures

-	Supports the concurrent sensing services.

-  Determines whether to trigger configuration of parameters based on the sensing result or sensing entity status

-  Supports the subscribe/notification mechanism to AMF to obtain the sensing entity configuration information

Sensing Entity:

-	Supports sensing configuration parameter provisioning procedures.

* * * * End of Changes * * * *