3GPP TSG-WG SA2 Meeting #171	S2-2508774

13th – 17th October, 2025, Wuhan, China	(revision of S2-250xxxx)

Source: 	Lenovo

Title: 	KI#1 Interim Agreements

Document for: 	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This contribution proposes interim agreements for KI#1.

# 1	Discussion

The KI#1 is about System Architecture to Support Sensing. There are currently 12 solutions documented in the TR 23.700-14 V1.0.0 as follows: #1, #3, #4, #5, #6, #7, #19, #27, #28, #31, #34, #38. Some solutions have overlapping concepts and other solutions are diverging.

There are several categories of high-level decision points which need to be concluded. One attempt to categorize the open points is as follows:

- Which new functionalities are introduced in the 5GC for sensing service? How are these functionalities grouped in one or several NFs?

  - It is proposed to specify a Sensing Function (SF) NF that can implement the following functionalities (a high-level description is provided):

  - Gateway sensing control functionality (gateway SCF) supports sensing service request authorization, selection of distributed SF, sensing result exposure to NEF/AF.

  - Distributed sensing control functionality (distributed SCF) supports SE discovery and selection, SE configuration, SPF selection and configuration.

  - Sensing processing functionality (SPF) supports sensing data collection from SE, sensing result calculation, sensing result delivery to gateway SCF.

  - The above SF functionalities can be implemented in a single SF or multiple SFs, whereas any SF would support one or more of the gateway SCF, distributed SCF and SPF. Note that the gateway SCF , distributed SCF and SPF can be observed as logical functions.

- What kind of connectivity is supported between SF and SE?

  - It is proposed to support the following two options: direct connectivity and indirect connectivity via AMF. The protocol between the RAN SE and the SF/SCF is expected to be the same for both connectivity options.

- Whether there is a common reference point between RAN SE and SF, or there are different reference points between RAN SE and SF for control plane signalling or sensing data reporting?

  - It is proposed that to introduce a distinct reference points RAN SE and SF – one for control signalling and one for data reporting. It is expected that the control signalling amount and the signalling data amount will be considerably different and different protocol stacks can be used.

  - The decision about the reference points and the protocols will be taken in coordination with RAN3.

- Whether RAN SE can process the sensing data?

  - It is proposed that the RAN SE can pre-process the radio measurements with the purpose of data compressing and perform radio signal level evaluation. The RAN SE sends to the SF a pre-processed data that is used by the SPF to create a sensing result.

  - The decision about the sensing data format will be taken in coordination with RAN1 and RAN3.

# 2 	Proposal

It is proposed to capture the following contents into TR 23.700-14 V1.0.0.

# * * * * First change* * * *

### 7.1.X	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

The proposed reference architecture for sensing is shown in Figure 7.1.X-1.

Figure 7.1.X-1: Reference Architecture for Sensing

A new NF named Sensing Function is introduced in the 5GC. The SF supports the following functionalities which can be implemented by one or multiple logical functions:

-	The SF implements the following functionalities:

-	Gateway sensing control functionality (gateway SCF) supports: sensing service authorization, selection of distributed SF, sensing result exposure to NEF/AF, creation of charging report.

-	Distributed sensing control functionality (distributed SCF) supports: RAN SE discovery and selection, SE configuration, SPF selection and configuration.

-	Sensing processing functionality (SPF) supports: sensing data report reception from RAN SE, sensing result calculation, sensing result delivery to gateway SCF.

NOTE 1:	The above SF functionalities can be implemented in a single SF or multiple SFs, whereas any SF would support one or more of the gateway SCF, distributed SCF and SPF. The gateway SCF, distributed SCF and SPF can be observed as logical functions.

-	The connectivity between RAN SE and SF supports two options: direct connectivity and indirect connectivity via AMF. The protocol between the RAN SE and the SF/SCF is expected to be the same for both connectivity options.

-	Distinct reference points between the RAN SE and SF are introduced – one for control signalling (Sensing-2) and one for data reporting (Sensing-3).

-	It is expected that the control signalling amount and the signalling data amount will be considerably different and different protocol stacks can be used.

NOTE 2:	The final decision about the reference points and the protocols between the RAN SE and SF will be taken in coordination with RAN3.

-	The RAN SE provides sensing data to the SPF via the Sensing-3 reference point. The sensing data is based on pre-processed radio measurements. The SPF uses the sensing data and creates the sensing result.

NOTE 3:	The final decision about the sensing data format will be taken in coordination with RAN1 and RAN3.

# * * * * End of changes* * * *