3GPP TSG-SA WG2#171	S2-2508783

Wuhan, China, 13-17 October 2025

Source:	Qualcomm Incorporated

Title:	Revoking QoS differentiation activated for Non-3GPP Device Identifiers

Document for:	Discussion/Approval

Agenda Item:	19.8.2

Work Item / Release:	UIA_ARC/ Rel-19

Abstract of the contribution: Discusses the options for revoking QoS differentiation activated for Non-3GPP Device Identifiers.

# 1.	Discussion

During SA2#170, the CR regarding the revocation of QoS differentiation for QoS rules previously activated for Non-3GPP Device Identifiers (S2-2506733) was discussed. However, participants were unable to reach a consensus at the meeting.

Several companies proposed that the User Equipment (UE) could use packet filters linked to specific traffic flows to revoke QoS differentiation for Non-3GPP Device Identifiers, in line with the procedure defined in clause 4.3.3 of TS 23.502.

From the UE's perspective, this approach may present challenges. When the SMF/PCF generates a set of packet filters associated with a QoS flow, it considers both existing traffic and local network configurations. Consequently, the packet filters sent to the UE can be broader than the UE's requested SDF for a Non-3GPP Device Identifier. Under these circumstances, the UE cannot always establish a precise mapping between the network-provided packet filters and a specific Non-3GPP Device Identifier. If the UE initiates a PDU Session Modification Request to revoke a packet filter for a Non-3GPP Device Identifier, there is a risk of inadvertently affecting traffic related to other Non-3GPP Device Identifiers or UE applications.

To address these concerns when revoking QoS differentiation for Non-3GPP Device Identifiers, two options are proposed:

Option 1: The UE explicitly provides the Non-3GPP Device Identifiers in the PDU Session Modification to revoke QoS differentiation. The SMF/PCF can revoke the impacted packet filters or generate new filters.

Option 2: UE initiated revocation of QoS differentiation for Non-3GPP Device Identifiers is not supported in this release. SMF/PCF may determine whether to revoke QoS differentiation for Non-3GPP Device Identifiers based on network implementation (e.g., per AF request).

# 2.	Proposal

Based on the discussion in SA2 #170, several companies raised concerns regarding the introduction of new parameters under Option 1.

Given that the Stage 2 specifications for Release 19 are already frozen, and to minimize further impact on Stage 3 work items, we propose proceeding with Option 2 in CR 250xxxx as the preferred way forward.