SA WG2 Meeting #171	S2-2508796

Wuhan, P.R. China; 13th – 17th October 2025

Source:	NTT DOCOMO

Title:	[WT#2] 6G Migration Paths

Document for:	Approval

Agenda Item:	20.6.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes an annex in TR 23.801-01 to document the migration paths to 6G.

# 1. Justifications

This contribution proposes to document the possible migration paths in the annex of TR 23.801-01. The contribution builds on the baseline of S2-2507747, which was postponed at SA2#170.

The existing description of WT#2 as in the agreed SID titled as “Study on Architecture for 6G System” [SP-250806] is described as follows:

WT#2: Study migration and interworking, including

-	How to support migration to 6GS

-	How to support interworking with 5GS

-	Whether and how to support interworking with EPS

-	How to support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS.

NOTE 2:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is assumed to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 3:  The detailed migration study scope will be coordinated and aligned with RAN.

RAN study on 6G Scenarios and Requirement in TR 38.914 has agreed that the 6G RAN shall support Multi-RAT Spectrum Sharing (MRSS) between 6GR and NR. This is illustrated in the following figure:

Observation 1: Whether MRSS is applied or not, the interfaces in CN side or in the UE side are the same, therefore the figures in this document do not show MRSS, although it may be present.

Based on the input papers in SA2#170, many companies propose that the Core Network for 6G System (6G CN) is an evolution of the 5GC. This would mean that the 6G NFs can be expected to support the same (mandatory) features and procedures as the corresponding 5G NF. Either the protocols can be designed in backwards compatible manner, or if not always possible, the 6G NFs will support fallback to corresponding 5GS operation, when available.

Observation 2:  It is assumed that a single core network (i.e. the 6G CN) can support 5G NR, 6G NR and 5GC features and services.

The migration path 3 as documented in S-2507747 is the same as migration path 1 from 6G system point of view, therefore it can be excluded from the Annex.

# 2. Proposal

It is proposed to add the following to TR 23.801-01:

**** 1st Change ****

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

MRSS	Multi-RAT Spectrum Sharing

**** 2nd Change (all new) ****

Annex X
6G Migration Paths

# X.1 Introduction

This clause describes potential migration paths to 6G standalone system.

RAN study on 6G Scenarios and Requirement in TR 38.914 [x] has agreed that the 6G RAN shall support Multi-RAT Spectrum Sharing (MRSS) between 6GR and NR. This is illustrated in the following figure:

Figure X.1-1: 6G final deployment with MRSS between 6GR and NR.

Whether MRSS is applied or not, the interfaces in CN side or in UE side are the same, therefore the figures in the Annex X do not show MRSS, although it may be present.

### X.2	6G Migration Path 1, from 5G SA into 6G SA

This migration describes the case when 5G standalone deployment migrates into 6G standalone via 5G standalone and 6G standalone interworking with or without MRSS.

6G Migration Path 1 includes the following steps as described in Figure X.2-1. The 6G CN consists of (possibly re-designed or enhanced) 5GC NFs and 6G-level NFs necessary e.g. for AI, sensing, data framework, computing, etc, as discussed in the other WTs in this study.

Figure X.2-1: 6G Migration Path 1, from 5G SA into 6G SA

1.	Current deployment: 5G SA

2.	Intermediate deployment: 5G SA and 6G SA coexist with or without MRSS. 6G RAN spectrum can be re-farmed, a new band with MRSS, or new FR3 introduction, and can be used with MRSS.

3.	Final deployment: 6G SA

### X.3	6G Migration Path 2, from 5G NSA into 6G SA

This migration describes the case when 5G non-standalone deployment migrates into 6G standalone via 5G non-standalone and 6G standalone interworking with or without MRSS.

6G Migration Path 2 includes the following steps as described in Figure X.3-1.

Figure X.3-1: 6G Migration Path 2, from 5G NSA into 6G SA

1.	Current deployment: 5G NSA

2.	Intermediate deployment: 5G NSA and 6G SA coexist with or without MRSS. Radio spectrum used by 6G RAN can be re-farmed, a new band with MRSS, or new FR3 introduction, and can be used with MRSS.

3.	Final deployment: 6G SA

This migration path requires 6GS and EPS interworking.

### X.4	6G Migration Path 3, from 5G NSA and 5G SA into 6G SA

This migration describes the case when 5G non-standalone and 5G standalone deployments migrate into 6G standalone via 5G non-standalone and 5G standalone and 6G standalone interworking with or without MRSS.

6G Migration Path 3 includes the following steps as described in Figure X.4-1.

Figure X.4-1: 6G Migration Path 3, from 5G NSA and 5G SA into 6G SA

1.	Current deployment: 5G NSA and 5G SA

2.	Intermediate deployment 1: 5G NSA and 5G SA and 6G SA coexist with or without MRSS. 6G RAN spectrum can be re-farmed, a new band with MRSS, or new FR3 introduction, and can be used with MRSS.

3.	Intermediate deployment 2: 5G SA and 6G SA coexist with or without MRSS. 6G RAN spectrum can be re-farmed, a new band with MRSS, or new FR3 introduction, and can be used with MRSS.

3.	Final deployment: 6G SA

This migration path requires 6GS and EPS interworking.

**** End of Changes ****