3GPP TSG-WG SA2 Meeting #171	S2-2508810

Wuhan, China, October 13 - 17, 2025

Source: 	Interdigital

Title: 	Proposal for Interim Agreements on KI#1

Document for: 	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: proposal for interim agreements on KI#1.

# 1	Introduction

This contribution provides the interim agreements on KI#1 on System Architecture to Support Sensing.

# 2 	Proposal

It is proposed to adopt the following changes into TR 23.700-14 V1.0.0.

# * * * * First change* * * *

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.1	Agreed Principles for KI#1

- The Sensing NF is defined to manage sensing operation with sensing entities (i.e., gNB) for a sensing service request from a sensing service consumer.

- For each sensing NF, a sensing service area is defined at which the sensing NF may manage sensing operation. A Sensing Service Area is at the tracking area level.

- A Sensing NF may have a Sensing Control Functionality and a Sensing Processing Functionality.

Editor’s Note: It is up to RAN WG whether a gNB may have a Sensing Processing Functionality.

- AF (trusted/untrusted) and NF may be considered as a Sensing Service Consumer.

- There may be a Gateway sensing NF which is dedicated for authorization and result exposure for sensing service consumer. If a Gateway Sensing NF is defined, the sensing service consumer communicates with a Gateway sensing NF for service request and service response (via NEF, if needed.)

# * * * * End of change* * * *