3GPP TSG-WG SA2 Meeting #171	S2-2508815

Wuhan, China, October 13 - 17, 2025

Source: 	Interdigital

Title: 	Proposal for Interim Agreements on KI#4

Document for: 	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: proposal for interim agreements on KI#4.

# 1	Introduction

This contribution provides the interim agreements on KI#4 on Sensing Data and the Associated Information Collection and Transport

# 2 	Proposal

It is proposed to adopt the following changes into TR 23.700-14 V1.0.0.

# * * * * First change* * * *

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.4	Agreed Principles for KI#4

The sensing NF and the gNB as a sensing entity establish a control plane connection to exchange control signalling (e.g. a sensing operation request/response).

The sensing NF and the gNB as a sensing entity establish a data connection to collect sensing measurement data.

If the gNB may derive a sensing result, it is delivered to the sensing NF through the control plane connection.

Editor’s note: Whether the gNB may derive a sensing result is up to RAN WG.

The sensing operation ID, assigned after authorization, is used throughout the sensing operation and in reporting measurement data or results, serving as a reference to the corresponding sensing service request by the sensing service consumer.

The information required to establish a data connection between the sensing NF and the gNB is exchanged over the control plane connection.

The sensing NF sends a sensing operation request to initiate the sensing operation and to provide configuration information for the sensing operation.

NOTE 1:	The protocol for a control plane connection and a data connection between the sensing function and the sensing entity are determined by RAN WG.

# * * * * End of change* * * *