SA WG2 Meeting #171	S2-2508817

13 - 17 October, 2025, Wuhan, China                  	(revision of S2-25xxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | 0104 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of AIoT device profile data for stored temporary ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Accordin to latest TS 33.369, the ADM may provide the stored Temorary Device ID upon the AIOTF request (for Inventory procedure) and the AIOTF may also request the ADM to generate a new Temporary Device ID and stores it (at the end of Inventory procedure). Several sections in TS 23.369 needs to be aligned with TS 33.369. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a “Temporary Device ID” filed in AIoT device profile data. - Add a “Temporary Device ID generation request indication” in Nudm_Update service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mechanism of using and mangaing Temporary Device ID for Inventory procedure is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5, 7.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 5.5	AIoT Device Profile Management

The ADM may hold operator’s subscription data for the AIoT Device used in the network. If the AIoT Device is managed by the network, then the profile data for an AIoT Device is required in the network, otherwise the corresponding profile data (e.g. AIoT Device Permanent ID or credentials) is stored external to the network.

The AIoT Device Permanent ID is used by the AIOTF together with local configuration, 3rd party related context to locate the entity which stores the profile data of an AIoT Device.

In case the AIoT Device is managed by the network, the AIOTF checks whether the AIoT Device Permanent ID from AIoT Device has the profile data in the network and retrieves the profile data. The profile data for AIoT Device is different with UE subscription data as defined in clause 5.2.3 of TS 23.502 [4], it is stored in the ADM  network entity that exclusively supports management of AIoT Device’s profile data. The AIoT Device Permanent ID is the primary key for AIoT device profile data in the ADM.

The table 5.5-1 below describes information storage structures for AIoT device profile data.

Table 5.5-1: AIoT Device Profile Data

| Field | Description |
| --- | --- |
| AIoT Device Permanent ID | Uniquely identifies the AIoT Device. |
| Last known AIOTF information | Indicate the last known AIOTF that serves the AIoT device, or unknown |
| Temporary Device ID | Temporary device identifier that the ADM generates and stores in device profile. |

NOTE:	Security materials and security mechanism involving ADM, including the generation of Temporary Device ID, are specified in TS 33.369 [9].

SECOND CHANGE

### 7.5.3	Nadm_DM_Update service operation

Service operation name: Nadm_DM_Update

Description: NF service consumer may update the AIoT device profile data in the ADM.

Inputs, Required: AIoT Device Permanent ID, updated AIoT device profile data.

Input, Optional: Temporary Device ID generation request indication.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure.

Output, Optional: None.

END OF CHANGES