3GPP TSG-WG2 Meeting #171	S2-2508852

October 13 – 17, 2025, Wuhan, China	(revision of S2-25xxxxx)

Source:	Ericsson

Title:	Solution#36 updates on association procedures

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution:

This contribution proposes an update for Solution#36 by adding details and including complementary procedures related to sensing association.

# 1. Discussion

This solution (#36 in the TR 23.700-14) has good alignment with S2-2506344 that has not been handled at SA2#170 meeting. This paper adds some missing details on sensing association establishment procedure and includes procedures for removal and configuration update of sensing association.

Added principles are also valid for solution #16 which also introduces association request from Sensing Entity.

# 2. Proposal

It is proposed to agree the following solution to 3GPP TR 23.700-14.

***** Start of Changes *****

## 6.36	Solution #36: Sensing Entity reselection/sensing service revocation

### 6.36.0	High-level solution Principles

The high-level principles for Sensing Entity reselection/sensing service revocation are:

-	Define Sensing Non-allowed Area.

-	Sensing Entity request Sensing Function to pause/revoke the sensing task with cause value of 1) the sensing object is approaching the Sensing Non-Allowed Area, 2) the sensing object is moving out of its coverage.

-	Sensing Function determines to pause/revoke the sensing service, or reselect Sensing Entity.

### 6.36.1	Description

Based on regulatory requirements, sensing may not be allowed in some places, e.g. Government office, Military restricted area etc. Therefore, we define Sensing Non-Allowed Area as follows:

-	Sensing Non-Allowed Area: An area where Sensing Entity is not allowed to initiate the sensing operation.

For gNB-based sensing, the sensing RAN node is generally fixed and static. Meanwhile, sensing object (e.g. UAV) may be mobile, which may move into the Sensing Non-Allowed area. Upon detection of sensing object is approaching the Sensing Non-Allowed Area, the Sensing RAN node may request to pause/revoke the sensing task. Besides, the Sensing RAN node may request to pause/revoke the sensing task if the sensing object moves out of its coverage.

NOTE:	Sensing Non-Allowed Area may be different for different sensing application/services/requests.

Editor's note:The form of the Sensing Non-allowed Area is FFS.

### 6.36.2	Procedures

#### 6.36.2.1	Procedures for Sensing association establishment and Sensing Entity reselection/sensing service revocation

Figure 6.36.2.1-1: Sensing Entity reselection/sensing service revocation procedure

1.	Sensing RAN node sends sensing association request to the SF, which may include RAN node ID, sensing capability information, sensing RAN node’s address and NG-RAN serving sensing area. It is assumed that the NG-RAN serving sensing area is pre-configured by OAM and Sensing RAN node obtains SF information (e.g., SF address) by pre-configuration, via DNS interaction or from AMF. E.g. the Sensing RAN node provides Sensing Capability information/indication to the AMF, and the AMF responds with SF information. Sensing capability information may include supported sensing mode, supported accuracy of sensing, confidence level, sensing resolution, false alarm probability, missed detection probability, refreshing rate, max sensing service latency, Tx/Rx support indicator, etc.

Editor's note:	Whether the sensing RAN node can send the sensing capability information to the SF is based on RAN decision and is FFS.

2.	SF sends sensing association response message to the sensing RAN node, which may include the Sensing Non-Allowed Area information. The Sensing Non-Allowed Area information may be preconfigured in SF or provided by the 3rd party AF in advance. SF stores the information obtained in step 1 and may also update the SF serving sensing area information in its NF profile in NRF.

3.	AF/NEF sends Sensing Service Request message to the SF, which includes sensing task ID, target sensing area sensing requirement and sensing exposure requirement etc. Sensing requirement may include sensing accuracy, sensing resolution, confidence level, missed detection probability, false alarm probability, max sensing service latency, refreshing rate etc. Sensing exposure requirement may include notify method (e.g. one time, periodic or event trigger), reporting periodicity, waiting time etc. Besides, the Sensing Non-Allowed Area information may also be provided by 3rd party AF.

4.	SF triggers sensing task assignment procedure towards the selected sensing RAN node. E.g. SF may send sensing task assignment request message to the sensing RAN node, which includes sensing task ID, sensing mode and sensing requirement. Besides, the request message may also include the Sensing Non-Allowed Area information if not provided in step 2.

5.	After sensing task assignment, SF sends Sensing Service Notify/Response message to AF/NEF.

6.	The sensing RAN node performs the sensing task. The sensing RAN node determines if the sensing object is approaching Sensing Non-Allowed Area or is out of its coverage based on the sensing measurement data.

Editor's note:	How RAN node makes the determination is FFS and depends on the coordination with RAN WGs.

7.	The sensing RAN node sends notification to the SF, which includes sensing task ID and event info. E.g. event info may indicate that the sensing object is approaching Sensing Non-Allowed Area, or the sensing object is out of coverage or other reasons.

8.	Upon receiving the notification from the sensing RAN node, the SF determines to pause/revoke the sensing task or reselect a new Sensing Entity. E.g. if the event#1 indicates that sensing object is approaching Sensing Non-Allowed Area, the SF may determine to pause/revoke the sensing task. If the event#2 indicates that sensing object is out of coverage, the SF may determine to reselect a new Sensing Entity.

9.	SF sends sensing task ID and sensing pause/revoke indication to the sensing RAN node. For event#2, SF sends sensing task ID and sensing revoke indication to the sensing RAN node. Besides, SF triggers sensing task assignment procedure towards the new Sensing Entity. In this case, step 10 is skipped.

10.	Optionally, SF sends sensing task ID and revoke indication to AF/NEF.

#### 6.36.2.2	Procedure for configuration and registration of SF serving sensing area

This clause describes procedure for configuration of SF serving area and its registration in NRF. As an alternative option to this procedure, the SF serving sensing area can be established and registered dynamically based on the association process described in steps 1 and 2 of figure 6.36.2.1-1.

Figure 6.36.2.2-1: SF serving sensing area configuration and registration in NRF

- SF is configured with the sensing area that it serves.

- SF sends Nnrf_NFManagement_NFUpdate Request message to NRF to update its NF Profile related to the SF serving sensing area.

NOTE: Exact composition of Serving Sensing Area is FFS

- The NRF acknowledges that NF Update is accepted via Nnrf_NFManagement_NFUpdate response.

#### 6.36.2.3	Procedure for Sensing Configuration Update

In this procedure sensing RAN node updates the configuration of the sensing association.

Figure 6.36.2.3-1: Sensing RAN node updates the configuration of the sensing association

- Sensing RAN node sends Sensing Configuration Update Request to the SF for the sensing service if e.g., the NG-RAN serving sensing area has been changed. SF updates its stored serving sensing area if it is required.

- SF sends an acknowledgement to the sensing RAN node to indicate the successful configuration update.

- [Conditional] If the serving sensing area of the SF has changed due to the update from the sensing RAN node, the SF sends the Nnrf_NFManagement_NFUpdate Request message to NRF to update its NF Profile related to its serving sensing area.

- [Conditional] The NRF acknowledges NF Update is accepted via Nnrf_NFManagement_NFUpdate response.

NOTE. Step 3 and 4 are not required for that option when sensing area is pre-defined in SF.

#### 6.36.2.4	Sensing Association Removal

Sensing RAN node may decide to remove the sensing association.

Figure 6.36.2.4-1: Sensing RAN node removes the sensing association

- Sensing RAN node sends a Sensing Association Removal request to SF. SF updates its stored serving sensing area if it is required.

- SF sends an acknowledgement to the sensing RAN node about the removal.

- [Conditional] If the removal changes the serving sensing area of the SF and an NF profile update is required, SF sends Nnrf_NFManagement_NFUpdate Request message to NRF to update its NF Profile related to serving sensing area.

- [Conditional] The NRF acknowledges NF Update is accepted via Nnrf_NFManagement_NFUpdate response.

NOTE. Step 3 and 4 are not required in case when serving sensing area is pre-defined in SF.

Editor’s Note: this can be further evaluated by RAN WGs.

### 6.36.3	Impacts on services, entities and interfaces

AF/NEF:

-	provides the Sensing Non-Allowed Area information to SF.

SF:

-	provides the Sensing Non-Allowed Area information to sensing RAN node.

-	receive the notification associated with the sensing task from the sensing RAN node. determine to pause/revoke the sensing task or reselect a new Sensing Entity based on the event information in the notification.

RAN:

-	receives the Sensing Non-Allowed Area information from SF.

-	sends notification of the sensing task.

-  Establishing direct sensing association with SF.

-  Sending sensing configuration updated message to SF.

-  Removing direct sensing association with SF.

***** End of Changes *****