SA WG2 Meeting #171	S2-2508867

13 – 17 October, 2025, Wuhan, China	revision of S2-250xxxx

Source:	Ericsson

Title:	[WT#1.3] WT and KI proposal for non-3GPP access

Document for:	Approval

Agenda Item:	20.6.1.3

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract of the contribution: This paper proposes WT and KI for WT#1.3 on non-3GPP access.

# Justification

Using non-3GPP access has become an important complement to 3GPP access for many operators. We propose a WT for non-3GPP in 6G based on the following justifications and considerations:

- 	The main use case deployed so far for non-3GPP access (in 4G/5G) is to support untrusted access primarily for voice over WiFi. Therefore, the WT should include similar support for 6G, i.e. support for untrusted non-3GPP access.

- 	It is not clear to us that wireline access should be specifically included in the first 6G release. For example, there is at present no coordination with BBF or Cablelabs. It may also be more suitable to stabilize the 6G System first before discussing with BBF/Cablelabs. In addition, it could be studied if the solution for untrusted non-3GPP access can be re-used also for wireline access, to avoid multiple solutions as we have seen in 5GS. This may be more suitable to do when the solution for 6GS and untrusted non-3GPP access are available.

- 	It is preferable, e.g. for clarity and tracking purposes, to divide general support for non-3GPP access, NSWO and Multi-access in separate sub-WTs. This should then become separate KIs or sub-KIs in the 6G TR.

- 	Coordination with SA3 is needed in several aspects of this WT, e.g. when it comes to authentication for non-3GPP access or for NSWO.

# Proposal

It is proposed to approve the following WT scope and key issue in TR23.801-01.

**** Start of Changes ****

# Annex A.1.3 	Non-3GPP access

This WT contains separate sub-WTs to address the non-3GPP access scenario and multi-access data connections.

WT#1.3.	Study whether and how to support and/or enhance different non-3GPP access (e.g. Wi-Fi) in 6G and support multi-access data connections between 3GPP access and non-3GPP access.

WT#1.3.1. For non-3GPP access connectivity via CN:

-	Study how to support untrusted non-3GPP access in 6G System architecture.

-	How to support service continuity between 3GPP access and untrusted non-3GPP access.

Solutions for the above bullets should minimize complexity in the device and in the network and ensure that they can be supported by devices of different capabilities. This WT targets a single solution that is common for different device capabilities.

NOTE 1:	Interworking aspects related to non-3GPP access will be coordinated with WT#2.

NOTE 2:  WT#1.2 aspects related to QoS, Policy, and UP applicable to non-3GPP access are studied under this WT with a checkpoint to align with WT#1.2 at TBD.

WT#1.3.2. For NSWO:

-	Whether and how to support Non-Seamless WLAN Offload (NSWO).

WT#1.3.3. For multi-access:

-	Based on support for non-3GPP access in 6G (WT#1.3.1), study whether and how to support the multi-access data connectivity (e.g. traffic steering, switching and splitting) between 3GPP access and untrusted non-3GPP access for different type of traffic (e.g. IPv4/v6, UDP, TCP, Ethernet) in 6G.

NOTE 3: Coordination with SA3 is needed for this WT (all sub-WTs)

**** Second Change ****

## 5.X Key Issue #X: Support for non-3GPP access in 6G

This key issue will study solutions that enable a UE to access via non-3GPP access (e.g. WiFi).

This KI targets a single solution for non-3GPP access support, common for different device capabilities, i.e. solutions to the KI should minimize complexity to ensure that it can be supported by devices of different capabilities.

The KI includes the following sub-KIs:

KI#X.1. For non-3GPP access connectivity via CN:

-	Study how to support untrusted non-3GPP access in 6G System architecture.

-	How to support service continuity between 3GPP access and untrusted non-3GPP access.

Solutions for the above bullets should minimize complexity in the device and in the network and ensure that they can be supported by devices of different capabilities. This sub-KI targets a single solution that is common for different device capabilities.

NOTE 1:	Interworking aspects related to non-3GPP access will be coordinated with WT#2.

NOTE 2:  Aspects related to QoS, Policy, and UP applicable to non-3GPP access are studied under this KI with a checkpoint to align with WT#1.2 at TBD.

KI#X.2. For NSWO:

-	Whether and how to support Non-Seamless WLAN Offload (NSWO).

KI#X.3. For multi-access:

-	Based on support for non-3GPP access in 6G (KI#X.1), study whether and how to support the multi-access data connectivity (e.g. traffic steering, switching and splitting) between 3GPP access and untrusted non-3GPP access for different type of traffic (e.g. IPv4/v6, UDP, TCP, Ethernet) in 6G.

NOTE 3: Coordination with SA3 is needed for this KI (all sub-KIs)

**** End of Changes ****