SA WG2 Meeting #171	S2-2508885

13 – 17 October 2025, Wuhan, China	(was S2-2508128)

Source:	Ericsson, BT

Title:	[WT#1.4, Essential Services] WT & KI for Essential Services

Document for:	Agreement

Agenda Item:	20.6.1.4

Work Item / Release:	FS_6G_ARC / R20

Abstract of the contribution: This contribution is intended to define the scope of the Work Task for essential service support in 6G Systems and related Key Issues.

# 1. Discussion

This contribution proposes to consolidate the interactions required to support Essential Services in 6G to avoid multiple options and duplication of reference points (e.g., Rx Diameter and N5 SBA), that exist today in 5G. The following aspects have been taken into account:

- Legacy IMS can provide service via Diameter through 5GC (and 4G interworking), HSS etc., which should be the predominant network when 6G is launched, so there is no clear drawback in not supporting IMS Diameter reference points in 6G

- The drawback of keeping legacy diameter interfaces in IMS and between IMS and Core Network is that legacy interfaces will need to be enhanced to support new capabilities for 6G as well as advanced IMS communications, and this results in two parallel tracks for 3GPP specifications (Diameter and SBA). Meaning additional cost and complexity.

- Maintaining both Diameter and SBI, will result in cost and complexity throughout 6G lifecycle which will imply many years ahead. The following reference points are currently duplicated in 5G:

  - PCF: N5 (Rx)

  - HSS: N70 (Cx), N71 (Sh), N72 (from DCSF - Sc), N65 (Zh)

  - CHF: N45 (Ro/Rf)

- Generally, the SBI interfaces provide increased security and are future proof aligned with the evolution of the rest of the network.

**** First Change ****

# Annex A.X. WT#1.4 6G Essential Services

Study whether and how to support and/or enhance the essential/regulatory services (i.e. voice, Messaging, location services, Emergency services, MPS, Mission Critical services, PWS) in 6G.

Study whether and how to consolidate the interactions required to support Essential Services in 6G, to avoid multiple options and duplication of reference points (i.e. Diameter and SBA) within IMS and between IMS and the Core Network.

**** Second Change ****

# 5.P. Key Issue #P: Voice Services for 6G

This Key Issue will investigate how to support voice services for the 6G system (using 5GS as the starting point for discussion) – the following aspects will be studied:

- How a UE supporting 6G RAN selects and connects to a network for voice service.

- Whether and how to update the IMS specification to support an IPCAN in the 6GS.

- How the 6GS supports IMS based voice over the 6G access network.

- Whether and how to simplify the signalling interactions required to support Essential Services in 6G, to avoid multiple options and duplication of reference points (i.e. Diameter and SBA) within IMS and between IMS and the Core Network.

Editor’s Note: It is TBD if voice services for 6G NTN will be studied as part of this Key Issue or by a Key Issue related to WT7.

Editor’s Note: It is TBD if interworking, migration, and service continuity (including the issue of “fallback”) for voice services will be studied as part of this Key Issue or by a Key Issue related to WT2.

Editor’s Note: It is still to be determined if other capabilities will be considered as part of this KI.

*** End of Changes ****