3GPP TSG-SA2 Meeting #171	S2-2508887

Wuhan, China, 13 – 17 October 2025	(revision of S2-2506407)

Source:	Ericsson

Title:	Energy Consumption information collection from SMF

Document for:	Discussion

Agenda Item:	19.4.2

Work Item / Release:	EnergySys

Abstract: UPF Event Exposure can be used for information collection by SMF for Energy Consumption with no impact on PCC Rules.

# 1. Introduction

Energy Consumption information collection from SMF is not related to Policy or Charging Control however it impacts the rules that SMF installs in UPF and that SMF determines mostly from PCC Rules from PCF or other Local Policies.

According to TS 23.501 clause 5.51.2.2.2 (Energy Consumption information collection from SMF)

When the SMF receives the Nsmf_EventExposure_Subscribe for a Service Data Flow from the EIF, the SMF generates a URR for the Data Volume counting with a Periodic measurement threshold set according to the Timer interval T and associates the subscription from EIF to the generated URR. The SMF is then associating the generated URR to an appropriate PDR. SMF logic ensures that the PDRs and their precedence values are configured appropriately in the UPF so that both, the URRs for Data Volume counting for EIF and the traffic handling instructions derived from the existing PCC rules can co-exist and the traffic of the Service Data Flow is treated in the same way as before. For example, the SMF may behave as follows:

-	If there is a PCC rule existing for that Service Data Flow, the SMF associates the generated URR with the PDR of that PCC rule.

-	If there is no PCC rule for the Service Data Flow, the SMF generates a PDR for the Service Data Flow and associates the generated URR with that PDR. All other instructions of the PDR related to the match-all PCC rule are copied (so that the traffic of the Service Data Flow is treated in the same way as before).

-	In all other cases, e.g. if the Service Data Flow is partially overlapping with an existing PCC rule (which can only occur when the subscription contains Packet Filters), the SMF may need to generate PDR(s), associate the generated URR with the(se) PDR(s) and copy all other instructions of the PDR related to the existing PCC rule so that both, the Data Volume counting for EIF and the traffic handling instructions derived from the existing PCC rule can be realized.

If the PCC rules are changed, the SMF shall again perform the above checks and actions.

Discussion follows that elaborates how the procedure specified does not deliver the results expected.

# 2. Discussion

From TS 23.503 clause 6.3.1:

The Policy and charging control rule (PCC rule) comprises the information that is required to enable the user plane detection of, the policy control and proper charging for a service data flow. The packets detected by applying the service data flow template of a PCC rule form a service data flow.

From TS 23.053 table 6.3.1-1, the PCC Rule information that is used by UPF for detecting the packets belonging to a service data flow consists of:

Precedence: “Determines the order, in which the service data flow templates are applied at service data flow detection, enforcement and charging”

Service data flow template: “For IP PDU traffic: Either a list of service data flow filters or an application identifier that references the corresponding application detection filter for the detection of the service data flow.

For Ethernet PDU traffic: Combination of traffic patterns of the Ethernet PDU traffic”

Observation 1: the service data flow is determined by the user plane detection and consists of the packets that match a given service data flow template after evaluation of one or more service data flow templates according to their precedence.

Observation 2: The service data flows are detected by UPF, requiring in some cases application detection filters which are defined in UPF. In principle, SMF is not aware of the service data flows.

According to TS 23.502 Table 5.2.8.3.1-1: (Example of Event Filters for SMF exposure events). SMF could receive one or more of the following event filters for the Usage data per gNB and (I-)UPF Event ID,:

<Parameter Type = S-NSSAI, Value = S-NSSAI1>

<Parameter Type = DNN, Value = DNN1>

<Parameter Type = Application Identifier, Value = Application Identifier1> (NOTE 2)

<Parameter Type = Flow Info, Value = Packet Filter Set1> (NOTE 2)

NOTE 2:	These Parameters are exclusive and only one of them can be provided

Observation 3: In TS 23.501 clause 5.51.2.2.2 (see Introduction) instead of “Service Data Flow”, the procedure should consider as input information that SMF has, in particular, “Application Identifier” or “Flow Info” received in EIF Nsmf_EE request.

According to the procedure in TS 23.501 clause 5.51.2.2.2, SMF should determine which N4 rules to send to UPF to satisfy the EIF request received. The procedure is however underspecified and oversimplified:

-  The procedure does not mention how the precedence of the N4 rules is set. Precedence impacts UPF service data flow detection and it needs to be selected with care. One fair assumption is that the new PDR takes precedence over the reference PDR (PDR whose all other instructions are copied) and takes precedence over exactly the same other PDRs than the reference PDR (and no other PDR).

-  Determining the reference PDR is a major challenge. As specified, SMF searches for the PCC rule with a service data flow template which exactly matches or contains the application identifier or flow info received (as in TS 23.501 clause 5.51.2.2.2, the “service data flow”).

But this is complex and not always possible:

a) The described procedure is not compatible with predefined rules, a common practice to save N4 signalling when many users share many PDRs. SMF is unaware of the contents of the PDRs which are predefined in UPF.

b) When Nsmf_EE request includes Flow Info:

-  Nsmf_EE request may include a Packet Filter Set which overlaps with more than one PCC Rule service data flow template. In this case, SMF needs to run a PCC Rule matching procedure (similar to PDR matching procedure) to evaluate the PCC rules in precedence order. This is complex and consumes resources.

-  Nsmf_EE request may include a Packet Filter Set which overlaps with more than one PCC Rule service data flow template, where the intention is to program different PCC enforcements in UPF considering more than the 5-tuple (e.g. ECN bits). When SMF selects a reference PDR, if the traffic matches in UPF the other PDR(s), the volume of this data (which is also according to the traffic Packet Filter Set) is not reported.

-  Nsmf_EE request may include a Packet Filter Set which corresponds to an application for which PCC Rule service data flow template includes an Application Identifier. SMF can not identify the PCC Rule. SMF sets the match-all as the reference PDR and the volume of this data (which is according to the traffic Packet Filter Set) is not reported.

c) Nsmf__EE request includes an application identifier:

-  SMF selects the PCC rule with a service data flow template which exactly matches or contains the application identifier received. If the application traffic matches a PDR with precedence based on packet filter set the volume of this data (which belongs to the application) is not reported.

Observation 4: TS 23.501 clause 5.51.2.2.2 describes a procedure that is complex and consumes resources in SMF and which does not deliver the results expected. In addition, it is incompatible with key functionalities for signalling (and energy) saving like predefined rules.

On the other hand,

- UPF is the entity that manages the data plane and is able to determine which packets match which input criteria and,

- Nupf_EventExposure service can be used to request data volumes and supports Flow Info and Application identifier as input filters in the service subscription.

Observation 5: as an alternative, SMF could use Nupf_Event Exposure for UPF event User Data Usage Measures to collect information of data volumes.

The following enhancements are needed on the Nupf_Event Exposure service:

- Indication that the system preconfigured measurement intervals shall be used for for the measurements (period duration but also start times for the periods). This can be used to guarantee the measurements are taken all over the system at the same time.

- Consumers should be able request at any time a UPF report with the latest volume measurement. This can be used by SMF at data path change to get the volume measurements for the former data path.

- For MA PDU Sessions, consumers should be able request the volume measured on each access (volume measured on the 3GPP access, and volume measured on the non-3GPP access). This can be used by SMF to consider the volume on the 3GPP access only.

In an alternative (Alt.A) where Nupf_EventExposure service is used for energy consumption information collection, the procedure would be as follows (taking TS 23.502 4.29.2 as a basis):

Figure 1: Energy Consumption information collection (based on Figure 4.29.2-1)

1.	The Consumer NF (i.e. trusted AF or 5GC NF) subscribes to the energy consumption event with the EIF by using Neif_EventExposure_Subscribe service for the required granularities as described in clause 5.51.2 of TS 23.501 [2] and other necessary information as defined in clause 5.2.28.

If the Consumer is untrusted AF, the subscription is sent via NEF. If the event subscription is authorized by the NEF, then the NEF sends the event subscription to the EIF via Neif_EventExposure_Subscribe.

2.	The EIF invokes Nudm_UECM_Get service operation to retrieve the SMF ID for the required granularity.

3.	The EIF subscribes to be notified using Nudm_SDM_Subscribe regarding the establishment of the PDU Session(s) of the UE.

4.	For the SMF ID retrieved from step 2 or for the new SMF ID retrieved from step 7, the EIF invokes the Nsmf_EventExposure_Subscribe request to the SMF to subscribe to Event ID Usage data per gNB and (I-)UPF, defined in clause 5.2.8.3.1, to collect the information as defined in clause 5.51.2 of TS 23.501 [2].

If the I-SMF is involved in the PDU session, the SMF of PDU session requests to I-SMF to report the ULI and I-UPF ID.

5a.	The SMF uses Nupf_EventExposure_Subscribe service to request to PSA UPF to report the data volume at the required granularity , per access (if the session is a MA PDu Session and with periodic reporting with period defined by the SMF to be the time interval T defined in clause 5.51.2.2.1 of TS 23.501 [2]. The SMF may also request  to report the current value of the measurements on demand by requesting immediate reporting when a new gNB ID and/or UPF-ID of a I-UPF is received at the SMF for the UE PDU session as defined in clause 5.51.2.2.2 of TS 23.501 [2].

5b.	The PSA UPF provides the data volume of the required granularity to the SMF as configured by step 5a in Nupf_EventExposure_Notify.

6.	The SMF periodically provides the information collected for energy consumption calculation at the end of each time interval T (defined in clause 5.51.2 of TS 23.501 [2]) to EIF starting from the time that the EIF subscribes by invoking Nsmf_EventExposure_Notify service operation for the required granularity. Upon the PDU session release, the SMF terminates the subscriptions and notifies the EIF the information it has collected since the last reporting. If there are no UP resources established on the 3GPP access for the PDU session of the UE, the SMF behaves as described in clause 5.51.2 of TS 23.501 [2]. For MA PDU Sessions, only the volume measurement corresponding to the 3GPP access is considered.

7.	The UDM notifies the EIF via Nudm_SDM_Notification including the PDU Session ID and SMF ID when PDU Session(s) of the UE are established at any time. Steps 4, 5 and 6 are repeated for the new PDU Session if it matches the required granularity.

8.	EIF unsubscribes the subscriptions upon the expiration of the request time period, or when the Consumer NF unsubscribes.

9.	Upon the expiration of the reporting time period, or when the Consumer unsubscribes, the EIF unsubscribes by sending Nsmf_EventExposure_UnSubscribe request, and SMF notifies the EIF the information it has collected since the last reporting.

10.	If the related Node-level energy consumption and Node-level data volume information is not available in the EIF, the EIF obtains the Node-level energy consumption and Node-level data volume information from OAM as defined in clause 5.51.2.2.3 of TS 23.501 [2].

11.	The EIF performs the energy consumption information calculation as described in clause 5.51.2.3 of TS 23.501 [2] and then reports the energy consumption information of the required granularity to the Consumer NF via Neif_EventExposure_Notify.

On the other hand, if URR/UR is used, the description in TS 23.501 5.51.2.2.2 would need to be clarified and all limitations documented, for example as follows:

When the SMF receives the Nsmf_EventExposure_Subscribe for energy consumption, the SMF generates a URR for the Data Volume counting with a Periodic measurement threshold set according to the Timer interval T and associates the subscription from EIF to the generated URR. The SMF is then associating the generated URR to the appropriate PDR(s). SMF logic ensures that the PDRs and their precedence values are configured appropriately in the UPF so that both, the URRs for Data Volume counting for EIF and the traffic handling instructions derived from the existing PCC rules can co-exist and the Service Data Flows are treated in the same way as before.

For example, when the SMF receives the Nsmf_EventExposure_Subscribe with application information, the SMF may behave as follows:

-	If there is an existing PCC rule for exactly the same service data flows than the application information in the energy consumption request (e.g. with same App Id or same packet filters), the SMF associates the generated URR with the PDR of that PCC rule.

-	If there is no PCC rule for the service data flows that would be according to the application information in the energy consumption request (e.g. the App Ids are different or no packet filter overlap) but a match-all PDR has been provisioned for the PDU session, the SMF generates a PDR for the Service Data Flow and associates the generated URR with that PDR. All other instructions of the PDR related to the match-all PCC rule are copied (so that the traffic of the Service Data Flow is treated in the same way as before).

NOTE x1:	The SMF cannot generate a PDR due to the energy consumption request and request the UPF to forward the matched traffic if the same is not allowed by the PCF or the local policy..

NOTE x2:	If there is no match-all PDR provisioned for the PDU session, any application traffic without being authorized by the PCF or the local policy will be discarded.

-	If the service data flows corresponding to the application information in the energy consumption request are partially overlapping with the service data flow(s) of an existing PCC rule (which can only occur when the subscription contains Packet Filters), the SMF may need to generate PDR(s), associate the generated URR with the(se) PDR(s) and copy all other instructions of the PDR related to the existing PCC rule so that both, the Data Volume counting for EIF and the traffic handling instructions derived from the existing PCC rule can be realized.

NOTE x: One or more of the above may apply to an energy consumption request.

NOTE y: SMF may be unable to determine the PCC rule and PDRs to update if the PCC rule includes App Id and EIF provides packet filters and the other way around.

NOTE z: This mechanism cannot be used together with predefined rules where packet detection rules are locally defined in UPF.

NOTE w: For MA PDU Sessions, the generated URRs shall be included specific for the 3GPP access in the Multi-Access Rule (see 5.8.5.8).

If the PCC rules are changed, the SMF shall again perform the above checks and actions.

When the SMF receives the Nsmf_EventExposure_Subscribe for the PDU Session (i.e. without application information) from the EIF, the SMF associates the subscription from EIF to the PDU Session, generates a URR for the Data Volume counting (with a Periodic measurement threshold set according to the Timer interval T) and associates it with all PDRs of the PDU Session.

# 3. Proposal

Correct the procedure for Data Collection for Energy Consumption.

There are two alternatives:

-   Alt.A Enhance and use the UPF Event Exposure Service instead of using N4 URR/UR Reports

-   Alt.B Update the description for how Usage Reports over N4 can be used for this use case, including limitations and issues encountered.

Alt.A is simpler and it has not the issues and limitations identified above, therefore it is preferred.

2 CRs (S2-2508888 and S2-2508889) are submitted to update 23.501 and 23.502 according to Alt.A and the procedure described above.