SA WG2 Meeting #171	S2-2508938

13 – 17 Oct, 2025, Wuhan, CN

Source:	Apple

Title:	[WT#2] Migration and Interworking

Document for:	Agreement

Agenda Item:	20.6.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution provides updated technical reasoning, gap and impact analysis for Migration and Interworking, proposes a revision of the scope for WT#2, and adds a Key Issue description based on the progress made in SA2#170.

# 1     Justifications

## Introduction

The 6G SA2 SID in SP-250806 [x2] approved at TSG SA #108 contains the following Work Task corresponding to migration and interworking:

WT#2: Study migration and interworking, including

-	How to support migration to 6GS

-	How to support interworking with 5GS

-	Whether and how to support interworking with EPS

-	How to support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS.

NOTE 2:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is assumed to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 3:  The detailed migration study scope will be coordinated and aligned with RAN.

### Key Learnings from 5G

The transition from 4G to 5G highlighted several critical lessons. Under the prospect of NextGen system migration 3GPP had studied 7 network architecture options out of which 5 got standardized but only two of them, SA option 2 and NSA option 3, got widely deployed. While NSA allows to leverage 4G deployments for gradual extensions with 5G radio technology, E2E services remain anchored in the EPC. As a result, the introduction of NSA delayed the deployment of the 5GC along with its native 5G services. NSA added considerable levels of complexities in specifications, products, and deployment processes. Moreover, the user experience with NSA often fell short, for example, due to latency issues in accessing higher data rates with 5G NR technologies. These insights are pivotal in shaping the strategic direction for 6G development.

Observation 1: Looking ahead to 6G, establishing a clear (ideally single) migration path is essential to prevent industry fragmentation and to ensure more predictable investment and deployment timelines.

## Migration to 6GS

### Technical Challenges

Any new generation requires a strategy for migrating from existing deployments to the next version of the system. The migration to 6G is influenced by several critical factors that require careful consideration. Key drivers include the support of new 6G services and use cases, alignment with business objectives, and strategic spectrum allocation. Additionally, the effectiveness of utilizing existing network infrastructure, alongside the need for network infrastructure upgrades and investments, plays a vital role.

6G will be introduced alongside the existing 5G network, employing a combination of new and legacy spectrum bands. The gradual deployment of new 6G equipment must be managed to ensure seamless network integration while minimizing the impact on current 5G hardware and software. It is vital to prevent market fragmentation by limiting multiple architecture options and to avoid unnecessary system complexity unless it directly enhances end-user value.

Technically, 6G is expected to deliver superior performance from the outset, offering improved uplink and downlink peak rates, reduced latency, and optimized battery consumption that matches or surpasses that of 5G devices. Moreover, 5G users should experience no degradation in performance during this migration.

Observation 2: The migration architecture needs to be studied.

### Architectural Assumptions

Architectural assumptions for 6G anticipate that by 2030 most operators will have deployed the 5GC in SA mode. It is expected that the 6G Core will evolve directly from 5GC, ensuring a smooth transition. From the outset, 6G Standalone may serve as the primary architectural option, providing robust and advanced capabilities. Additionally, the architecture needs to be designed to support deployments in environments with non-homogeneous 6G coverage, ensuring flexibility and broad adaptability.

A 6G system is expected to support new UEs, device types and services. To efficiently support these new UEs and services, it is expected that the 6G system will include at least one new RAT. In the 6G timeframe of 2030 and beyond, it is also anticipated there will be a need to support co-existence of 5G.

Proposal 1: 6G shall offer a migration path suitable for all types of 6G UEs, including single radio, dual radio, and devices of varying levels of complexity (e.g. low complexity devices).

## Interworking with Other Systems

According to SA1 requirements in TR 22.870, it is assumed that the 6G system shall support mobility between the core network of the 6G system and a 5G core network with minimum impact to the user experience (e.g. QoS, QoE).

Given 6G coverage will gradually expand, interworking with legacy systems is crucial to develop. Seamless mobility between 5G and 6G will be essential to ensure continuous service coverage, even with the occurrence of coverage islands. This will require inter-RAT mobility solutions that maintain session continuity and effective network control, similar to existing mobility cases. Additionally, voice support will be critical for 6G, including the ability to rapidly switch voice calls between 5G and 6G networks. The definition of a conditional fallback procedure (e.g. to VoNR) may also need to be studied, especially in early deployment phases in case 6G cannot provide sufficient coverage or stable user experience.

Observation 3: It is assumed that preference should be given to interactions between NR-SA and 6G-SA.

The gradual deployment of 6G equipment must integrate seamlessly with existing 5G infrastructure. This necessitates advanced interworking protocols between 5G and 6G networks to ensure a smooth and efficient technological evolution. The Inter-System CN interface plays a crucial role in the registration procedure. In 5G, registration can be achieved using the N26 interface, which is a P2P interface between MME and AMF to facilitate efficient transfer of UE context and PDU sessions. With SBA integrated into the 5GC and assuming 6G will support SBA also, 3GPP can study adopting a service-based interface between 5G and 6G networks. From SA2 perspective, architectural implications, e.g., to describe NF service and functional procedural aspects, can be studied under WT#2 (in general) or as part of WT#1.2 under SBA framework (for more specific aspects).

Proposal 2: The impact of interworking on inter-system interfaces and procedures needs to be studied.

The existing 5G architecture describes 5GC interworking with EPS not only within the same PLMN but for roaming scenarios as well. Namely there are roaming architectures for local breakout and home-routed roaming (refer to e.g. TS 23.501, clause 4.3.2).

Similarly, it can be expected that SA2 needs to identify roaming scenarios between an operator that deploys the 6G system and an operator that only deploys the 5G System. In particular, the level of service (including service continuity) provided to a roaming user needs to be studied. Furthermore, key system procedures for session management, mobility management, QoS and inter-PLMN interfaces need to satisfy roaming requirements for the UE and the network.

Proposal 3: Roaming architectures for interworking scenarios and related key system procedures need to be studied.

In addition to the above, SA2 may also have to consider how to provide service exposure in interworking scenarios, including support of interworking for monitoring events and service level APIs. However, this task is of slightly lower priority and can be done once the basic interworking architecture and the 6G system architecture is reasonably known. Therefore, this item may not form an immediate part of WT#2 in the 6G SID, but we are open to discuss when to consider it.

Proposal 4: Service exposure in interworking scenarios can be considered at a later time.

Given the scenario where not all geographical areas can be covered by 6G, interworking with legacy systems is essential to develop. Last but not least satellite systems, including 4G/5G NTN/satellite access that use EPS/5GS, are expected to extend coverage to mobile subscribers. The 6G SID already mentions this area and apart from a key issue, no further additions are suggested to this part of the WT at this time.

## Conclusions

In conclusion, this contribution discusses the scope of the SA2 6G SID for WT#2 in Rel-20.

Architectural Motivation: Any new generation requires a strategy for migrating from existing deployments to the next version of the 3GPP system. Since migration typically is a gradual process, there is a need for interworking between the new system and the existing system.

Gap Analysis: Due to the specification of multiple network architecture deployment options and migration paths, 4G/5G migration and interworking has proven overly complex and expensive.

Technical Impact: This WT touches upon the overall architecture for migration and interworking, including related system procedures and inter-system interfaces.

We have the following observations and proposals:

Observation 1: Looking ahead to 6G, establishing a clear (ideally single) migration path is essential to prevent industry fragmentation and to ensure more predictable investment and deployment timelines.

Observation 2: The migration architecture needs to be studied.

Proposal 1: 6G shall offer a migration path suitable for all types of 6G UEs, including single radio, dual radio, and devices of varying levels of complexity (e.g. low complexity devices).

Observation 3: It is assumed that preference should be given to interactions between NR-SA and 6G-SA.

Proposal 2: The impact of interworking on inter-system interfaces and procedures needs to be studied.

Proposal 3: Roaming architectures for interworking scenarios and related key system procedures need to be studied.

Proposal 4: Service exposure in interworking scenarios can be considered at a later time.

Based on the observations and analysis above and following the discussions in SA2#170, this paper proposes a revision of the Work Task scope based on the postponed document S2-2508129 at SA2#170 as well as a Key Issue.

# 2   Proposed Changes to 23.801-01

**** First Change ****

## Annex A.X. WT#2 Scope for Migration and Interworking

WT#2: Study migration and interworking, including

1	How to support migration to 6GS.

2	How to support interworking with 5GS (including when using 5G NTN/satellite access).

3	Whether and how to support interworking with EPS (including when using 4G NTN/satellite access).

NOTE 1:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is assumed to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 2: The detailed migration study scope will be coordinated and aligned with RAN.

NOTE 3: The study should consider roaming aspects.

NOTE 4: Among other tasks the study should identify a minimum migration path that can apply to any UE capable to support both 6GS and earlier 3GPP systems (e.g. 5GS).

NOTE 5: The study scope covers general interworking and migration aspects for topics identified in other work tasks (e.g., UE–CN signalling, user plane, QoS, slicing, service continuity, inter-RAT mobility, FWA, localized service, cellular IoT, etc). Specific interworking requirements resulting from new 6GS feature/enhancement can be analysed and if needed further addressed in the corresponding dedicated WT.

**** Second Change (all new text) ****

## 5.X.1 Key Issue #2: Migration and Interworking

This key issue studies migration and interworking. The study is conducted under the following general principles:

- Strive for a balance between complexity, performance, spectrum efficiency, and energy efficiency.

- Minimize the number of migration architecture options.

- Migration applies to UEs with support for both 6G and earlier generation(s), including single radio, dual radio, low complexity devices, etc.

The key issue covers the following aspects:

- How to support migration to 6GS, including

  - How the 6GS can support migration from 5GS while enabling interoperability, coexistence and consistent user experience.

- How to support interworking with 5GS (including when using 5G NTN/satellite access) while considering

  - Interworking architecture, mobility management, session management, service continuity

  - Architectural aspects of inter-system (intra-PLMN and inter-PLMN) interfaces and procedures in non-roaming and roaming scenarios.

- Whether and how to support interworking with EPS (including when using 4G NTN/satellite access) while considering

  - Interworking architecture, mobility management, session management, service continuity

  - Architectural aspects of inter-system (intra-PLMN and inter-PLMN) interfaces and procedures in non-roaming and roaming scenarios.

NOTE 1:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is assumed to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 2: The detailed migration study scope will be coordinated and aligned with RAN.

NOTE 3: The Key Issue should identify a minimum migration path that can apply to any UE capable to support both 6GS and earlier 3GPP systems (e.g. 5GS).

NOTE 4: The Key Issue covers general interworking and migration aspects for topics identified in other Key Issues (e.g., UE–CN signalling, user plane, QoS, slicing, service continuity, inter-RAT mobility, FWA, localized service, cellular IoT, etc). Specific interworking requirements resulting from new 6GS feature/enhancement can be analysed and if needed further addressed in the corresponding dedicated Key Issue.

**** End of Changes ****