SA WG2 Meeting #171	S2-2508941

13 – 17 Oct, 2025, Wuhan, CN	(revision of S2-2507902)

Source:	CableLabs, Charter Communications, Rogers Communications

Title:	[WT#4] WT and KI on non-3GPP sensing

Document for:	Approval

Agenda Item:	20.6.4

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution:  Proposes scope of Work Task #4 and Key Issue on non-3GPP sensing.

# 1. Justifications

Work task 4 scope as agreed in 6G System Study item SP-250806:

WT#4: Study the integration of Sensing and Communication over 3GPP access, considering the sensing modes to be supported and other sources of sensing data.

NOTE 5:	The detailed scope of WT#4 will be determined considering the scope and work of R20 FS_Sensing_ARC.

Architectural Motivation: Wi-Fi sensing uses existing Wi-Fi signals to detect and interpret motion, gestures, and other changes in an environment. It transforms the existing Wi-Fi infrastructure into a sensing network without requiring additional hardware. Wi-Fi sensing enables a range of applications such as home security, aging-in-place, healthcare, automotive, XR/VR, IoT control, and business/enterprise.

IEEE 802.11bf, a Task Group on WLAN sensing within the IEEE 802.11 WG, has defined modifications to 802.11 MAC and PHY to enhance WLAN sensing operation. The standard was approved on May 28, 2025 and has been published.

The evolution towards 6G networks requires the intelligent integration of communication and sensing capabilities to support diverse and complex applications. However, relying on single-modal sensors has certain limitations, such as limited representation of environment features and insufficient coverage. Just as humans use multiple senses (sight, sound, touch) to perceive the world, multi-modality sensing fuses data from various sources to build a richer, more reliable picture of the surroundings. Using multiple sources to gather sensing data allows for compensating the limitation of one type of sensor with another. Integrating Wi-Fi sensing within 6G’s sensing framework provides several critical benefits:

- Enhanced environmental awareness: Wi-Fi sensing can detect motion, occupancy, and even breathing patterns through existing communication infrastructure. When combined with other sensing modalities, it provides redundant and complementary information that improves overall system reliability and accuracy.

- Ubiquitous sensing coverage: Wi-Fi infrastructure is already deployed extensively in indoor environments. By leveraging this for sensing alongside communication, 6G networks can provide sensing capabilities wherever Wi-Fi coverage exists, without requiring additional dedicated sensors.

- Cross-modal validation and fusion: Wi-Fi sensing can validate and complement information from other sensors. For example, if a base station detects motion but Wi-Fi sensing doesn't detect corresponding RF signal changes, this could indicate potential security threats or sensor malfunctions.

Thus, the multi-modal approach addresses the limitations that single sensing modalities often have - gaps in coverage, accuracy or functionality - creating more robust and comprehensive sensing systems that are essential for 6G's ambitious application requirements.

Furthermore, several requirements have been agreed in SA1 on both combining results from 3GPP and non-3GPP sensing, as well as utilizing non-3GPP sensing solely –

- From 5G TS 22.137 –

Subject to user consent, regulation, and operator’s policy, the 5G system shall be able to collect non-3GPP sensing data from authorized non-3GPP sensors and securely provide it to 5G network.

Subject to user consent, regulation, and operator’s policy, the 5G system should support the joint processing of the 3GPP sensing data and non-3GPP sensing data to derive a combined sensing result.

- From 6G TR 22.870 –

[PR 7.16.6-2] Subject to regulation and operator’s policies, the 6G network shall be able to configure and/or authorize or revoke authorization of non-3GPP sensing station(s) for 6G wireless sensing service.

[PR 7.16.6-3] Subject to operator’s policy and regulation, the 6G system shall be able to provide secure means for a trusted third-party to receive sensing results derived from non-3GPP sensing data.

Technical Impact: System architecture to support Wi-Fi sensing, sensing data and the associated information collection and transport, sensing result exposure.

**** First Change (All New Text) ****

# Annex A.X. WT#4 Integration of sensing and communication

Study below aspects on integration of sensing and communication in 6G:

- Support for non-3GPP sensing data and data fusion from multiple sensing entities.

**** Second Change (All New Text) ****

# Key Issue #X: Non-3GPP sensing

Study below aspects on non-3GPP sensing in 6G:

- Support for collection of non-3GPP sensing data (e.g., from WLAN sensing, camera, etc.), including transport and storage of sensing related data, and exposure of sensing result.

- Support for fusion of sensing data (e.g., 3GPP sensing data and non-3GPP sensing data) from multiple sensing entities.

NOTE: Security and privacy aspects will be addressed by SA3.

**** End of Changes ****