SA WG2 Meeting #171	S2-2508944

13 – 17 October, 2025, Wuhan, China	(Revision of S2-2508122)

Source:	[Ericsson, Xiaomi, OPPO, Nokia, InterDigital, Google, Charter Communications], CATT

Title:	[WT#1.2.x, QoS] QoS Framework

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution provides content on WT 1.2.x QoS Framework with 5GS QoS Framework as a baseline.

**** First Change ****

Annex A.X WT#1.2.X: Evolution of QoS Framework for 6G

Editor’s Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

- Investigate  the unified 6G QoS framework considering the emerging new traffic characteristics and application needs, the QoS requirements of the traffic in the data plane and the QoS enforcement in a more efficient and better or intelligent way than the 5G QoS framework.

NOTE 1: The definition of 6GS QoS concepts in the 6G System perspective and in the applicationperspective respectively should have clear boundary (enable clear & measurable service descriptions for SLAs).

- Whether and how to enhance QoS control mechanisms to

  - to support different QoS control to the same traffic stream.

  - to support more QoS targets than the existing non-GBR and less resource intensive than the existing GBR, Delay Critical-GBR.

NOTE 2: It does not exclude that the GBR and Delay Critical-GBR are not supported in 6G QoS framework.

  - to improve QoS with dynamic application requirements and dynamic network states.

- Whether and what to monitor and provide QoS related information between 6G network and application in the UE and server side 	in order to improve awareness in the 6GS of connectivity performance needs.

- Study how to ensure interworking with 5GS.

NOTE 3: The content of the 5GS QoS Framework that will be used as a starting point for discussion on 6G QoS will be determined by moderated discussion process.

- Study what information to be used to do traffic detection and what labels to be marked to the UL and or DL traffic.

NOTE 4: How to perform the traffic detection and marking in the User Plane based on the request from the QoS control NF are assumed to be handled by the WT#1.2 User Plane.

NOTE 5: Impacts on QoS originating from new services will be considered based on and when relevant input is identified and provided by the relevant WTs.

NOTE 6: Whether and how the AI functionality within the 6G network to support Control Plane and User Plane QoS control will be coordinated with the WT#3 AI.

NOTE 7: This WT will coordinate with SA WG4 and RAN WGs.

**** End of Changes ****