SA WG2 Meeting #171	S2-2508954

Wuhan, China, October 13 – 17, 2025

Source:	NTT DOCOMO

Title:	KI#2: Update Interim agreement

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This contribution updates interim agreements for KI#2.

# 1	Discussion

This paper proposes interim agreements.

Regarding the reduction of reporting load from input data sources and minimizing the burden on C-Plane NFs, the examples stated in Further consideration are already described in several solutions and can therefore be moved to agreed principles. In particular, when using the Nupf_EventExposure service to collect UPF data, it is important to limit the amount of data transmitted over the SBI.

Furthermore, regarding “Whether new UPF or SMF events are needed or not for NWDAF data collection”, the essential issue is how to reduce the amount of data as described above, and the introduction of new services on SBI does not have substantial benefit itself. Therefore, it is sufficient to extend existing services.

Finally, since for both use cases it is common that NWDAF provides Analytics to the consumer, we propose to move this description to the general section for conciseness.

# 2 Proposal

It is proposed to update TR 23.700-04 according to the following text.

********** First Change **********

#### 7.1.2.1	General

No impact on RAN and UE operation.

#### To reduce the reporting load of input data sources (e.g. UPF), the analytics consumer may

-	include instructions (e.g., the Analytics Filter Information for input data filtering or pre-processing) to UPF.

-	combine notification to NWDAF at UPF for multiple events.

To collect UPF data from NWDAF to UPF, existing service can be extended.

NWDAF provides analytics on UPF abnormal traffic and traffic patterns to the consumers.

#### 7.1.2.2	Agreed Principles for Use Case #1

-	The service consumer of the analytics service may be SMF, PCF, OAM and AF. The consumer may take the output Analytics into account.

-	To derive the output analytics, the following input data may be collected from the UPF to support the NWDAF-based analytics:

-	Information to identify the abnormal traffic.

#### 7.1.2.3	Agreed Principles for Use Case #2

-	The service consumer of the analytics service is PCF.

-	To derive the output analytics, the following input data may be collected from NFs to support the NWDAF-based analytics:

-	Information identifying the associated application/service data flow of the collected QoS monitoring information.

-	The consumer actions may take the output Analytics into account, e.g. generate or update PCC rules.

********** Second Change **********

### 7.2.2	Topics for further consideration for KI#2

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#2. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in clause 7.1.2) and can either be then marked as not pursued or postponed to a future release.

The following topics are for further consideration for KI#2:

-	Whether the UPF can be the consumer of the user plane traffic pattern and behaviour analysis or not.

-	Consumer actions are FFS.

********** End of Changes **********