SA WG2 Meeting #171	S2-2508984

13-17 Oct, 2025, Wuhan, China

Source:	CSCN

Title:	[General aspects] Architecture Assumptions for 6G

Document for:	Approval

Agenda Item:	20.6.0

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution puts forward the assumption of a 6G system with integrated design for both Terrestrial Networks (TN) and Non-Terrestrial Networks (NTN).

Justifications

A unified 6G system architecture with inherent support for both Terrestrial Networks (TN) and Non-Terrestrial Networks (NTN) via core network (CN) functions is imperative. As outlined in the RAN-led study on 6G scenarios and requirements (RP-250810) and the RAN1-led study on 6G radio technologies (RP-251881), RAN explicitly targets a harmonized 6G radio design for TN and NTN, including their deep integration—this radio-level unification requires corresponding alignment at the system architecture layer to realize end-to-end integration.

Hence, the 6G Radio Access Technology (RAT) is expected to natively support NTN—and by extension, the 6G system architecture must align with this unified radio framework to accommodate both TN and NTN deployment scenarios. A fragmented architecture (e.g., separate CN components for TN and NTN) would directly contradict this radio-level integration, leading to redundant signaling, elevated latency, and inconsistent user experiences—ultimately undermining 6G’s vision of seamless ubiquitous connectivity.

Thus, this proposal advances the assumption that the 6G system architecture—encompassing core network (CN) functions—should be designed to inherently support both Terrestrial Networks (TN) and Non-Terrestrial Networks (NTN).

Proposal

It is proposed to agree the following changes into TR 23.801-01.

**** First Change (all text new) ****

4.1	Architectural Assumptions

Editor's Note: This clause documents the common architecture assumptions identified for the study.

The following is assumed for 6GS architecture:

1.	The framework of SBA specified for 5GC is assumed as a starting point for discussion.

[OPTION1]

2.	For 3GPP access, the scope of the study is single technology framework based on a stand-alone architecture.

[OPTION2]

2.  The study assumes a standalone architecture [ref: RP-251881].

[ENDOFOPTIONS]

NOTE: approaches beyond stand-alone architecture will be considered in coordination with TSG RAN [ref: RP-251881].

3.	The 6G system is assumed to maintain the existing RAN and CN functionality split as supported in 5GS, and avoid duplication of functionality in 6G RAN and CN.

4.	Support of multi-media services such as voice, video (essential and regulatory services) will be using IMS.

x. The 6G system architecture, encompassing core network (CN) functions, is assumed to natively support both Terrestrial Networks (TN) and Non-Terrestrial Networks (NTN).

**** End of Changes ****