3GPP TSG-SA WG2 Meeting #171	S2-2509003

Wuhan, China, October 13 – 17, 2025

Source:	MediaTek Inc.

Title:	Interim agreements for KI#3

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes the interim agreements for KI#3

Discussion

Based on the merged tdoc in S2-2507677, the NF profile table does not list all the corresponding information for NF selection and reselection and the usage of renew energy power information can be provided from SA5 since it is included in clause 8.3.2 EnergySourceInfo in TS 28.310 to define the energy resource type. The usage information of the renewable energy  can be obtained from OAM and can be included as the NF profile information for NF selection and reselection.

* * * Start of Change * * * *

7	Interim agreements

7.1	Agreed Principles

7.1.3	Agreed Principles for KI#3

The following principles are agreed for KI#3:

-	For NF selection, the NF profile is enhanced to include the parameters in the table below. This is a feature requiring the software to be upgraded in NRF and its consumers.

| Information | Description |
| --- | --- |
| EnergySaving /Not EnergySaving State | Value of the energy state as specified in TS 28.310 [9], e.g. notEnergySaving state, Energy Saving state. (NOTE 1) |
| Schedule of EnergySaving/not EnergySaving | The schedule of the EnergySaving/not EnergySaving states, includes the value of EnergySaving or not EnergySaving state which applies at each time interval in the schedule. If this is present, the EnergySaving /Not EnergySaving State attribute is not present. |
| Energy Priority Information | Value of Energy priority relative to other NFs of the same type considering the energy characteristics of the NF. |
| Schedule of Energy Priority Information | The Schedule of the Energy Priority includes the value of Energy Priority information which applies at each time interval in the schedule. If this is present, the Energy Priority information attribute is not present |
| The usage of renewable energy | Value (Boolean type) of the usage of renewable energy is to represent the energy source is renewable or non-renewable and this value is configured by operator with the information obtained from external sources. |
| NOTE 1: EnergySaving/not EnergySaving refers to states of a network function which are defined in TS 28.310 [9]. |  |

-	For NF profile, additional parameter such as renewable energy consumption ratio may be considered is SA5 indicates support.

-	For UP path selection and reselection, the UPFs are selected/reselected by using NF discovery and selection procedures which may be Energy-aware if NF profile is updated as per bullets above. Changes in NF profile of a UPF may trigger UPF reselection subject to SSC mode of the PDU sessions.

7.2.3	Topics for further consideration for KI#3

The following principles are for further discussions for KI#3:

-	whether the NF profile needs to include explicit indication of using energy Priority to implicitly take this into account is sufficient.

-	Whether considering the energy consumption of a PDU sessions or of a NF over a Path should be a trigger for reselection of UPFs (including embedding this information in DNAIs )

-	In case of multiple configured energy policies in network, the "energy priority" is an array of priorities. Each entry of the array is the priority of the NF in the corresponding energy policy configured by the operator.

* * * End of Change * * * *