SA WG2 Meeting #171	S2-2509012

Wuhan, CN, 13th Oct – 17th Oct, 2025

Source:	Huawei, HiSilicon

Title:	[WT#2, Interworking and Migration] Inter-System interworking and migration consideration

Document for:	Approval

Agenda Item:	20.6.2

Work Item / Release:  FS_6G_ARC/Rel-20

Abstract of the contribution: This paper discusses the interworking and migration aspect of 6G system. It proposes the related Key Issues.

1. Justifications

Based on the last SA2 meeting, the WT#2 scope for interworking with legacy system is basically stable. Therefore, the following KIs on the interworking with 5GC are proposed, including:

-   How to support Idle mode and Connected mode mobility between 3GPP access, it includes following aspects:

- how to support Service Continuity (i.e. IP address preservation) if required.

- non-roaming and roaming scenario.

Interworking between 6GS and 5GS is assumed to work even if the UE has previously registered in 2G, 3G or 4G. Thus, the PDU sessions switched from 2G, 3G or 4G to 5G for the 5G registered UE that had previously registered in 2G, 3G or 4G, as well as the PDU sessions purely established at 5G shall both be considered during Interworking between 6GS and 5GS.

Additionally, it is also agreed to study the interworking with non-3GPP access in WT#2, instead of WT#1.3, therefore, the KI on how to support interworking with non-3GPP access is added in this part.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.801-01.

**** First Change****

Annex A.X. WT#2 Migration and Interworking Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#2: Study migration and interworking, including

1	How to support migration to 6GS.

2	How to support interworking with 5GS (including when using 5G NTN/satellite access and non-3GPP access).

3	Whether and how to support interworking with EPS (including when using 4G NTN/satellite access).

NOTE 1:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is assumed to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 2: The detailed migration study scope will be coordinated and aligned with RAN.

NOTE 3: The Work Task should consider roaming aspects.

**** Next Change ****

5.X. Key Issue: 6GS interworking and migration

This key issue for 6GS migration and interworking includes the following:

1	How to support migration to 6GS.

NOTE 1: The detailed migration study scope will be coordinated and aligned with RAN.

2	How to support interworking with 5GS (including when using 5G NTN/satellite access and non-3GPP access). The interworking with 5GS needs to consider the PDU sessions successfully switched from 2G, 3G or 4G to 5G for the UE previously registered in 2G, 3G or 4G, as well as the PDU sessions newly established at 5G.

-  How to support Idle mode and Connected mode mobility between 3GPP access, it includes following aspects:

-  how to support Service Continuity (i.e. IP address preservation) if required.

-  non-roaming and roaming scenario.

-  How to support interworking with non-3GPP access, e.g. handover from 5GC via 3GPP access to 6GC via non-3GPP access, or vice versa.

3	Whether and how to support interworking with EPS (including when using 4G NTN/satellite access).

NOTE 2:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is assumed to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 2: The detailed migration study scope will be coordinated and aligned with RAN.

**** End of Changes ****